|  | ЧЕК-ЛИСТ / checklist (Търговски въздушен транспорт) (Commercial Air Transport) МОДУЛ / Module ORO / FLIGHT CREW |  | № ORO_FC- |  |
| --- | --- | --- | --- | --- |
| Оператор/Организация: Operator /Organisation: | Инспектор / Inspector (име, фамилия / name, surname) | Отдел: Department: | САО №: AOC No: |  |
|  |  |  | Дата: Date: |  |
| Обхват на инспекцията/одита: Scope of Audit: | Annex III, Part ORO - ORGANISATION REQUIREMENTS FOR AIR OPERATIONS Subpart FC - FLIGHT CREW |  |  |  |

Забележка: В полето „№“ след абревиатурата се поставя номера на заповедта за комисия.(Пример - № ORO_PR-45-01-00/01.01.2016)

| Note | ОСНОВАНИЕ References | ВЪПРОСИ, НАСОКИ ЗА ИНСПЕКТОРА Questions, guidelines to inspectors | СЪОТВЕТСВИЕ Compliance |  |  |  |  | ДОКАЗАТЕЛСТВА/ ЗАБЕЛЕЖКА Evidence/Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | NC | S | NS | NA |  |
| 1. |  | Make sure you have the latest revision or/and easy access rules of the Regulation (EU) No 965/2012, either in electronic or hard copy, readily available. |  |  |  |  |  |  |
| 2. | ORO.FC | When checking revision or a new edition of OM D, or when initial certification or operation of a new type of aircraft is concerned, please make sure the relevant OSD is available. |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | (Please insert reference issue – date or/and number) |

| № | ОСНОВАНИЕ References | ВЪПРОСИ, НАСОКИ ЗА ИНСПЕКТОРА Questions, guidelines to inspectors | СЪОТВЕТСТВИЕ Compliance |  |  |  |  | ДОКАЗАТЕЛСТВА/ ЗАБЕЛЕЖКА Evidence/Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | NC | S | NS | NA |  |
| 1. | ORO.FC.100 | Regulation (EU) 2021/2237 Composition of flight crew |  |  |  |  |  |  |
|  |  | (a) The composition of the flight crew and the number of flight crew members at designated crew stations shall be not less than the minimum specified in the aircraft flight manual or operating limitations prescribed for the aircraft. |  |  |  |  |  |  |
|  |  | (b) The flight crew shall include additional flight crew members when required by the type of operation and shall not be reduced below the number specified in the operations manual. |  |  |  |  |  |  |
|  |  | (c) All flight crew members shall hold a licence and ratings issued or accepted in accordance with Commission Regulation (EU) No 1178/2011 and appropriate to the duties assigned to them. |  |  |  |  |  |  |
|  |  | (d) The flight crew member may be relieved in flight of his or her duties at the controls by another suitably qualified flight crew member. |  |  |  |  |  |  |
|  |  | (e) When engaging the services of flight crew members who are working on a freelance or part-time basis, the operator shall verify that all applicable requirements of this Subpart and the relevant elements of Annex I (Part-FCL) to Regulation (EU) No 1178/2011, including the requirements on recent experience, are complied with, taking into account all services rendered by the flight crew member to other operator(s) to determine in particular: (1) the total number of aircraft types or variants operated; and (2) the applicable flight and duty time limitations and rest requirements. |  |  |  |  |  |  |
|  |  | (f) Specific requirements for helicopter operations If the helicopter is operated with a crew of two pilots, each pilot shall either: (1) hold a certificate of satisfactory completion of a multi-crew cooperation (MCC) course in helicopters in accordance with Regulation (EU) No 1178/2011; or (2) have at least 500 hours of flight time as a pilot in multi-pilot operations. |  |  |  |  |  |  |
| 2. | GM1 ORO.FC.100(c) | Licence and ratings in accordance with commission regulation (EU) No 1178/2011 |  |  |  |  |  |  |
|  |  | When determining the composition of the crew, and monitoring whether the flight crew holds the appropriate licence and ratings, the operator needs to take into account any limitations prescribed in Regulation (EU) No 1178/2011 applicable to the flight crew members such as, but not limited to, recent experience and operational multi-pilot limitation |  |  |  |  |  |  |
| 3. | ORO.FC.105 | Regulation (EU) 2021/2237 Designation as pilot-in-command/commander |  |  |  |  |  |  |
|  |  | (a) In accordance with point 8.6 of Annex V to Regulation (EU) 2018/1139, one pilot amongst the flight crew, qualified as pilot-in-command in accordance with Annex I (Part-FCL) to Regulation (EU) No 1178/2011, shall be designated by the operator as pilot-in-command or, for commercial air transport operations, as commander. |  |  |  |  |  |  |
|  |  | (b) The operator shall only designate a flight crew member to act as pilot-in-command/commander if all of the following apply: (1) the flight crew member has the minimum level of experience specified in the operations manual; (2) the flight crew member has adequate knowledge of the route or area to be flown and of the aerodromes, including alternate aerodromes, facilities and procedures to be used; (3) in the case of multi-crew operations, the flight crew member has completed an operator’s command course if upgrading from co-pilot to pilot-in-command/commander. |  |  |  |  |  |  |
|  |  | (c) In the case of commercial operations of aeroplanes and helicopters, the pilot-in-command/commander or the pilot to whom the conduct of the flight may be delegated shall have had initial familiarisation training on the route or area to be flown and on the aerodromes, facilities and procedures to be used and shall maintain this knowledge as follows: (1) The validity of the aerodrome knowledge shall be maintained by operating at least once on the aerodrome within a 12 calendar months’ period. (2) The route or area knowledge shall be maintained by operating at least once to the route or area within a 36 months’ period. In addition, refresher training is required regarding route or area knowledge if not operating on a route or area for 12 months within the 36-month period. |  |  |  |  |  |  |
|  |  | (d) Notwithstanding point (c), in the case of operations under VFR by day with performance class B and C aeroplanes and helicopters, familiarisation training on the route and aerodromes may be replaced by area familiarisation training. |  |  |  |  |  |  |
|  |  | - Check the adequate implementation of the operator’s methodology for route/ area and aerodrome competency. Sample flight return documentation and crew training records. - Check the operator’s process for the monitoring of the route/aarea (36M) and aerodrome (12M) competency. - Check that all trainings are recorded (including self-briefing). |  |  |  |  |  |  |
| 4. | AMC1 ORO.FC.105(b)(2);(c) | ED Decision 2016/022/R GENERAL The operator should comply with the national training and checking requirements published in the aeronautical information publication (AIP). Route/area and aerodrome knowledge for commercial air transport operations The experience of the route or area to be flown and of the aerodrome facilities and procedures to be used should include the following: |  |  |  |  |  |  |
|  |  | (a) Area and route knowledge |  |  |  |  |  |  |
|  |  | (1) An objective of the area and route training should be to ensure that the pilot has knowledge of: (i) terrain and minimum safe altitudes; (ii) seasonal meteorological conditions; (iii) meteorological, communication and air traffic facilities, services and procedures; (iv) search and rescue procedures where available; and (v) navigational facilities associated with the area or route along which the flight is to take place. |  |  |  |  |  |  |
|  |  | (2) Another objective of the area and route training should be to ensure that the pilots are aware of the most significant underlying risks and threats of a route or an area that could affect their operations following the ‘threat and error management model’ or an alternative risk model agreed with the authority. |  |  |  |  |  |  |
|  |  | (3) The area and route familiarisation training should: (i) be based on an assessment by the operator of the underlying risks and threats of a route or an area using: (A) internal evidence; (B) external evidence; (ii) be conducted: (A) as an initial training before operating to a route and area; (B) as a refresher training after not operating to a route and area for 12 months. |  |  |  |  |  |  |
|  |  | (4) The area and route familiarisation training should be delivered using different methods and tools. (i) The selection of the method and tools should result from a combination of the learning objectives and the type of risk or threat that needs to be trained. (ii) The selection of the appropriate method and tool should be driven by the desired outcome in terms of adequate knowledge and awareness. (iii) The methods and tools employed should include one or more of the following: Training in a flight simulation training device (FSTD), computer-based training, familiarisation flight as a pilot in-command/commander or co-pilot under supervision or an observer, video training, virtual reality training, familiarisation by self-briefing with route documentation and audio training. |  |  |  |  |  |  |
|  |  | (b) Aerodrome knowledge |  |  |  |  |  |  |
|  |  | (1) Aerodrome amiliarisation training should include knowledge of obstructions, physical layout, lighting, approach aids and arrival, departure, holding and instrument approach procedures, applicable operating minima and ground movement considerations. |  |  |  |  |  |  |
|  |  | (2) The operations manual should describe the method of categorisation of aerodromes and, in the case of CAT operations, provide a list of those aerodromes categorised as B or C. |  |  |  |  |  |  |
|  |  | (3) All aerodromes to which an operator operates should be categorised in one of these three categories: |  |  |  |  |  |  |
|  |  | (i) category A — an aerodrome that meets all the following conditions: (A) straight-in 3D instrument approach procedure with a glide path angle of not more than 3.5 degrees to each runway expected to be used for landing;; (B) at least one runway with no performance limited procedure for take-off and/or landing; such as no requirement to follow a contingency procedure for obstacle clearance in the event of an engine failure on take-off from any runway expected to be used for departure; and (C) night operations capability. |  |  |  |  |  |  |
|  |  | (ii) category B — an aerodrome that does not meet the category A conditions or which requires extra considerations due to : (A) non-standard approach aids and/or approach patterns, such as restrictions on the availability of straight-in instrument approach procedures; (B) unusual local weather conditions, such as environmental features that can give rise to turbulence, windshear or unusual wind conditions;; (C) unusual characteristics or performance limitations, such as unusual runway characteristics in length, width, slope, markings or lighting that present an atypical visual perspective on approach (D) any other relevant considerations, including obstructions, physical layout, lighting, etc., such as restrictions on circling in certain sectors due to obstacles in the circling area; . (E) training or flight crew experience requirements stipulated by the competent authority responsible for the aerodrome that do not include instruction in an FSTD or visiting the aerodrome. |  |  |  |  |  |  |
|  |  | (iii) category C — an aerodrome: (A) that requires additional considerations to those of a category B aerodrome; or (B) for which flight crew experience or qualification requirements stipulated by the competent authority responsible for the aerodrome include instruction in an FSTD or visiting the aerodrome. Offshore installations may be categorised as category B or C aerodromes, taking into account the limitations determined in accordance with AMC1 SPA.HOFO.115 ‘Use of offshore locations’. |  |  |  |  |  |  |
|  |  | (c) Prior to operating to a category B aerodrome (planned destination or required alternate), the pilot-in-command/commander should: |  |  |  |  |  |  |
|  |  | (1) comply with any requirements stipulated by the competent authority responsible for the aerodrome; and |  |  |  |  |  |  |
|  |  | (2) be briefed, or self-brief by means of programmed instruction, about the additional considerations applicable to operations to that category B aerodrome. The completion of the briefing should be recorded. This recording may be accomplished after completion or confirmed by the pilot-in-command/commander before departure on a flight involving category B aerodrome(s) as destination or alternate aerodromes. |  |  |  |  |  |  |
|  |  | (d) Prior to operating to a category C aerodrome (planned destination or required alternate), the pilot-in-command/commander should: |  |  |  |  |  |  |
|  |  | (1) comply with any requirements stipulated by the competent authority responsible for the aerodrome; and |  |  |  |  |  |  |
|  |  | (2) be briefed or self-brief by means of programmed instruction, about the additional considerations applicable to operations to that category C aerodrome; and |  |  |  |  |  |  |
|  |  | (3) visit the aerodrome as an observer and/or undertake instruction in a suitable FSTD. The observer should occupy an observer’s seat where installed. If an observer’s seat is not available and cannot be installed, the pilot-in-command/commander may occupy a pilot seat to conduct the aerodrome visit with a suitably qualified commander nominated by the category C aerodrome operator. The completion of the briefing, visit and/or instruction should be recorded. |  |  |  |  |  |  |
| 5. | AMC2 ORO.FC.105(b)(2);(c) | GENERAL The operator should comply with the national training and checking requirements published in the AIP. Route, area and aerodrome knowledge for non-commercial operations The knowledge of the route and area to be flown and of the aerodrome facilities and procedures to be used should include the following: |  |  |  |  |  |  |
|  |  | (a) Area and route knowledge |  |  |  |  |  |  |
|  |  | (1) The objective of the area and route familiarisation should be to ensure that the pilot has knowledge of: (i) terrain and minimum safe altitudes; (ii) seasonal meteorological conditions; (iii) meteorological, communication and air traffic facilities, services and procedures; (iv) search and rescue procedures where available; and (v) navigational facilities associated with the area or route along which the flight is to take place. |  |  |  |  |  |  |
|  |  | (2) The operations manual should describe appropriate methods of familiarisation depending on the complexity of the area or route and the experience of the pilot-in-command. |  |  |  |  |  |  |
|  |  | (b) Aerodrome knowledge |  |  |  |  |  |  |
|  |  | (1) Aerodrome familiarisation should include knowledge of obstructions, physical layout, lighting, approach aids and arrival, departure, holding and instrument approach procedures, applicable operating minima and ground movement considerations. |  |  |  |  |  |  |
|  |  | (2) The operator’s manual should describe appropriate methods of familiarisation depending on the complexity of the aerodrome. |  |  |  |  |  |  |
|  |  | (3) If the competent authority of the aerodrome or area requires specific training or familiarisation, the operator should maintain all records of this training or familiarisation in accordance with ORO.GEN.220. |  |  |  |  |  |  |
|  |  | (4) For offshore installations, the limitations determined in accordance with AMC1 SPA.HOFO.115 should be taken into account. |  |  |  |  |  |  |
| 6. | GM1 ORO.FC.105(b)(2) | ED Decision 2015/012/R Environmental knowledge related to the prevention of aeroplane upsets The knowledge should include understanding of: |  |  |  |  |  |  |
|  |  | (a) the relevant environmental hazards, such as: — Clear Air Turbulence (CAT), — Intertropical Convergence Zone (ITCZ), — thunderstorms, — microbursts, — wind shear, — icing, — mountain waves, — wake turbulence, and — temperature changes at high altitude; |  |  |  |  |  |  |
|  |  | (b) the evaluation and management of the associated risks of the relevant hazards in (a); and |  |  |  |  |  |  |
|  |  | (c) the available mitigating procedures for the relevant hazards in (a) related to the specific route, route area, or aerodrome used by the operator. |  |  |  |  |  |  |
| 7. | GM2 ORO.FC.105(b)(2) | Aerodrome knowledge for non-commercial operations The operator may, based on complexity, categorise all aerodromes in one of the following three categories: |  |  |  |  |  |  |
|  |  | (a) category A — an aerodrome that meets all the following conditions: (1) an approved instrument approach procedure; (2) at least one runway with no performance-limited procedure for take-off and/or landing; (3) published circling minima not higher than 1 000 ft above aerodrome level; and (4) night operations capability. |  |  |  |  |  |  |
|  |  | (b) category B — an aerodrome that does not meet the category A conditions or which requires extra considerations due to: (1) non-standard approach aids and/or approach patterns; (2) unusual local weather conditions; (3) unusual characteristics or performance limitations; (4) any other relevant considerations, including obstacles, physical layout, lighting, etc. |  |  |  |  |  |  |
|  |  | (c) category C — an aerodrome that requires additional considerations to those of a category B aerodrome. |  |  |  |  |  |  |
|  |  | Offshore installations may be categorised as category B or C aerodromes, taking into account the limitations determined in accordance with AMC1 SPA.HOFO.115 ‘Use of offshore locations’. |  |  |  |  |  |  |
| 8. | AMC1 ORO.FC.105(b)(3) | Operator’s command course for non-commercial operations with complex motor-powered aircraft (ncc) |  |  |  |  |  |  |
|  |  | (a) For aeroplane and helicopter operations, when upgrading from co-pilot to pilot-in-command, the flight crew member should be trained at least on the following elements, as part of the command course: (1) command responsibilities training; (2) demonstration of competence operating as pilot-in-command. |  |  |  |  |  |  |
|  |  | (b) Demonstration of competence operating as pilot-in-command may be achieved by: (1) completing a proficiency check in the role of pilot-in-command; or (2) operating at least one flight under the supervision and to the satisfaction of a suitably qualified pilot-in-command nominated by the operator. |  |  |  |  |  |  |
| 9. | AMC1 ORO.FC.105© | ED Decision 2014/017/R Route/area and aerodrome recency |  |  |  |  |  |  |
|  |  | (a) The 12-month period should be counted from the last day of the month: (1) when the familiarisation training was undertaken; or (2) of the latest operation on the route or area to be flown and of the aerodromes, facilities and procedures to be used. (b) The 36-month period should be counted from the last day of the month: (1) when the familiarisation training was undertaken; or (2) when the latest operation on the route or area was flown. |  |  |  |  |  |  |
| 10. | AMC2 ORO.FC.105© | ED Decision 2014/017/R Route/area and aerodrome recency – performance class b aeroplanes operated under vfr by night or ifr in cat operations In the case of CAT operations with performance class B aeroplanes operating under visual flight rules (VFR) by night or instrument flight rules (IFR), the knowledge should be maintained as follows: |  |  |  |  |  |  |
|  |  | (a) except for operations to the most demanding aerodromes, by completion of at least 10 flight sectors within the area of operation during the preceding 12 months in addition to any required self-briefing; |  |  |  |  |  |  |
|  |  | (b) operations to the most demanding aerodromes may be performed only if: (1) the pilot-in-command/commander has been qualified at the aerodrome within the preceding 36 months by a visit as an operating flight crew member or as an observer; (2) the approach is performed in visual meteorological conditions (VMC) from the applicable minimum sector altitude; and (3) an adequate self-briefing has been made prior to the flight. |  |  |  |  |  |  |
| 11. | GM1 ORO.FC.105(c) | Area and route familiarisation training delivery When developing the area and route familiarisation training, the operator may apply the following methodology: |  |  |  |  |  |  |
|  |  | (a) Internal evidence (1) Operator assessment by conducting an operational risk evaluation according to the following criteria: (i) terrain and minimum safe altitudes; (ii) seasonal meteorological conditions; (iii) meteorological, communication and air traffic facilities, services and procedures; (iv) search and rescue procedures where available; and (v) navigational facilities associated with the area or route along which the flight is to take place. |  |  |  |  |  |  |
|  |  | (2) Operator-specific evidence gathered through the safety management process in accordance with ORO.GEN.200. |  |  |  |  |  |  |
|  |  | (b) External evidence (1) notices to airmen (NOTAMs); (2) AIP. |  |  |  |  |  |  |
|  |  | (c) When selecting the method and tool, operators should be driven by the objective of reaching the optimum in terms of the desired outcome, which is the maximum possible knowledge increase. This methodology intends that such selection is based on the type of the underlying risks of a route / area as determined in accordance with (a) and (b) and the learning objectives. For example: for the less complex areas or routes, familiarisation by self-briefing with route documentation, or by means of programmed instruction; and for the more complex areas or routes, in-flight familiarisation as a pilot-in-command/commander or co-pilot under supervision or an observer, or familiarisation in a flight simulation training device (FSTD) using a database appropriate to the route concerned. |  |  |  |  |  |  |
| 12. | AMC1 ORO.FC.105(d) | Area familiarisation training that includes route /aerodrome familiarisation — helicopters |  |  |  |  |  |  |
|  |  | (a) The area familiarisation training for day VFR should ensure that a pilot is capable of selecting aerodromes and operating sites from the ground and from the air, and of establishing a safe flight path for landing and take-off. |  |  |  |  |  |  |
|  |  | Area familiarisation training |  |  |  |  |  |  |
|  |  | (b) The following areas and conditions should require specific area familiarisation training: (1) mountain environment; (2) offshore environment; (3) complex airspace; (4) areas that are regularly covered by snow and are prone to white-out phenomena during the cruise or landing phase; and (5) other challenging areas or conditions. |  |  |  |  |  |  |
| 13. | GM1 ORO.FC.105(d) | ED Decision 2014/017/R Performance class b aeroplanes operated under VFR by day in CAT operations For CAT operations under VFR by day with performance class B aeroplanes, the operator should take account of any requirement that might be stipulated in specific cases by the State of the aerodrome. |  |  |  |  |  |  |
|  |  | - Check that the operator has identified any applicable national FCTC requirement (depending on routes flown, aerodromes used,…). |  |  |  |  |  |  |
|  |  | - Check that all applicable national requirements are addressed in the FCTC programme. |  |  |  |  |  |  |
|  |  | Check how aerodrome qualifications are followed-up. |  |  |  |  |  |  |
|  |  | - Check how route/area familiarisation training is organised |  |  |  |  |  |  |
|  |  | - Check that route/area training - is based on an assessment of the underlying risks and threats of a route or an area; - is delivered using different methods and tools (FSTD, CBT, familiarisation flight, video training, virtual reality training, self-briefing) based on the type of risks and desired outcome. |  |  |  |  |  |  |
|  |  | - Check the compliance of the criteria for the categorisation of aerodromes (A, B and C). |  |  |  |  |  |  |
|  |  | - Check that specific criteria for the area familiarisation training related to helicopter operations have been taken into account: - mountain environment; - offshore environment; - complex airspace; - areas that are regularly covered by snow and are prone to white-out phenomena during the cruise or landing phase; and - other challenging areas or conditions. |  |  |  |  |  |  |
| 14. | ORO.FC.110 | ED Decision 2014/017/R Flight engineer When a separate flight engineer station is incorporated in the design of an aeroplane, the flight crew shall include one crew member who is suitably qualified in accordance with applicable national rules. |  |  |  |  |  |  |
| 15. | ORO.FC.115 | Regulation (EU) No 965/2012 Crew resource management (CRM) training |  |  |  |  |  |  |
|  |  | (a) Before operating, the flight crew member shall have received CRM training, appropriate to his/her role, as specified in the operations manual. |  |  |  |  |  |  |
|  |  | (b) Elements of CRM training shall be included in the aircraft type or class training and recurrent training as well as in the command course. |  |  |  |  |  |  |
| 16. | AMC1 ORO.FC.115 | ED Decision 2015/022/R CRM training — multi-pilot operations- See table in the Annex 1 to the check list |  |  |  |  |  |  |
|  |  | (a) General |  |  |  |  |  |  |
|  |  | (1) Training environment CRM training should be conducted in the non-operational environment (classroom and computer-based) and in the operational environment (flight simulation training device (FSTD) and aircraft). Tools such as group discussions, team task analysis, team task simulation and feedback should be used. |  |  |  |  |  |  |
|  |  | (2) Classroom training Whenever possible, classroom training should be conducted in a group session away from the pressures of the usual working environment, so that the opportunity is provided for flight crew members to interact and communicate in an environment conducive to learning. |  |  |  |  |  |  |
|  |  | (3) Computer-based training (CBT) Computer-based training should not be conducted as a stand-alone training method but may be conducted as a complementary training method. Complementary training method in the context of EBT: advanced CBT following the aviation blended learning environment, such as virtual reality, chatbots, interactive scenario trainers, etc. may serve as the principal method to deliver training in the non-operational environment. In such case, the classroom training may be the complementary method. |  |  |  |  |  |  |
|  |  | (4) Flight simulation training devices (FSTDs) (i) Whenever practicable, parts of the CRM training should be conducted in FSTDs that reproduce a realistic operational environment and permit interaction. This includes but is not limited to line-oriented flight training (LOFT) scenarios. (ii) If the operator proficiency check is conducted in a FSTD, it should include a line-oriented flight during which a complementary CRM assessment should take place, in conditions that reproduce a realistic operational environment. |  |  |  |  |  |  |
|  |  | (5) Integration into flight crew training CRM principles should be integrated into relevant parts of flight crew training and operations including checklists, briefings, abnormal and emergency procedures. |  |  |  |  |  |  |
|  |  | (6) Combined CRM training for flight crew, cabin crew and technical crew (i) Operators should provide combined training for flight crew, cabin crew and technical crew during recurrent CRM training. (ii) The combined training should address at least: (A) effective communication, coordination of tasks and functions of flight crew, cabin crew and technical crew; and (B) mixed multinational and cross-cultural flight crew, cabin crew and technical crew, and their interaction, if applicable. (iii) The combined training should be expanded to include medical passengers, if applicable to the operation. (iv) Combined CRM training should be conducted by flight crew CRM trainer or cabin crew CRM trainer. (v) There should be an effective liaison between flight crew, cabin crew and technical crew training departments. Provision should be made for transfer of relevant knowledge and skills between flight crew, cabin crew and technical crew CRM trainers. |  |  |  |  |  |  |
|  |  | (7) Management system CRM training should address hazards and risks identified by the operator’s management system described in ORO.GEN.200. |  |  |  |  |  |  |
|  |  | (8) Competency-based CRM training (i) Whenever practicable, the compliance-based approach concerning CRM training may be substituted by a competency-based approach such as evidence-based training. In this context, CRM training should be characterised by a performance orientation, with emphasis on standards of performance and their measurement, and the development of training to the specified performance standards. (ii) CRM training should be an essential element of the alternative training and qualification programme (ATQP) described in ORO.FC.A.245, when the operator applies ATQP. |  |  |  |  |  |  |
|  |  | (9) Contracted CRM training If the operator chooses not to establish its own CRM training, another operator, a third party or a training organisation may be contracted to provide the training in accordance with ORO.GEN.205. In case of contracted CRM training, the operator should ensure that the content of the course covers the specific culture, the type of operations and the associated procedures of the operator. When crew members from different operators attend the same course, the CRM training should be specific to the relevant flight operations and to the trainees concerned. |  |  |  |  |  |  |
|  |  | (b) Initial operator’s CRM training |  |  |  |  |  |  |
|  |  | (1) The flight crew member should complete the initial operator’s CRM training once. When the type of operation of a new operator is not different, the new operator should not be required to provide the initial operator’s CRM training to this flight crew member a second time. |  |  |  |  |  |  |
|  |  | (2) The initial training should cover all elements specified in Table 1 of (g). |  |  |  |  |  |  |
|  |  | (c) Operator conversion course — CRM training When the flight crew member undertakes a conversion course with a change of aircraft type or when joining an operator, elements of CRM training should be integrated into all appropriate phases of the operator’s conversion course, as specified in Table 1 of (g). |  |  |  |  |  |  |
|  |  | (d) Annual recurrent CRM training |  |  |  |  |  |  |
|  |  | (1) Annual recurrent CRM training should be provided in such a way that all CRM training elements specified for the annual recurrent training in Table 1 of (g) are covered over a period not exceeding 3 years. |  |  |  |  |  |  |
|  |  | (2) Operators should update their CRM recurrent training programme over a period not exceeding 3 years. The revision of the programme should take into account information from the operator’s management system including the results of the CRM assessment. |  |  |  |  |  |  |
|  |  | (e) Command course — CRM training The operator should ensure that elements of CRM training are integrated into the command course, as specified in Table 1 of (g). |  |  |  |  |  |  |
|  |  | (f) Training elements The CRM training elements to be covered are specified in Table 1 of (g). The operator should ensure that the following aspects are addressed: |  |  |  |  |  |  |
|  |  | (1) Automation and philosophy on the use of automation (i) The CRM training should include training in the use and knowledge of automation, and in the recognition of systems and human limitations associated with the use of automation. The operator should, therefore, ensure that the flight crew member receives training on: (A) the application of the operations policy concerning the use of automation as stated in the operations manual; and (B) system and human limitations associated with the use of automation, giving special attention to issues of mode awareness, automation surprises and over-reliance including false sense of security and complacency. (ii) The objective of this training should be to provide appropriate knowledge, skills and attitudes for managing and operating automated systems. Special attention should be given to how automation increases the need for crews to have a common understanding of the way in which the system performs, and any features of automation that make this understanding difficult. Easy (iii) If conducted in an FSTD, the training should include automation surprises of different origin (system- and pilot-induced). |  |  |  |  |  |  |
|  |  | (2) Monitoring and intervention Flight crew should be trained in CRM-related aspects of operation monitoring before, during and after flight, together with any associated priorities. This CRM training should include guidance to the pilot monitoring on when it would be appropriate to intervene, if felt necessary, and how this should be done in a timely manner. Reference should be made to the operator procedures for structured intervention as specified in the operations manual. |  |  |  |  |  |  |
|  |  | (3) Resilience development CRM training should address the main aspects of resilience development. The training should cover: (i) Mental flexibility Flight crew should be trained to: (A) understand that mental flexibility is necessary to recognise critical changes; (B) reflect on their judgement and adjust it to the unique situation; (C) avoid fixed prejudices and over-reliance on standard solutions; and (D) remain open to changing assumptions and perceptions. (ii) Performance adaptation Flight crew should be trained to: (A) mitigate frozen behaviours, overreactions and inappropriate hesitation; and (B) adjust actions to current conditions. |  |  |  |  |  |  |
|  |  | (4) Surprise and startle effect CRM training should address unexpected, unusual and stressful situations. The training should cover: (i) surprises and startle effects; and (ii) management of abnormal and emergency situations, including: (A) the development and maintenance of the capacity to manage crew resources; (B) the acquisition and maintenance of adequate automatic behavioural responses; and (C) recognising the loss and re-building situation awareness and control. |  |  |  |  |  |  |
|  |  | (5) Cultural differences CRM training should cover cultural differences of multinational and cross-cultural crews. This includes recognising that: (i) different cultures may have different communication specifics, ways of understanding and approaches to the same situation or problem; (ii) difficulties may arise when crew members with different mother tongue communicate (iii) cultural differences may lead to different methods for identifying a situation and solving a problem. |  |  |  |  |  |  |
|  |  | (6) Operator’s safety culture and company culture CRM training should cover the operator’s safety culture, its company culture, the type of operations and the associated procedures of the operator. This should include areas of operations that may lead to particular difficulties or involve unusual hazards. |  |  |  |  |  |  |
|  |  | (7) Case studies (i) CRM training should cover aircraft type-specific case studies, based on the information available within the operator’s management system, including: (A) accident and serious incident reviews to analyse and identify any associated non-technical causal and contributory factors, and instances or examples of lack of CRM; and (B) analysis of occurrences that were well managed. (ii) If relevant aircraft type-specific or operator-specific case studies are not available, the operator should consider other case studies relevant to the scale and scope of its operations. |  |  |  |  |  |  |
|  |  | (g) CRM training syllabus Table 1 below specifies which CRM training elements should be covered in each type of training. The levels of training in Table 1 can be described as follows: |  |  |  |  |  |  |
|  |  | (1) ‘Required’ means training that should be instructional or interactive in style to meet the objectives specified in the CRM training programme or to refresh and strengthen knowledge gained in a previous training. |  |  |  |  |  |  |
|  |  | (2) ‘In-depth’ means training that should be instructional or interactive in style taking full advantage of group discussions, team task analysis, team task simulation, etc., for the acquisition or consolidation of knowledge, skills and attitudes. The CRM training elements should be tailored to the specific needs of the training phase being undertaken. |  |  |  |  |  |  |
|  |  | (h) Assessment of CRM skills |  |  |  |  |  |  |
|  |  | (1) Assessment of CRM skills is the process of observing, recording, interpreting and debriefing crews and crew member’s performance using an accepted methodology in the context of the overall performance. |  |  |  |  |  |  |
|  |  | (2) The flight crew member’s CRM skills should be assessed in the operational environment, but not during CRM training in the non-operational environment. Nevertheless, during training in the non-operational environment, feedback from the flight crew CRM trainer or from trainees on individual and crew performance may be given to the crew members concerned. |  |  |  |  |  |  |
|  |  | (3) The assessment of CRM skills should: (i) include debriefing the crew and the individual crew member; (ii) serve to identify additional training, where needed, for the crew or the individual crew member; and (iii) be used to improve the CRM training system by evaluating de-identified summaries of all CRM assessments. |  |  |  |  |  |  |
|  |  | (4) Prior to the introduction of CRM skills assessment, a detailed description of the CRM methodology, including the required CRM standards and the terminology used for the assessment, should be published in the operations manual. |  |  |  |  |  |  |
|  |  | (5) Methodology of CRM skills assessment The assessment should be based on the following principles: (i) only observable behaviours are assessed; (ii) the assessment should positively reflect any CRM skills that result in enhanced safety; and (iii) assessments should include behaviour that results in an unacceptable reduction in safety margin. |  |  |  |  |  |  |
|  |  | (6) Operators should establish procedures, including additional training, to be applied in the event that flight crew members do not achieve or maintain the required CRM standards. |  |  |  |  |  |  |
| 17. | AMC2 ORO.FC.115 | ED Decision 2015/022/R CRM training — single-pilot operations |  |  |  |  |  |  |
|  |  | (a) For single-pilot helicopter operations with technical crew, AMC1 ORO.FC.115 should be applied. |  |  |  |  |  |  |
|  |  | (b) For single-pilot operations other than those specified in (a), AMC1 ORO.FC.115 should be applied with the following differences: |  |  |  |  |  |  |
|  |  | (1) Relevant training Training should cover the relevant CRM training, i.e. initial operator’s training, the operator conversion course and recurrent training. |  |  |  |  |  |  |
|  |  | (2) Relevant training elements CRM training should focus on the elements specified in Table 1 of (g) of AMC1 ORO.FC.115 which are relevant to single-pilot operations. Therefore, single-pilot CRM training should include, among others: (i) situation awareness; (ii) workload management; (iii) decision-making; (iv) resilience development; (v) surprise and startle effect; and (vi) effective communication and coordination with other operational personnel and ground services. |  |  |  |  |  |  |
|  |  | (3) Virtual classroom training Notwithstanding (a)(2) of AMC1 ORO.FC.115, classroom training may take place remotely, using a videoconferencing tool. The tool should permit real-time interaction between the trainees and the trainer, including speech and elements of body language. It should also be capable of transmitting any document to the trainee that the trainer wishes to present. The CRM trainer should establish the list of trainees in advance. Their numbersshould be limited to 6 to ensure a sufficient level of interaction during the training session. . |  |  |  |  |  |  |
|  |  | (4) Operation with ELA2 aircraft Notwithstanding (1) and (2), for operations with ELA2 aircraft the relevant CRM training and its duration should be determined by the operator, based on the aircraft type and the complexity of the operation. |  |  |  |  |  |  |
| 18. | GM1 ORO.FC.115 | ED Decision 2015/022/R General |  |  |  |  |  |  |
|  |  | (a) CRM is the effective utilisation of all available resources (e.g. crew members, aircraft systems, supporting facilities and persons) to achieve safe and efficient operation. |  |  |  |  |  |  |
|  |  | (b) The objective of CRM is to enhance the communication and management skills of the flight crew member concerned. Emphasis is placed on the non-technical knowledge, skills and attitudes of flight crew performance. |  |  |  |  |  |  |
| 19. | GM2 ORO.FC.115 | ED Decision 2015/022/R Training environment, trainers and instructors |  |  |  |  |  |  |
|  |  | (a) Flight crew CRM training can be separated as follows: (1) training in the non-operational environment: (i) classroom; and (ii) computer-based; (2) training in the operational environment: (i) flight simulation training device (FSTD); and (ii) aircraft. |  |  |  |  |  |  |
|  |  | (b) In general, CRM training is provided as follows: (1) classroom training by a flight crew CRM trainer; (2) training in the operational environment by an instructor holding a certificate in accordance with Commission Regulation (EU) No 1178/2011; (3) computer-based training as a self-study training method. If needed, directions concerning CRM-related issues are provided by a flight crew CRM trainer or by an instructor holding a certificate in accordance with Commission Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
| 20. | GM3 ORO.FC.115 | ED Decision 2015/022/R Minimum training times |  |  |  |  |  |  |
|  |  | (a) The following minimum training times are appropriate: (1) multi-pilot operations: (i) combined CRM training: 6 training hours over a period of 3 years, or, for EBT operators, a minimum of 3 training hours within 3 years; and (ii) initial operator’s CRM training: 18 training hours with a minimum of 12 training hours in classroom training; (2) initial operator’s CRM training for single-pilot operations: 6 training hours; and (3) flight crew CRM trainer: (i) basic training: (A) 18 training hours for trainees holding an instructor certificate for complex motor-powered aircraft, as specified in Commission Regulation (EU) No 1178/2011, which includes 25-hour training in teaching and learning; or (B) 30 training hours for trainees who do not hold an instructor certificate as specified in (A); and (ii) refresher training: 6 training hours. |  |  |  |  |  |  |
|  |  | (b) ‘Training hours’ means actual training time excluding breaks and assessment. |  |  |  |  |  |  |
| 21. | GM4 ORO.FC.115 | ED Decision 2015/022/R Design, implementation and evaluation of crm training |  |  |  |  |  |  |
|  |  | The checklist in Table 1 provides guidance on the design, implementation and evaluation of CRM training, and on their incorporation into the operator’s safety culture. Elements of the operator’s management systems and the competency-based approach are incorporated in the checklist. |  |  |  |  |  |  |
|  |  | Step No; Description; Element. 1; Needs analysis; Determine the necessary CRM competencies. Develop CRM training goals. Ensure the organisation is ready for CRM training. 2; Design; Develop CRM training objectives. Determine what to measure and how to measure it. 3; Development; Describe the CRM learning environment. Develop full-scale prototype of training. Validate and modify CRM training. 4; Implementation; Prepare trainees and environment. Set a climate for learning (e.g. practice and feedback). Implement the CRM training programme. 5; Evaluation; Determine training effectiveness. Evaluate CRM training at multiple levels. Revise the CRM training programme to improve effectiveness. 6; Incorporation; Establish an environment where CRM training is positively recognised. Reinforce CRM behaviours in daily work. Provide recurrent CRM training. |
| 22. | GM5 ORO.FC.115 | ED Decision 2015/022/R Resilience development |  |  |  |  |  |  |
|  |  | (a) The main aspects of resilience development can be described as the ability to: (1) learn (‘knowing what has happened’); (2) monitor (‘knowing what to look for’); (3) anticipate (‘finding out and knowing what to expect’); and (4) respond (‘knowing what to do and being capable of doing it’). |  |  |  |  |  |  |
|  |  | (b) Operational safety is a continuous process of evaluation of and adjustment to existing and future conditions. In this context, and following the description in (a), resilience development involves an ongoing and adaptable process including situation assessment, self-review, decision and action. Training in resilience development enables crew members to draw the right conclusions from both positive and negative experiences. Based on those experiences, crew members are better prepared to maintain or create safety margins by adapting to dynamic complex situations. |  |  |  |  |  |  |
|  |  | (c) The training topics in (f)(3) of AMC1 ORO.FC.115 are to be understood as follows: (1) Mental flexibility (i) The phrase ‘understand that mental flexibility is necessary to recognise critical changes’ means that crew members are prepared to respond to situations for which there is no set procedure. (ii) The phrase ‘reflect on their judgement and adjust it to the unique situation’ means that crew members learn to review their judgement based on the unique characteristics of the given circumstances. (iii) The phrase ‘avoid fixed prejudices and over-reliance on standard solutions’ means that crew members learn to update solutions and standard response sets, which have been formed on prior knowledge. (iv) The phrase ‘remain open to changing assumptions and perceptions’ means that crew members constantly monitor the situation, and are prepared to adjust their understanding of the evolving conditions. (2) Performance adaptation (i) The phrase ‘mitigate frozen behaviours, overreactions and inappropriate hesitation’ means that crew members correct improper actions with a balanced response. (ii) The phrase ‘adjust actions to current conditions’ means that crew members’ responses are in accordance with the actual situation. |  |  |  |  |  |  |
| 23. | GM6 ORO.FC.115 | ED Decision 2015/022/R Non-technical skills assessment |  |  |  |  |  |  |
|  |  | (a) NOTECHS (non-technical skills) is a validated method for assessing flight crew CRM skills. The NOTECHS framework consists of four main categories: (1) Cooperation: Cooperation is the ability to work effectively in a crew. (2) Leadership and managerial skills: Effective leadership and managerial skills help to achieve joint task completion within a motivated, fully functioning team through coordination and persuasiveness. (3) Situation awareness: Situation awareness relates to one’s ability to accurately perceive what is in the flight crew compartment and outside the aircraft. It is also one’s ability to comprehend the meaning of different elements in the environment and the projection of their status in the near future. (4) Decision-making: Decision-making is the process of reaching a judgement or choosing an option. |  |  |  |  |  |  |
|  |  | (b) Each of the four categories is subdivided into elements and behavioural markers. The elements are specified in Table 1 with examples of behavioural markers (effective behaviour). The behavioural markers are assessed by a rating scale to be established by the operator. |  |  |  |  |  |  |
|  |  | Category; Element; Behavioural marker (examples). Cooperation; Team building and maintaining; Establishes atmosphere for open communication and participation. Considering others; Takes condition of other crew members into account. Supporting others; Helps other crew members in demanding situations. Conflict solving; Concentrates on what is right rather than who is right. Leadership and managerial skills; Use of authority and assertiveness; Takes initiative to ensure crew involvement and task completion. Maintaining standards; Intervenes if task completion deviates from standards. Planning and coordination; Clearly states intentions and goals. Workload management; Allocates adequate time to complete tasks. Situation awareness; Awareness of aircraft systems; Monitors and reports changes in systems’ states. Awareness of external environment; Collects information about environment (position, weather and traffic). Anticipation; Identifies possible future problems. Decision-making; Problem definition and diagnosis; Reviews causal factors with other crew members. Option generation; States alternative courses of action. Asks other crew members for options. Risk assessment and option selection; Considers and shares estimated risk of alternative courses of action. Outcome review; Checks outcome against plan. |
| 24. | GM7 ORO.FC.115 | ED Decision 2015/022/R Flight crew crm trainer assessment |  |  |  |  |  |  |
|  |  | (a) For assessing flight crew CRM trainers, the operator may nominate experienced flight crew CRM trainers who have demonstrated continued compliance with the provisions for a flight crew CRM trainer and capability in that role for at least 3 years. |  |  |  |  |  |  |
|  |  | (b) An operator that does not have the resources to conduct the assessment may employ a contractor. The standard as regards the assessment is confirmed on a 3-year basis by the operator. |  |  |  |  |  |  |
|  |  | (c) The checklist in Table 1 provides guidance on the assessment of a flight crew CRM trainer. If a flight crew CRM trainer is competent in his/her role, the response to the questions in Table 1 should be ‘yes’. When answering the questions in Table 1, justifications and examples related to the responses given should be provided. |  |  |  |  |  |  |
|  |  | Questions to assess a flight crew CRM trainer; Response yes/no. Did the CRM trainer demonstrate the knowledge required for the role?. Did the CRM trainer support CRM concepts?. Did the CRM trainer encourage trainees to participate, share their experiences and self-analyse?. Did the CRM trainer identify and respond to the trainees’ needs relative to expertise/experience?. Did the CRM trainer show how CRM is integrated in technical training and line operations?. Did the CRM trainer incorporate company CRM standards when appropriate?. Did the CRM trainer identify and discuss the non-technical reasons involved in accidents, incidents and events included in case studies?. Did the CRM trainer regularly check for understanding and resolve ambiguities?. Did the CRM trainer demonstrate effective instruction and facilitation skills?. |
| 25. | GM8 ORO.FC.115 | Virtual classroom training — single-pilot operations |  |  |  |  |  |  |
|  |  | (a) A successful virtual classroom training relies on the ability of the trainer to make best use of the associated technologies in the context of CRM training. The flight crew CRM trainer may need to receive appropriate training covering the following: (1) learning style; (2) teaching method associated with virtual classroom instruction, such as videoconferencing, and a familiarisation with the virtual classroom instruction system in use, including management of time, training media and equipment and tools. |  |  |  |  |  |  |
|  |  | (b) The assessment of CRM skills may be used by the operator to improve the CRM training system by evaluating de-identified summaries of all CRM assessments. |  |  |  |  |  |  |
|  |  | (c) The requirement of ORO.GEN.140 for the operator to grant access to the competent authority also applies to the virtual classroom training. |  |  |  |  |  |  |
|  |  | (d) More information on virtual classroom training is provided in the EASA Guidance for allowing virtual classroom instruction and distance learning. |  |  |  |  |  |  |
|  |  | - Check that CBT is not used as a stand-alone method. |  |  |  |  |  |  |
|  |  | - Check that the policy of the operator is to conduct CRM training in a non-operational environment (classroom and CBT) and operational environment (FSTD). |  |  |  |  |  |  |
|  |  | - In case the OPC is conducted in an FSTD, check that it includes a line-oriented flight during which a complementary CRM assessment takes place. |  |  |  |  |  |  |
|  |  | - Check that combined RCM training with CC/TC (if applicable) is foreseen and addresses the following: - effective communication, coordination of tasks and functions of FC, CC and TC; and - mixed multinational and cross-cultural FC, CC and TC, and their interaction. |  |  |  |  |  |  |
|  |  | - Il the operator contracts CRM training, check that operator’s policy foresees the verification of the adequacy of the content of the course. |  |  |  |  |  |  |
|  |  | - Check the content of the initial CRM training syllabus (see table 1 of para (g) of AMC1 ORO.FC.115). |  |  |  |  |  |  |
|  |  | - Check that the duration is specified and adequate |  |  |  |  |  |  |
|  |  | - Check content of the conversion course CRM training syllabus (see table 1 of para (g) of AMC1 ORO.FC.115). |  |  |  |  |  |  |
|  |  | - Check content of the annual recurrent CRM training syllabus and in particular that it covers all the elements of table 1 of para (g) of AMC1 ORO.FC.115 over a period not exceeding 3 Y.. |  |  |  |  |  |  |
|  |  | - Check that the operator updates its CRM training programme at a frequency not exceeding 3Y. |  |  |  |  |  |  |
|  |  | - Check content of the command course CRM training syllabus (see table 1 of para (g) of AMC1 ORO.FC.115). |  |  |  |  |  |  |
|  |  | - Check that a methodology has been established, including the required CRM standards and terminology used. |  |  |  |  |  |  |
|  |  | - Check the consistency of the methodology with the forms used to conduct this assessment (e.g. line check form) - Check that it is based on an adequate grading system. |  |  |  |  |  |  |
|  |  | - Check that it foresees the use of de-identified summaries of all CRM assessments to improve the CRM training system. |  |  |  |  |  |  |
|  |  | - Check that a training on the defined methodology is foreseen for the staff conducting assessment of CRM skills (e.g. line checkers) |  |  |  |  |  |  |
|  |  | - Check that a procedure has been established covering cases when a FC does not achieve or maintain the required CRM standards. |  |  |  |  |  |  |
| 26. | ORO.FC.120 | Regulation (EU) No 965/2012 Operator conversion training |  |  |  |  |  |  |
|  |  | (a) In the case of aeroplane or helicopter operations, the flight crew member shall complete the operator conversion training course before commencing unsupervised line flying: (1) when changing to an aircraft for which a new type or class rating is required; (2) when joining an operator. |  |  |  |  |  |  |
|  |  | (b) The operator conversion training course shall include training on the equipment installed on the aircraft as relevant to flight crew members’ roles. |  |  |  |  |  |  |
| 27. | AMC1 ORO.FC.120 | Operator conversion training for non-commercial operations with complex motor-powered aircraft (ncc) |  |  |  |  |  |  |
|  |  | (a) General |  |  |  |  |  |  |
|  |  | (1) The operator conversion training should include: (i) ground training, including the following: (A) aircraft systems; (B) normal procedures, which include flight planning, ground-handling and flight operations, including performance, mass and balance, fuel schemes, selection of alternates, and ground de-icing/anti-icing; (C) abnormal and emergency procedures, which include pilot incapacitation, as applicable; (D) a review of relevant samples of accidents/incidents and occurrences to increase awareness of the occurrences that may be relevant for the intended operation; (ii) emergency and safety equipment training and checking, including survival equipment training (completed before operating on any passenger-carrying flight); (iii) passenger handling for operations where no cabin crew is carried; and (iv) a minimum number of sectors and/or flight hours operated under the supervision of a flight crew member nominated by the operator, to demonstrate the standard of qualification specified in the operator’s manual. |  |  |  |  |  |  |
|  |  | (2) The operator conversion course may be combined with a new type rating course, as required by Commission Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
|  |  | (3) The conversion training should ensure that each flight crew member: (i) has been trained to competency on the emergency and safety equipment installed on the aircraft they are to operate; and (ii) is competent in the operating procedures and the use of checklists used by the operator. |  |  |  |  |  |  |
|  |  | (b) Emergency and safety equipment training should: |  |  |  |  |  |  |
|  |  | (1) take place in conjunction with cabin crew and technical crew as far as practicable. Emphasis should be placed on the importance of effective coordination and two-way communication between crew members in various emergency situations; |  |  |  |  |  |  |
|  |  | (2) address the operational procedures of rescue and emergency services; and |  |  |  |  |  |  |
|  |  | (3) cover the items of point (a)(2) of AMC1 ORO.FC.130. |  |  |  |  |  |  |
| 28. | AMC2 ORO.FC.120 | Form of operations — single-pilot helicopters |  |  |  |  |  |  |
|  |  | The training for conversion from single-pilot operations to multi-pilot operations and vice versa on a given helicopter type, as specified in point FCL.725(d)(2) of Annex I (Part-FCL) to Regulation (EU) No 1178/2011, should take into account all of the following: (a) the SOPs of the operator; (b) the flight crew member’s previous trainings and experience. |  |  |  |  |  |  |
| 29. | AMC3 ORO.FC.120 | SPO operator conversion course — ground training |  |  |  |  |  |  |
|  |  | (a) General The operator conversion training should include ground training and checking, including all of the following: (1) aircraft systems, (2) normal procedures, which include flight planning ground-handling and flight operations, including performance, mass and balance, fuel schemes selection of alternates, and ground de-icing/anti-icing; (3) abnormal and emergency procedures, which include pilot incapacitation as applicable; (4) a review of relevant samples of accident/incident and occurrences to increase awareness of the occurrences that may be relevant for the intended operation. |  |  |  |  |  |  |
|  |  | SPECIALISED OPERATIONS If a flight crew member undergoes training with regard to SOPs related to a specialised operation, either as part of an equipment and procedure training or a conversion training, the following should apply: (b) Initial training for a given specialised operation (1) In-depth training should achieve competence in carrying out normal, abnormal and emergency procedures, covering the SOPs associated with the specialised task. (2) The training should include ground training associated with the specialised task, completed before any flight training in an aircraft commences. (3) If one or more task specialists are on board, the training should include emergency and safety equipment training, completed before any flight training in an aircraft commences. The training should ensure that all emergency equipment can be used timely and efficiently, that an emergency evacuation and first aid can be conducted, taking into account the training and operating procedures of the task specialist(s). (4) Unless the flight crew member has significant experience in similar specialised operations as defined in the operations manual, the training should include aircraft/FSTD training associated with the specialised task. |  |  |  |  |  |  |
|  |  | (c) Initial training and experience for any level of HEC and HESLO operations: AMC1 SPO.SPEC.HEC.100 and AMC1 SPO.SPEC.HESLO.100 should apply in combination with point (b) above. |  |  |  |  |  |  |
|  |  | (d) Training when changing operators (1) The training should focus on the elements of the SOPs that are specific to the operator. (2) The operator should determine the amount of training required in the operator’s conversion course in accordance with the standards of qualification and experience specified in the operations manual, taking into account the flight crew member’s previous training and experience in the given specialised operation and in similar operations. |  |  |  |  |  |  |
|  |  | (e) Training when changing specialised operations within the same operator, with previous experience of the specialised operation: point (d) above should apply. |  |  |  |  |  |  |
|  |  | (f) Training when changing types or variants: The training should focus on the elements of the SOPs that are specific to the type or variant. The operator should assess whether the flight crew should require ground training, aircraft/FSTD training or both, when changing type or variants within the framework of the same specialised operations. The assessment should take the following into account: (1) the validity of the flight crew type rating; (2) the experience and recency of the flight crew on the type or variant; (3) whether any type or variant specific procedures exist; (4) differences in equipment related to the specialised operations; (5) differences in limitations or procedures related to the specialised operations. |  |  |  |  |  |  |
| 30. | GM1 ORO.FC.120 | Standard operating procedures for multi-pilot operations — single-pilot helicopters MCC training is generic to all types. A pilot holding a certificate of completion of MCC training requires additional training to implement the multi-pilot SOPs of a given helicopter type. |  |  |  |  |  |  |
| 31. | AMC1 ORO.FC.120&130 | ED Decision 2019/005/R For the operation of aeroplanes with a maximum cruising altitude above FL300, training elements from the following table should be integrated into: |  |  |  |  |  |  |
|  |  | (a) operator conversion training; and |  |  |  |  |  |  |
|  |  | (b) recurrent training at least every 12 calendar months, such that all elements are covered over a period not exceeding 3 years: See Table |  |  |  |  |  |  |
|  |  | Element; Theoretical Knowledge; Practical training. Basic flight physics principles concerning flight at high altitude, with a particular emphasis on the relative proximity of the critical Mach number and the stall, pitch behaviour, and an understanding of the reduced stall angle of attack when compared with low-altitude flight.; •; •. Interaction of the automation (autopilot, flight director, auto-throttle/auto-thrust) and the consequences of failures inducing disconnection of the automation.; •; •. Consequences of an unreliable airspeed indication and other failures at high altitude and the need for the flight crew to promptly identify the failure and react with appropriate (minimal) control inputs to keep the aircraft in a safe envelope.; •; •. Degradation of fly-by-wire (FBW) flight control laws/modes and its consequence on aircraft stability and flight envelope protections, including stall warnings.; •; •. Practical training, using appropriate simulators, on manual handling at high altitude in normal and non-normal flight control laws/modes, with particular emphasis on pre-stall buffet, the reduced stall angle of attack when compared with low-altitude flight and the effect of pitch inputs on the aircraft trajectory and energy state.; •. The requirement to promptly and accurately apply the stall recovery procedure, as provided by the aircraft manufacturer, at the first indication of an impending stall. Differences between high-altitude and low-altitude stalls must be addressed.; •; •. Procedures for taking over and transferring manual control of the aircraft, especially for FBW aeroplanes with independent side-sticks.; •; •. Task sharing and crew coordination in high workload/stress conditions with appropriate call-out and acknowledgement to confirm changes to the aircraft flight control law/mode.; •; •. |
|  |  | - Check that the duration of the training is defined and adequate |  |  |  |  |  |  |
| 32. | ORO.FC.125 | Regulation (EU) No 2021/2237 Differences training, familiarisation, equipment and procedure training |  |  |  |  |  |  |
|  |  | (a) Flight crew members shall complete differences training or familiarisation when required by Annex I (Part- FCL) to Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
|  |  | (b) Flight crew members shall complete equipment and procedure training when changing equipment or changing procedures requiring additional knowledge on types or variants currently operated. |  |  |  |  |  |  |
|  |  | (c) The operations manual shall specify when such differences training or familiarisation or equipment and procedure training is required. |  |  |  |  |  |  |
| 33. | AMC1 ORO.FC.125 | ED Decision 2014/017/R General (a) Differences training requires additional knowledge and training on the aircraft or an appropriate training device. It should be carried out: (1) in the case of aeroplanes, when operating another variant of an aeroplane of the same type or another type of the same class currently operated; or (2) in the case of helicopters, when operating a variant of a helicopter currently operated. |  |  |  |  |  |  |
|  |  | (b) Familiarisation requires only the acquisition of additional knowledge. It should be carried out when operating another helicopter or aeroplane of the same type; |  |  |  |  |  |  |
| 34. | AMC2 ORO.FC.125 | Operator difference requirements (ODRs) When defining the needs for differences training, familiarisation or equipment training, the operator should make use of the concept of ODRs and of the methodology described in AMC1 ORO.FC.140(a), including the ODRs tables. |  |  |  |  |  |  |
|  |  | Form of operations — single-pilot helicopters If the differences training, familiarisation, equipment or procedure training includes the conversion from single-pilot operations to multi-pilot operations and vice versa, it should take into account all elements described in AMC2 ORO.FC.120. |  |  |  |  |  |  |
| 35. | GM1 ORO.FC.125 | Operator difference requirements (ODRs) The ODRs tables may result in different training programmes, depending on the training needs, regardless of the ‘base aircraft’ used to establish the table (e.g. the trainee may know the ‘other aircraft’ and be trained towards the ‘base aircraft’). |  |  |  |  |  |  |
| 36. | AMC1 ORO.FC.125(b) | Specialised operations If the differences training, familiarisation, equipment and procedure training includes training for SOPs related to a specialised operation, points (b) to (f) of AMC3 ORO.FC.120 should apply. |  |  |  |  |  |  |
| 37. | GM1 ORO.FC.125(b) | General Introducing a change of equipment and/or procedures on types or variants currently operated may require additional knowledge or additional training on the aircraft, or an appropriate training device, or both. |  |  |  |  |  |  |
| 38. | GM2 ORO.FC.125(b) | Procedure training — standard operating procedures for multi-pilot operations — single-pilot helicopters MCC training is generic to all types. A pilot holding a certificate of completion of MCC training requires additional procedures training to implement the multi-pilot SOPs of a given single-pilot helicopter type. |  |  |  |  |  |  |
| 39. | ORO.FC.130 | Regulation (EU) 2021/2237 Recurrent training and checking |  |  |  |  |  |  |
|  |  | (a) Each flight crew member shall complete annual recurrent flight and ground training relevant to the type or variant, and associated equipment of aircraft on which he or she operates, including training on the location and use of all emergency and safety equipment carried on board the aircraft. |  |  |  |  |  |  |
|  |  | (b) Each flight crew member shall be periodically checked to demonstrate competence in carrying out normal, abnormal and emergency procedures. |  |  |  |  |  |  |
| 40. | AMC1 ORO.FC.130 | Recurrent training and checking to demonstrate competence for non-commercial operations with complex motor-powered aircraft (NCC) |  |  |  |  |  |  |
|  |  | (a) Recurrent training Recurrent training should comprise the following: (1) Ground training The ground training programme should include: (i) aircraft systems; (ii) normal procedures, which include flight planning, ground-handling and flight operations, including performance, mass and balance, fuel schemes, selection of alternates, and ground de-icing/anti-icing; (iii) abnormal and emergency procedures, which include pilot incapacitation as applicable; (iv) a review of relevant samples of accidents/incidents and occurrences to increase awareness of the occurrences that may be relevant for the intended operation; |  |  |  |  |  |  |
|  |  | (2) Emergency and safety equipment training (i) Emergency and safety equipment training may be combined with emergency and safety equipment checking and should be conducted in an aircraft or a suitable alternative training device. (ii) Every year the emergency and safety equipment training programme should include the following: (A) actual donning of a life jacket, where fitted; (B) actual donning of protective breathing equipment, where fitted; (C) actual handling of fire extinguishers of the type used; (D) instruction on the location and use of all emergency and safety equipment carried on the aircraft; and (E) instruction on the location and use of all types of exits. |  |  |  |  |  |  |
|  |  | (3) Elements of CRM as specified in Table 1 of AMC1 ORO.FC.115 should be integrated into all appropriate phases of recurrent training. |  |  |  |  |  |  |
|  |  | (4) Aircraft/FSTD training (i) The aircraft/FSTD training programme should be established in such a way that all the major failures of aircraft systems and associated procedures will have been covered in the preceding 3-year period. (ii) When engine-out manoeuvres are carried out in an aircraft, the engine failure should be simulated. (iii) When an FSTD is not available or accessible, the operator should establish mitigating measures to ensure that an adequate level of safety is maintained when conducting the training or checking in an aircraft. If one or more of the major failures cannot be practised in the aircraft because of their associated risks or because of environmental considerations, the failure(s) may be partially replicated for crew training purposes using pre-briefed, risk-assessed measures that avoid degrading the aircraft’s performance below a predetermined level, and which permit immediate reversion to normal operating conditions. |  |  |  |  |  |  |
|  |  | (b) Periodic check to demonstrate competence (1) Each flight crew member should complete the periodic check as part of the normal crew complement. (2) Periodic demonstrations of competence should be conducted every 12 months and may be combined with the proficiency check required by Commission Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
| 41. | GM1 ORO.FC.130 | Periodic checks |  |  |  |  |  |  |
|  |  | (a) For CAT operations, the operator proficiency checks and the line checks are both part of the periodic checks. For EBT operators, the EBT module and the line evaluations of competence are both part of the periodic checks. (b) For SPO operations, the operator proficiency checks are part of the periodic checks. (c) For non-CAT operations, the periodic checks may include a line check. |  |  |  |  |  |  |
| 42. | AMC1 ORO.FC.130(a) | Operations with variations in aircraft configuration |  |  |  |  |  |  |
|  |  | AMC1 ORO.FC.140(a) should be used to determine the recurrent ground training and checking relevant to variations in aircraft configuration, if all of the following apply: (a) the pilot operates variations in aircraft configuration; (b) the aircraft operated do not all belong to the same group of types defined under ORO.FC.140(b); and (c) credit (as defined in point (a)(4) of AMC1 ORO.FC.140(a)) is sought. |  |  |  |  |  |  |
| 43. | ORO.FC.135 | Regulation (EU) No 965/2012 Pilot qualification to operate in either pilot's seat |  |  |  |  |  |  |
|  |  | Flight crew members who may be assigned to operate in either pilot’s seat shall complete appropriate training and checking as specified in the operations manual. |  |  |  |  |  |  |
| 44. | AMC1 ORO.FC.135 | General The training and checking for pilot qualification to operate in either pilot’s seat should include any safety-critical items as specified in the operations manual where the action to be taken by the pilot is different depending on which seat they occupy. |  |  |  |  |  |  |
|  |  | Non-commercial operations with complex motor-powered aircraft (NCC) Training should be arranged so that all such items will have been covered in the preceding 3-year period. |  |  |  |  |  |  |
| 45. | ORO.FC.140 | Regulation (EU) 2021/2237 Operation on more than one type or variant |  |  |  |  |  |  |
|  |  | (a) Flight crew members that operate more than one type or variant of aircraft shall comply with the requirements prescribed in this Subpart for each type or variant, unless credits related to the training, checking, and recent experience requirements are defined in the mandatory part of the operational suitability data established in accordance with Regulation (EU) No 748/2012 for the relevant types or variants. |  |  |  |  |  |  |
|  |  | (b) The operator may define groups of single-engined helicopter types. An operator proficiency check on one type shall be valid for all the other types within the group if both of the following conditions are met: (1) the group either includes only single-engined turbine helicopters operated under VFR or it includes only single-engined piston helicopters operated under VFR; (2) for CAT operations, at least two operator proficiency checks per type shall be conducted within a 3-year cycle. |  |  |  |  |  |  |
|  |  | (c) For specialised operations, elements of the aircraft/FSTD training and operator proficiency check that cover the relevant aspects associated with the specialised task and are not related to the type or group of types may be credited towards the other groups or types, based on a risk assessment performed by the operator. |  |  |  |  |  |  |
|  |  | (d) For operations on more than one helicopter type or variant that are used for conducting sufficiently similar operations, if line checks rotate between types or variants, each line check shall revalidate the line check for the other helicopter types or variants. |  |  |  |  |  |  |
|  |  | (e) Appropriate procedures and any operational restrictions shall be specified in the operations manual for any operation on more than one type or variant. |  |  |  |  |  |  |
| 46. | GM1 ORO.FC.140 | General |  |  |  |  |  |  |
|  |  | (a) The concept of operating more than one type or variant depends on the experience, knowledge and ability of the operator and the flight crew concerned. (b) The first consideration is whether operations on one aircraft type or variant allow the safe operation of all other types and variants. (c) The second consideration is whether and how adequate training to address potential confusion and increased workload caused by the operation of several types or variants is achieved. |  |  |  |  |  |  |
| 47. | AMC1 ORO.FC.140(a) | General |  |  |  |  |  |  |
|  |  | (a) Terminology The terms used in the context of operation on more than one type or variant have the following meaning: (1) ‘Base aircraft’ refers to an aircraft used as a reference to compare differences with another aircraft. (2) ‘Variant’ refers to an aircraft or a group of aircraft within the same pilot type or class rating that has differences with the base aircraft and requires differences training or familiarisation. (3) A ‘variation in aircraft configuration’ refers to an aircraft or a group of aircraft within the same variant that has differences with the base aircraft and requires equipment and procedure training. (4) ‘Credit’ refers to the recognition of recurrent training, checking or recent experience based on commonalities between aircraft. (5) ‘Operator difference requirements (ODRs)’ refer to a formal description of differences between types or variants or aircraft configurations flown by a particular operator. (6) ‘Training’ refers to differences training, familiarisation and equipment training. (7) ‘Currency’ refers to the recurrent training on types and variants. |  |  |  |  |  |  |
|  |  | (b) Scope of ODRs The operator should use the ODRs methodology, a means of evaluating aircraft differences and similarities, in order to define the training and checking in the following cases: (1) for the introduction of a change of equipment on a type or variant currently operated; (2) for the introduction of a new variant within a type or class currently operated; (3) for the recurrent training and checking of variations in aircraft configuration. The operator may define credit based on ODRs tables; (4) for the operation of more than one type or variant when credit is sought, in which case all of the following should apply: (i) All training, checking and currency requirements should be completed independently for each type or variant unless credits have been established by using ODRs tables. (ii) All recent experience requirements should be completed independently for each type unless credits have been established by using ODRs tables. (iii) The operator may define credit based on ODRs tables that should not be less restrictive than the OSD. |  |  |  |  |  |  |
|  |  | (c) ODRs methodology (1) The operator should conduct a detailed evaluation of the differences or similarities of the aircraft concerned in order to establish appropriate procedures or operational restrictions. This evaluation should be based on the OSD for the relevant types or variants and should be adapted to the operator’s specific variations in aircraft configuration. This evaluation should take into account all of the following: (i) the level of technology; (ii) operational procedures; and (iii) handling characteristics. |  |  |  |  |  |  |
|  |  | (2) ODRs tables The operator should first nominate one aircraft as the base aircraft from which to show differences with the second aircraft type or variant or variation in aircraft configuration, the ‘difference aircraft’, in terms of technology (systems), procedures, pilot handling and aircraft management. These differences, known as ODRs, preferably presented in tabular format, constitute part of the justification for operating more than one type or variant and also the basis for the associated differences/familiarisation or reduced type rating training for the flight crew. |  |  |  |  |  |  |
|  |  | (3) The ODRs tables should be presented as follows: |  |  |  |  |  |  |
|  |  | GENERAL OPERATOR DIFFERENCE REQUIREMENTS TABLE. DIFFERENCE AIRCRAFT: BASE AIRCRAFT:; COMPLIANCE METHOD. TRAINING; CHECKING/ CURRENCY. General; Differences; Flt char; Proc chg; A; B; C; D; E; FLT CHK; CURRENCY. GENERAL; Range ETOPS certified; No; Yes; CBT. DIMENSIONS; Configuration per AFM, FCOM; Yes; No; CBT. |
|  |  | SYSTEM OPERATOR DIFFERENCE REQUIREMENTS TABLE. DIFFERENCE AIRCRAFT: BASE AIRCRAFT:; COMPLIANCE METHOD. TRAINING; CHECKING/ CURRENCY. System; Differences; Flt char; Proc chg; A; B; C; D; E; FLT CHK; CURRENCY. 21 – AIR CONDITIONING; CONTROLS AND INDICATORS: - Panel layout; No; Yes; HO. 21 – AIR CONDITIONING; PACKS: - Switch type - Automatically controlled - Reset switch for both packs; No; Yes; CBT. |
|  |  | MANOEUVRE OPERATOR DIFFERENCE REQUIREMENTS TABLE. DIFFERENCE AIRCRAFT: BASE AIRCRAFT:; COMPLIANCE METHOD. TRAINING; CHECKING/ CURRENCY. Manoeuvre; Differences; Flt char; Proc chg; A; B; C; D; E; FLT CHK; CURRENCY. Exterior Preflight; Minor differences; No; No; HO. Preflight; Differences due to systems, ECL; No; Yes; CBT; FTD. Normal take-off; FBW handling v conventional; AFDS TAKE-OFF: Autothrottle engagement FMA indications; No; Yes; CBT; FFS. |
|  |  | (4) Compilation of ODRs tables (i) ODRs 1: General The general characteristics of the candidate aircraft are compared with the base aircraft with regard to: (A) general dimensions and aircraft design (number and type of rotors, wing span or category); (B) flight deck general design; (C) cabin layout; (D) engines (number, type and position); (E) limitations (flight envelope). (ii) ODRs 2: Systems Consideration is given to differences in design between the candidate aircraft and the base aircraft. For this comparison, the Air Transport Association (ATA) 100 index is used. This index establishes a system and subsystem classification and then an analysis is performed for each index item with respect to the main architectural, functional and operations elements, including controls and indications on the systems control panel. (iii) ODRs 3: Manoeuvres Operational differences encompass normal, abnormal and emergency situations and include any change in aircraft handling and flight management. It is necessary to establish a list of operational items for consideration on which an analysis of differences can be made. The operational analysis should take the following into account: (A) flight deck dimensions (size, cut-off angle and pilot eye height); (B) differences in controls (design, shape, location and function); (C) additional or altered function (flight controls) in normal or abnormal conditions; (D) handling qualities (including inertia) in normal and in abnormal configurations; (E) aircraft performance in specific manoeuvres; (F) aircraft status following failure; (G) management (e.g. ECAM, EICAS, navaid selection, automatic checklists). (iv) Once the differences for ODRs 1, ODRs 2 and ODRs 3 have been established, the consequences of differences evaluated in terms of flight characteristics (FLT CHAR) and change of procedures (PROC CHNG) should be entered into the appropriate columns. (v) Difference levels — crew training, checking and currency (A) In order to operate more than one type or variant, the operator should establish crew training, checking and currency requirements. This may be done by applying the coded difference levels from the table in point (d)(2) to the compliance method column of the ODRs tables. (B) Differences identified in the ODRs tables as impacting flight characteristics or procedures, should be analysed in the corresponding ATA section of the ODRs manoeuvres. Normal, abnormal and emergency situations should be addressed accordingly. |  |  |  |  |  |  |
|  |  | (d) Difference levels (1) Difference levels — general Difference levels are used to identify the extent of a difference between a base and a candidate aircraft with reference to the elements described in the ODRs tables. These levels are proportionate to the differences between a base and a candidate aircraft. A range of five difference levels in order of increasing requirements, identified as A through E, are each specified for training, checking, and currency. Difference levels apply when a difference with the potential to affect flight safety exists between a base and a candidate aircraft. Differences may also affect the knowledge, skills, or abilities required from a pilot. If no differences exist, or if differences exist but do not affect flight safety, or if differences exist but do not affect knowledge, skills or abilities, then difference levels are neither assigned nor applicable to pilot qualification. When difference levels apply, each level is based on a scale of differences related to design features, systems, or manoeuvres. In assessing the effects of differences, both flight characteristics and procedures are considered since flight characteristics address handling qualities and performance, while procedures include normal, non-normal and emergency items. Levels for training, checking, and currency are assigned independently, but are linked depending on the differences between a base and candidate aircraft. Training at level E usually identifies that the candidate aircraft is a different type from the base aircraft. |  |  |  |  |  |  |
|  |  | (2) Difference levels are summarised in the table below regarding training, checking, and currency. |  |  |  |  |  |  |
|  |  | DIFFERENCE LEVEL; TRAINING; CHECKING; CURRENCY. A; Self-instruction; Not applicable or integrated with next proficiency check; Not applicable. B; Aided instruction; Task or system check; Self-review. C; System devices; Partial proficiency check using qualified device; Designated system. D; Manoeuvre training devices1 or aircraft to accomplish specific manoeuvres; Partial proficiency check using qualified device1; Designated manoeuvre(s)1. E; FSTDs2 or aircraft; Proficiency check using FSTDs2 or aircraft; As per regulation, using FSTDs2 or aircraft. |
|  |  | Footnote (1): – Aeroplane: FTD level 2, or FFS, or aeroplane – Helicopter: FTD levels 2 and 3, or FFS, or helicopter Footnote (2): – Aeroplane: FFS level C or D, or aeroplane – Helicopter: FSTDs having dual qualification: FFS level B and FTD level 3, or FFS level C or D, or helicopter Training levels A and B require knowledge, levels C and D require additional skills. Training level E means that the differences are such that type rating training is required or, in the context of equipment and procedure training, aircraft/FSTD training and checking is required. |  |  |  |  |  |  |
|  |  | (3) Difference levels — training The training difference levels specified represent the minimum requirements. Devices associated with a higher difference level may be used to satisfy a training differences requirement. |  |  |  |  |  |  |
|  |  | (i) Level A training Level A differences training is applicable to aircraft with differences that can adequately be addressed through self-instruction. Level A training represents a knowledge requirement such that once appropriate information is provided, understanding and compliance can be assumed to be demonstrated. Training needs not covered by level A training may require level B training or higher, depending on the outcome of the evaluations described in the aircraft evaluation process (CS FCD.420). |  |  |  |  |  |  |
|  |  | (ii) Level B training Level B differences training is applicable to aircraft with system or procedure differences that can adequately be addressed through aided instruction. At level B aided instruction, it is appropriate to ensure pilot understanding, emphasise issues, provide a standardised method of presentation of material, or to aid retention of material following training. |  |  |  |  |  |  |
|  |  | (iii) Level C training Level C differences training can only be accomplished through the use of devices capable of systems training. Level C differences training is applicable to variants having ‘part task’ differences that affect skills or abilities as well as knowledge. Training objectives focus on mastering individual systems, procedures, or tasks, as opposed to performing highly integrated flight operations and manoeuvres in ‘real time’. Level C may also require self-instruction or aided instruction of a pilot, but cannot be adequately addressed by a knowledge requirement alone. Training devices are required to supplement instruction to ensure attainment or retention of pilot skills and abilities to accomplish the more complex tasks, usually related to operation of particular aircraft systems. The minimum acceptable training media for level C are interactive computer-based training, cockpit systems simulators, cockpit procedure trainers, part task trainers (such as inertial navigation system (INS), flight management system (FMS), or traffic collision avoidance system (TCAS) trainers), or similar devices. |  |  |  |  |  |  |
|  |  | (iv) Level D training Level D differences training can only be accomplished with devices capable of performing flight manoeuvres and addressing full task differences affecting knowledge, skills, or abilities. Devices capable of flight manoeuvres address full task performance in a dynamic ‘real time’ environment and enable integration of knowledge, skills and abilities in a simulated flight environment, involving combinations of operationally oriented tasks and realistic task loading for each relevant phase of flight. At level D, knowledge and skills to complete necessary normal, non-normal and emergency procedures are fully addressed for each variant. Level D differences training requires mastery of interrelated skills that cannot be adequately addressed by separate acquisition of a series of knowledge areas or skills that are interrelated. However, the differences are not so significant that a full type rating training course is required. If demonstration of interrelationships between the systems was important, the use of a series of separate devices for systems training would not suffice. Training for level D differences requires a training device that has accurate, high-fidelity integration of systems and controls and realistic instrument indications. Level D training may also require manoeuvre visual cues, motion cues, dynamics, control loading or specific environmental conditions. Weather phenomena such as low-visibility conditions or wind shear may or may not be incorporated. Where simplified or generic characteristics of an aircraft type are used in devices to satisfy level D differences training, significant negative training should not occur as a result of the simplification. Appropriate devices as described in CS FCD.415(a), satisfying level D differences training range from those where relevant elements of aircraft flight manoeuvring, performance, and handling qualities are incorporated. The use of a manoeuvre training device or aircraft is limited for the conduct of specific manoeuvres or handling differences, or for specific equipment or procedures. |  |  |  |  |  |  |
|  |  | (v) Level E training Level E differences training is applicable to candidate aircraft that have such significant ‘full task’ differences that a full type rating training course or a type rating training course with credit for previous experience on similar aircraft types is required to meet the training objectives. The training requires a ‘high-fidelity’ environment to attain or maintain knowledge, skills, or abilities that can only be satisfied by the use of FSTDs or the aircraft itself as mentioned in CS FCD.415(a). Level E training, if done in an aircraft, should be modified for safety reasons where manoeuvres can result in a high degree of risk. When level E differences training is assigned, suitable credit or constraints may be applied for knowledge, skills or abilities related to other pertinent aircraft types. The training programme should specify the relevant subjects, procedures or manoeuvres. |  |  |  |  |  |  |
|  |  | (4) Difference levels — checking Differences checking addresses any pertinent pilot testing or checking. Initial and recurrent checking levels are the same unless otherwise specified. It may be possible to satisfactorily accomplish recurrent checking objectives in devices that do not meet the initial checking requirements. In such instances, the applicant may propose for revalidation checks the use of certain devices that do not meet the initial checking requirements. |  |  |  |  |  |  |
|  |  | (i) Level A checking Level A differences checking indicates that no check related to differences is required at the time of differences training. However, a pilot is responsible for knowledge of each variant flown. (ii) Level B checking Level B differences checking indicates that a ‘task’ or ‘systems’ check is required following initial and recurring training. (iii) Level C checking Level C differences checking requires a partial check using a suitable qualified device. A partial check is conducted relative to particular manoeuvres or systems. (iv) Level D checking Level D differences checking indicates that a partial proficiency check is required following both initial and recurrent training. In conducting the partial proficiency check, manoeuvres common to each variant may be credited and need not be repeated. The partial proficiency check covers the specified particular manoeuvres, systems or devices. Level D checking is performed using scenarios that represent a ‘real-time’ flight environment and uses qualified devices permitted for level D training or higher. (v) Level E checking Level E differences checking requires that a full proficiency check be conducted in FSTDs or in an aircraft as mentioned in CS FCD.415(a), following both initial and recurrent training. If appropriate, alternating Level E checking between relevant aircraft is possible and credit may be defined for procedures or manoeuvres based on commonality. Assignment of level E checking requirements alone, or in conjunction with level E currency, does not necessarily result in assignment of a separate type rating. |  |  |  |  |  |  |
|  |  | (5) Difference levels — currency Differences currency addresses any currency and re-currency levels. Initial and recurrent currency levels are the same unless otherwise specified. |  |  |  |  |  |  |
|  |  | (i) Level A currency Level A currency is common to each aircraft and does not require separate tracking. Maintenance of currency in any aircraft suffices for any other variant within the same type rating. |  |  |  |  |  |  |
|  |  | (ii) Level B currency Level B currency is ‘knowledge-related’ currency, typically achieved through self-review by individual pilots. |  |  |  |  |  |  |
|  |  | (iii) Level C currency (A) Level C currency is applicable to one or more designated systems or procedures and it relates to both skill and knowledge requirements. When level C currency applies, any pertinent lower-level currency is also to be addressed. ( B) Re-establishing level C currency When currency is lost, it may be re-established by completing required items using a device equal to or higher than that specified for level C training and checking. |  |  |  |  |  |  |
|  |  | (iv) Level D currency (A) Level D currency is related to designated manoeuvres and addresses knowledge and skills required for performing aircraft control tasks in real time with integrated use of associated systems and procedures. Level D currency may also address certain differences in flight characteristics including performance of any required manoeuvres and related normal, non-normal and emergency procedures. When level D is necessary, any pertinent lower-level currency is also to be addressed. (B) Re-establishing level D currency When currency is lost, currency may be re-established by completing pertinent manoeuvres using a device equal to or higher than that specified for level D differences training and checking. |  |  |  |  |  |  |
|  |  | (v) Level E currency (A) Level E currency requires that recent experience requirements of Part-FCL and operational requirements be complied with in each aircraft separately. Level E currency may also specify other system, procedure, or manoeuvre currency item(s) necessary for safe operations and may require procedures or manoeuvres to be accomplished in FSTDs or in an aircraft as mentioned in CS FCD.415(a). Provisions are applied in a way which addresses the required system or manoeuvre experience. When level E is assigned between aircraft of common characteristics, credit may be permitted. Assignment of level E currency requirements does not automatically lead to a determination on same or separate type rating. Level E currency is tracked by a means that is acceptable to the competent authority. When common take-off and landing credit (CTLC) is permitted, any credit or constraints applicable to using FSTDs, as mentioned in CS FCD.415(a), are also to be determined. (B) Re-establishing level E currency When currency is lost, currency may be re-established by completing pertinent manoeuvres using a device specified for level E differences training and checking. |  |  |  |  |  |  |
|  |  | (6) Competency regarding non-normal and emergency procedures — currency Competency for non-normal and emergency manoeuvres or procedures is generally addressed by checking requirements. Particular non-normal and emergency manoeuvres or procedures may not be considered mandatory for checking or training. In this situation, it may be necessary to periodically practise or demonstrate those manoeuvres or procedures specifying currency requirements for those manoeuvres or procedures. |  |  |  |  |  |  |
| 48. | GM1 ORO.FC.140(a) | Operator difference requirements (ODRS) The ODRs tables may result in different training programmes, depending on the training needs, regardless of the ‘base aircraft’ used to establish the table (e.g. the trainee may know the ‘other aircraft’ and be trained towards the ‘base aircraft’ ). |  |  |  |  |  |  |
| 49. | AMC1 ORO.FC.140(b) | Groups of single-engined piston helicopter types for the revalidation of the OPC When establishing groups of single-engined helicopter types for the purpose of crediting of proficiency checks, the operator should only take into account the helicopter types considered for crediting in AMC1 FCL.740.H(a)(3). |  |  |  |  |  |  |
| 50. | AMC1 ORO.FC.140(d) | Line checks — helicopters |  |  |  |  |  |  |
|  |  | (a) Prior to using a line check on one helicopter type or variant to revalidate the line check on other helicopter types or variants, the operator should consider whether the type of operations are sufficiently similar in terms of: (1) use of aerodromes or operating sites; (2) day VFR or night VFR; (3) use of operational approvals and specific approvals; (4) normal procedures, including flight preparation, take-off and landing procedures; and (5) use of automation. |  |  |  |  |  |  |
|  |  | (b) For IFR operations of helicopters, an operation should only be considered sufficiently similar to allow a line check on one type or variant to revalidate the line check for the other type or variant if such credits are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/20121, as determined in point (a) of ORO.FC.140. |  |  |  |  |  |  |
|  |  | (c) Line check cross-crediting should be defined in the operations manual. |  |  |  |  |  |  |
| 51. | ORO.FC.145 | Regulation (EU) 2021/2237 Provision of training, checking and assessment |  |  |  |  |  |  |
|  |  | (a)All training, checking and assessment required in this Subpart shall be conducted in accordance with the training programmes and syllabi established by the operator in the operations manual; |  |  |  |  |  |  |
|  |  | (b)When establishing the training programmes and syllabi, the operator shall include the relevant elements defined in the mandatory part of the operational suitability data established in accordance with Regulation (EU) No 748/2012. |  |  |  |  |  |  |
|  |  | (c) In the case of CAT operations, training and checking programmes, including syllabi and the use of the means to deliver the programme such as individual flight simulation training devices (FSTDs) and other training solutions, shall be approved by the competent authority. |  |  |  |  |  |  |
|  |  | (d) The FSTD used to meet the requirements of this Subpart shall be qualified in accordance with Regulation (EU) No 1178/2011 and it shall replicate the aircraft used by the operator, as far as practicable. Differences between the FSTD and the aircraft shall be described and addressed through a briefing or training, as appropriate. |  |  |  |  |  |  |
|  |  | (e) The operator shall establish a system to adequately monitor changes to the FSTD and to ensure that those changes do not affect the adequacy of the training programmes.”; |  |  |  |  |  |  |
|  |  | (f) The operator shall monitor the validity of each recurrent training and checking. |  |  |  |  |  |  |
|  |  | (g) The validity periods required in this Subpart shall be counted from the end of the month in which the recency, training or check was completed. |  |  |  |  |  |  |
| 52. | AMC1 ORO.FC.145 | Acceptance of previous training for non-commercial operations with complex motor-powered aircraft, including non-commercial specialised operations |  |  |  |  |  |  |
|  |  | (a) If the operator chooses to make use of previous training received by the pilot, the operator should develop a policy for the crediting of such training. Details of such policy should be included in the operations manual. |  |  |  |  |  |  |
|  |  | (b) The policy should as a minimum include measures to assess: (1) the content of the previous training; (2) whether the previous training was delivered by suitably qualified personnel or organisations; (3) whether the aircraft, FSTD or other equipment used for the previous training was sufficiently similar to the aircraft and equipment the crew member will operate; and (4) whether the operating procedures used during such previous training were sufficiently representative of the procedures used by the new operator. |  |  |  |  |  |  |
|  |  | (c) Where previous training delivered by other suitably qualified personnel or organisations is found to satisfy all or some of the requirements in ORO.FC.120, the training may be credited and an abbreviated conversion course may be used. Such an abbreviated course should cover all items not credited from previous training. |  |  |  |  |  |  |
|  |  | (d) Where a pilot flies for more than one operator and the training delivered by that other operator is found to satisfy some of the requirements of ORO.FC.130, then such training may be credited and an abbreviated recurrent training programme may be used. Such an abbreviated recurrent training programme should cover all items not credited from the training delivered by the other operator. |  |  |  |  |  |  |
|  |  | (e) An aircraft operator remains responsible for all training required by this Part regardless of whether the training is conducted by the operator, another operator, a certified organisation or another subcontractor, as defined in ORO.GEN.205. |  |  |  |  |  |  |
|  |  | (f) An operator accepting any previous training should be satisfied that the flight crew member is competent to operate in accordance with that operator’s procedures and to use the specific equipment installed on the aircraft to be operated. |  |  |  |  |  |  |
|  |  | (g) Previous training needs to be formally documented. |  |  |  |  |  |  |
|  |  | (h) The assessment under (b) and the documents referred to under (g) should be stored as part of the crew member training, checking and qualifications records. |  |  |  |  |  |  |
| 53. | GM1 ORO.FC.145 | Policy for acceptance of previous training and checking for other than commercial air transport operations (NCC) If the operator chooses to make use of previous training received by the pilot, in accordance with AMC1 ORO.FC.145, the operator may wish to enter into arrangements with other operators in order to satisfy the requirements of ORO.GEN.205 in relation to contracted training providers or other aircraft operators. |  |  |  |  |  |  |
| 54. | AMC1 ORO.FC.145(a) | Training and checking programmes and syllabi |  |  |  |  |  |  |
|  |  | (a) Training and checking programmes and syllabi should include as a minimum: (1) when training and checking take place during the same session, the distinction between the two; (2) a list of the items covered; (3) the minimum time allocation (duration); (4) the means of delivery (e.g. FSTD, OTD, computer-based, VR, etc.); (5) the personnel providing the training and conducting the checks. |  |  |  |  |  |  |
|  |  | (b) Further details on the training and checking programmes and syllabi should be included in the operations manual depending on the complexity of the operations (e.g. further contextualisation of the training programme, details of the airport in which some items will be covered, time allocation to brief and debrief, whether the item to be trained is a legal requirement or an SMS item, etc.). |  |  |  |  |  |  |
| 55. | GM1 ORO.FC.145(a) | Training and checking programmes and syllabi |  |  |  |  |  |  |
|  |  | The syllabus lists the topics to be covered in a training and checking programme. A syllabus may include: — the personnel providing the training and conducting the checks; — a description of the content; — the means of delivery (e.g. FSTD, aircraft, OTD, (virtual) classroom, computer-based training, VR, etc.); — the minimum time allocation (duration); — the prerequisites to be fulfilled before starting the training or checking; — the standard of performance; — the training objectives; — a reference to training/checking material; — the checking requirements, if any; — when training and checking is combined, the distinction between trained and checked items. |  |  |  |  |  |  |
| 56. | AMC1 ORO.FC.145(b) | ED Decision 2014/017/R Non-mandatory(recommendation)elements When developing the training programmes and syllabi, the operator should include the non-mandatory (recommendation) elements for the relevant type that are provided in the data established in accordance with Regulation (EC) No 748/2012. |  |  |  |  |  |  |
| 57. | AMC1 ORO.FC.145(d) | ED Decision 2014/017/R Full flight simulators (FFS) The operator should classify any differences between the aircraft and FFS in accordance with the Air Transport Association (ATA) chapters as follows: Compliance Levels |  |  |  |  |  |  |
|  |  | (a) Level A differences: (1) no influence on flight characteristics; (2) no influence on procedures (normal and/or abnormal); (3) differences in presentation; and (4) differences in operation. Method: self-instruction via the operations manual or flight crew information. |  |  |  |  |  |  |
|  |  | (b) Level B differences: (1) no influence on flight characteristics; (2) influence on procedures (normal and/or abnormal); and (3) possible differences in presentation and operation. Method: flight crew information, computer-based training, system device training or special instruction by instructor. |  |  |  |  |  |  |
|  |  | (c) Level C differences: (1) influence on flight characteristics; (2) influence on procedures (normal and/or abnormal); and Easy (3) eventually differences in presentation and operation. Method: special instruction by instructor, a selected partial training on another FSTD or aircraft or a waiver because of previous experience, special instruction or training programme. |  |  |  |  |  |  |
|  |  | (d) Level D differences: (1) influence on flight characteristics; and/or (2) influence on procedures (normal and/or abnormal); and/or (3) differences in presentation and/or operation; and (4) FSTD is level D qualified and is used for zero flight-time training (ZFTT). Method: a specified partial training on another FSTD or aircraft or a waiver because of previous experience, special instruction or training programme. |  |  |  |  |  |  |
| 58. | AMC2 ORO.FC.145(d) | FSTDs |  |  |  |  |  |  |
|  |  | (a) Before the operator extracts the data from an FSTD that can be related to a pilot, it should develop a data access and security policy. |  |  |  |  |  |  |
|  |  | (b) ‘Availability’ and ‘accessibility’ of FSTD used in this Subpart. (1) ‘Available FSTD’ refers to any flight simulation training device (FSTD) that is vacant for use by the FSTD operator or by the customers irrespective of any time consideration. (2) ‘Accessible’ refers to a device that can be used by the operator to conduct training or checking pertaining to this Subpart, and by the nominated person conducting the training or checking. More information on these definitions can be found in Part-FCL of Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
| 59. | GM1 ORO.FC.145(d) | Confidentiality and protection of training data in commercial air transport |  |  |  |  |  |  |
|  |  | (a) Without prejudice to applicable national legislation on the protection of individuals with regard to the processing of personal data, for the training conducted in accordance with ORO.FC.145 the operator may have a training data access and security policy (including the procedure to prevent disclosure of crew identity). |  |  |  |  |  |  |
|  |  | (b) If the operator decides to have such a policy, it should: (1) be agreed by all parties involved (airline management and flight crew member representatives nominated either by the union or the flight crew themselves); (2) be in line with the organisation’s safety policy in order to not make available or to not make use of the training data to attribute blame or liability. |  |  |  |  |  |  |
|  |  | (c) The training data access and security policy may include a policy for access to information only to specifically authorised persons identified by their position in order to perform their duties. |  |  |  |  |  |  |
| 60. | AMC1 ORO.FC.145(g) | Validity period of recurrent assessment, training and checking |  |  |  |  |  |  |
|  |  | (a) When the recency, training or check is completed within the last 3 months of the validity period, the new validity period should be counted from the original expiry date. |  |  |  |  |  |  |
|  |  | (b) When the recency, training or check is completed before the last 3 months of the validity period, the new validity period should be counted from the end of the month when the recency, training or check was completed and not from the original expiry date. |  |  |  |  |  |  |
|  |  | (c) Notwithstanding (a), the revalidation of CRM instructor and EBT instructor qualifications should follow AMC2 ORO.FC.146 and AMC2 ORO.FC.146(c). |  |  |  |  |  |  |
|  |  | Guidelines for inspectors |  |  |  |  |  |  |
|  |  | - Check that the operator has adequately defined the criteria for the accessibility and availability of an FSTD. |  |  |  |  |  |  |
|  |  | - Check that training and checking (except line checks) is carried out in an FSTD when available and accessible. |  |  |  |  |  |  |
|  |  | - Check for each FSTD used, the means used by the operator to identify all the differences between the FSTD and the real aircraft. |  |  |  |  |  |  |
|  |  | - Check that the capabilities of the FSTD listed allow the planned training to be conducted. |  |  |  |  |  |  |
|  |  | - Check that if the operator extracts data from an FSTD that can be related to a pilot, it has established a data access and security policy. |  |  |  |  |  |  |
|  |  | - Check that the syllabi and the related forms to be used for all training and checking activities, are provided within the FCTC programme submitted. |  |  |  |  |  |  |
|  |  | - Check that the syllabi include the following: - when training and checking take place during the same session, the distinction between the two; - a list of the items covered; - the minimum time allocation (duration); - the means of delivery (e.g. FSTD, OTD, computer-based, VR, etc.); - the personnel providing the training and conducting the checks; any additional details depending on the complexity of the operations (.e.g. further contextualisation of the training programme, details of the airport in which some items will be covered, time allocation to brief and debrief, whether the item to be trained is a legal requirement or an SMS item, etc.) |  |  |  |  |  |  |
|  |  | - Check that the prerequisites include at least the following: o adequate knowledge of human performance and limitations (HPL), whilst: - having obtained a CPL; or - having followed a theoretical HPL course covering the whole syllabus of the HPL examination; o completion of FC initial operator’s CRM training; o training in group facilitation skills;, except for instructors holding a certificate in accordance with Commission Regulation (EU) No 1178/2011. - Check that the following additional criteria are included: - adequate knowledge of the relevant flight operations at one operator; - Knowledge to be assessed by the operator in accordance with a method described in the OM; - Training to be provided by the operator if the trainer has not the relevant knowledge. - Initial CRM training of the operator; - Assessed by that operator (see criteria later) and - If CRM trainer at an operator, adequate knowledge of the relevant flight operations at that operator (refer to evaluation criteria above); or - Part of a team of trainers (i.e. supported by a training assistant, under the following conditions: - Training assistant received initial/refresher trainings - Training assistant is an instructor or has experience in ground training; - Training sessions to be prepared/provided jointly, but the responsibility remains with the trainer. |  |  |  |  |  |  |
| 61. | ORO.FC.146 | Regulation (EU) № 2020/2036 Personnel providing training, checking and assessment |  |  |  |  |  |  |
|  |  | (a)All training, checking and assessment required in this Subpart shall be conducted by appropriately qualified personnel. |  |  |  |  |  |  |
|  |  | (b) In the case of flight and flight simulation training, checking and assessment, the personnel that provide the training and conduct the checking or assessment shall be qualified in accordance with Annex I (Part- FCL) to Regulation (EU) No 1178/2011. Additionally, the personnel providing training and conducting checking towards specialised operations shall be suitably qualified for the relevant operation. |  |  |  |  |  |  |
|  |  | (c) For an EBT programme, the personnel that performs assessment and provides training shall: (1) hold an Annex I (Part-FCL) instructor or examiner certificate; (2) complete the operator’s EBT instructor standardisation programme. This shall include an initial standardisation programme and a recurrent standardisation programme. Completion of the operator’s EBT initial standardisation will qualify the instructor to perform EBT practical assessment. |  |  |  |  |  |  |
|  |  | (d) Notwithstanding point (b), the line evaluation of competence may be conducted by a suitably qualified commander nominated by the operator that is standardised in EBT concepts and the assessment of competencies (line evaluator). |  |  |  |  |  |  |
|  |  | (e) Notwithstanding point (b), the aircraft/FSTD training and the operator proficiency check may be conducted by a suitably qualified commander holding a FI/TRI/SFI certificate and nominated by the operator for any of the following operations: (1) CAT operations of helicopters meeting the criteria defined in point ORO.FC.005(b)(2); (2) CAT operations of other than complex motor-powered helicopters by day and over routes navigated by reference to visual landmarks; (3) CAT operations of performance class B aeroplanes that do not meet the criteria defined in point ORO.FC.005(b)(1). |  |  |  |  |  |  |
|  |  | (f) Notwithstanding point (b), the aircraft/FSTD training and the demonstration of competence/operator proficiency check may be conducted by a suitably qualified pilot-in-command/commander nominated by the operator for any of the following operations: (1) specialised operations; ‘(2) CAT operations of aeroplanes meeting the criteria defined in point ORO.FC.005(b)(1).’; |  |  |  |  |  |  |
|  |  | (g) Notwithstanding point (b), the line check may be conducted by a suitably qualified commander nominated by the operator. |  |  |  |  |  |  |
|  |  | (h) The operator shall inform the competent authority about the persons nominated under points (e) to (g). |  |  |  |  |  |  |
|  |  | - Check the described qualification/training requirements for personnel providing training and/or conducting the checks: - CRM training - UPRT training - Line check (list of nominated commanders to be maintained (if applicable)) - Aircraft/FSTD training - OPC - Ground training - Emergency and safety equipment training - LIFUS |  |  |  |  |  |  |
| 62. | AMC1 ORO.FC.146 | Personnel conducting training and checking — general |  |  |  |  |  |  |
|  |  | Training and checking should be provided by the following personnel: (a) Ground and refresher training by suitably qualified personnel; |  |  |  |  |  |  |
|  |  | (b) Emergency and safety equipment training and checking by suitably qualified personnel as specified in the operator’s manual; |  |  |  |  |  |  |
|  |  | (c) CRM (1) Integration of CRM elements into the different phases of training by all the personnel conducting the training, as per AMC1 and AMC2 ORO.FC.115. (2) The operator should ensure that all personnel conducting such training are suitably qualified to integrate elements of CRM into this training. (3) Classroom CRM training by at least one CRM trainer, qualified as specified in AMC2 ORO.FC.146 who may be assisted by experts in order to address specific areas. |  |  |  |  |  |  |
| 63. | AMC2 ORO.FC. 146 | Flight crew CRM trainer |  |  |  |  |  |  |
|  |  | (a) Applicability The provisions described herein: (1) should be fulfilled by flight crew CRM trainers responsible for classroom CRM training; and (2) are not applicable to: (i) instructors, holding a certificate in accordance with Commission Regulation (EU) No 1178/2011, when conducting CRM training in the operational environment; and (ii) trainers or instructors when conducting training other than CRM training, but integrating CRM elements into this training. |  |  |  |  |  |  |
|  |  | (b) Qualification of a flight crew CRM trainer (1) Prerequisites. A flight crew CRM trainer should: (i) have adequate knowledge of human performance and limitations (HPL), whilst: (A) having obtained a commercial pilot licence in accordance with Commission Regulation (EU) No 1178/2011; or (B) having followed a theoretical HPL course covering the whole syllabus of the HPL examination; (ii) have completed flight crew initial operator’s CRM training; (iii) have received training in group facilitation skills;, except for instructors holding a certificate in accordance with Commission Regulation (EU) No 1178/2011. (2) In order to qualify as flight crew CRM trainer, a person meeting the prerequisites should: (i) have adequate knowledge of the relevant flight operations at one operator, in accordance with (d); (ii) receive the initial training in accordance with (c)(3); and (iii) be assessed by that operator in accordance with (f). (3) In order to act as flight crew CRM trainer at an operator, a qualified and current flight crew CRM trainer should meet one of the following conditions: (i) have adequate knowledge of the relevant flight operations at that operator, in accordance with (d); or (ii) be part of a team of trainers in accordance with (e). (4) The period of validity of the flight crew CRM trainer qualification should be 3 years. (5) Recency and renewal of the flight crew CRM trainer qualification (i) The flight crew CRM trainer should complete CRM trainer refresher training within the last 12 months of the 3-year validity period; and (ii) The flight crew CRM trainer should meet one or both of the following conditions: (A) conduct at least 3 CRM training events within the 3-year validity period; (B) be assessed within the last 12 months of the 3-year validity period in accordance with (f); and (iii) If the flight crew CRM trainer qualification has expired, it can be renewed if all of the conditions below are met. The validity should be 3 years after completion of (A) and (C) below, whichever comes first: - complete CRM trainer refresher training; (B) receive refresher training on knowledge of the relevant flight operations, as necessary; (C) be assessed in accordance with (f). |  |  |  |  |  |  |
|  |  | (c) Training of flight crew CRM trainer (1) If the operator trains flight crew CRM trainers, the training syllabi should be described in the operations manual. The operator should ensure that the initial and refresher training of the flight crew CRM trainers are conducted by flight crew CRM trainers with a minimum of 3 years of experience. (2) Training of flight crew CRM trainers should be both theoretical and practical. Practical elements should include the development of specific trainer skills, particularly the integration of CRM into line operations. (3) The initial training of flight crew CRM trainers should include the the following: (i) introduction to CRM training and competencies for CRM trainers:; (A) ability to interact with and manage a group; (B) ability to pre-plan an objective and timely training session; (C) ability to deliver a good balance of ‘telling’, ‘selling’ and ‘facilitating’; (D) ability to connect realistically poor and good CRM to the operations; (E) ability to assess the performance, the progress and needs of trainees in a meaningfully way; (ii) operator’s management system as defined in point (a)(7) of AMC1 ORO.FC.115; and (iii) characteristics of the flight crew CRM training as defined in Table 1 of AMC1 ORO.FC.115 and its integration into line operations, as applicable: (A) the different types of CRM trainings (initial, recurrent, etc.); (B) combined training; and (C) training related to the type of aircraft or operation. Instructors holding a certificate in accordance with Commission Regulation (EU) No 1178/2011 may be credited towards (i) and (ii) if they have completed the refresher training defined in (4). (4) The refresher training of flight crew CRM trainers should include new methodologies, procedures and lessons learned, as well as additional topics such as the following: (i) Group facilitation skills including team dynamics, moderation skills and use of questions (ii) Course preparation, defining objectives and selecting methods to best convey knowledge (e.g. lecture, group work, case analysis, gamification, scenario-based training, individual research) (iii) Safety culture and management systems (iv) An example of an analysis of CRM factors in an accident or serious incident. (v) New developments or research in human factors and CRM (vi) TEM principles and their practical implementation in normal operations (5) Instructors, holding a certificate in accordance with Commission Regulation (EU) No 1178/2011, who are also CRM trainers, may combine the CRM trainer refresher training with instructor refresher training if the instructor refresher training meets all of the conditions defined in (4). (6) Instructors for other-than complex motor-powered aeroplanes should be qualified as flight crew CRM trainers for this aircraft category with no additional training, as specified in (3) and (4) when: (i) holding a certificate in accordance with Commission Regulation (EU) No 1178/2011; and (ii) fulfilling the provisions of (b)(2) or (b)(5). |  |  |  |  |  |  |
|  |  | (d) Knowledge of the relevant flight operations (1) The operator should evaluate the experience and knowledge of the flight crew CRM trainer. The evaluation of the operator should include at least: (i) the operational experience of the flight crew CRM trainer as a flight crew member; (ii) whether this experience as a flight crew member or a former flight crew member covers the aircraft category, the aircraft generation and the form of operations, as relevant to the operator. (2) If the flight crew CRM trainer does not have the relevant knowledge of the relevant flight operation based on the evaluation in (1), the operator should provide training to the flight crew CRM trainer to provide the adequate knowledge. (3) The operator should describe the assessment and training in the operations manual. |  |  |  |  |  |  |
|  |  | (e) Team of CRM trainers If the flight crew CRM trainer is qualified in accordance with (b) but does not meet the conditions defined in (d), he or she may be assisted by a training assistant that has the knowledge of the relevant flight operations. The operator should ensure that all the following conditions are met: (1) The training assistant should meet the condition defined in (c) but needs not meet the conditions defined in (b). The training assistant should be an instructor or have experience in ground training. (2) The flight crew CRM trainer and the training assistant should prepare the training session together and adapt it to the operational needs of the operator. (3) If the flight crew CRM trainer and the training assistant have already provided training for the operator or for a similar operator, the operator may determine that condition (2) is met. (4) The flight crew CRM trainer and the training assistant should provide the training together. (5) The flight crew CRM trainer remains responsible for the training. |  |  |  |  |  |  |
|  |  | (f) Assessment of a flight crew CRM trainer (1) The operator should ensure that the process for the assessment is included in the operations manual describing methods for observing, recording, interpreting and debriefing the flight crew CRM trainer. All personnel involved in the assessment must be credible and competent in their role. (2) The assessment should enable the flight crew CRM trainer to demonstrate the knowledge and ability to train the CRM training elements in the non-operational environment. Special attention should be given to fields such as group management, group dynamics and personal awareness. (3) The initial assessment of a flight crew CRM trainer by the operator may take place when conducting their first CRM training course. (4) The assessment of flight crew CRM trainers should be conducted by flight crew CRM trainers with a minimum of 3 years of experience. |  |  |  |  |  |  |
|  |  | (g) The operator should only select a qualified and current flight crew CRM trainer meeting the conditions defined in (d) or (e). |  |  |  |  |  |  |
| 64. | AMC1 ORO.FC.146(b) | Personnel providing aircraft/fstd training and conducting operator proficiency checking and qualified under annex i (part-FCL) to regulation (eu) no 1178/2011 |  |  |  |  |  |  |
|  |  | Training and checking should be provided by the following personnel: (a) Flight training by a type rating instructor (TRI) or class rating instructor (CRI), flight instructor (FI) or, in the case of the FSTD content, a synthetic flight instructor (SFI). For commercial air transport, the FI, TRI, CRI or SFI should satisfy the operator’s experience and knowledge requirements sufficiently to instruct on aircraft systems and operational procedures and requirements. |  |  |  |  |  |  |
|  |  | (b) Operator proficiency check by a type rating examiner (TRE), class rating examiner (CRE) or, if the check is conducted in an FSTD, a synthetic flight examiner (SFE). The TRE, CRE or SFE should be trained in CRM concepts and the assessment of CRM skills. |  |  |  |  |  |  |
|  |  | (c) For aircraft/FSTD training, line flying under supervision, operator proficiency checks and line checks, if the training or checking includes multi-pilot operations in helicopters, in addition to (a) and (b) the personnel conducting training or checking should have 350 hours flying experience in multi-pilot operations. |  |  |  |  |  |  |
|  |  | (d) In the case of CAT operations in helicopters, the 350 hours flying experience in multi-pilot operations defined in (c) may be reduced on an individual basis, as part of the approval of the training and checking programmes. The operator may apply for such a reduced flying experience based on the unavailability of experienced pilots in both multi-pilot operations and in their types of operations. A FI/TRI/SFI rating and MCC training in helicopters should be a prerequisite for any reduced flying experience in multi-pilot operations. In addition, the operator should define mitigation measures after having performed a risk assessment. The following should be taken into account: (1) flying experience criteria in single-pilot operations in the types of operations; (2) any other training, checking, recency and experience criteria; (3) robustness and maturity of multi-pilot SOPs. |  |  |  |  |  |  |
|  |  | (e) In the case of training and checking towards the relevant aspects associated with a specialised operation, points (j)(2) to (j)(4) of AMC1 ORO.FC.146(e);(f)&(g) should apply. |  |  |  |  |  |  |
| 65. | AMC1 ORO.FC.146(e);(f)&(g) | Suitably qualified PIC or commander nominated by the operator — general |  |  |  |  |  |  |
|  |  | (a) The nominated PIC/commander conducting training should either be qualified as an instructor under Regulation (EU) No 1178/2011 or receive training which should cover at least: (1) techniques of briefing and debriefing; (2) CRM concepts and CRM assessment; (3) for SPO, which manoeuvres the nominated PIC/commander should not train or check unless qualified as an instructor. |  |  |  |  |  |  |
|  |  | (b) In addition, the nominated PIC/commander conducting operator proficiency checks or line checks should either be qualified as an examiner under Regulation (EU) No 1178/2011 or receive additional training which should cover at least: (1) how to perform a check; (2) flight techniques applicable to checks performed in flight; (3) the assessment of CRM skills. |  |  |  |  |  |  |
|  |  | (c) The nominated PIC/commander conducting aircraft/FSTD training, line flying under supervision, operator proficiency checks or line checks taking place under multi-pilot operations in helicopters should have 350 hours flying experience in multi-pilot operations. |  |  |  |  |  |  |
|  |  | (d) The nominated PICs/commanders, or the criteria for nominating PICs/commanders, should be included in the operations manual. |  |  |  |  |  |  |
|  |  | (e) The nominated PIC/commander should be type rated or class rated in the type or class where he or she provides the training, checking or assessment. |  |  |  |  |  |  |
|  |  | CAT — suitably qualified commander or instructor nominated by the operator (f) For CAT operations under VFR by day, the minimum experience of the nominated commander should be more than 750 hours total flight time with at least 50 hours on the type, class or the aircraft variant. |  |  |  |  |  |  |
|  |  | (g) For CAT operations in performance class B aeroplanes under night VFR or under IFR, the minimum experience of the nominated commander should be more than 1 000 hours total flight time with at least 100 hours on the type, class or the aircraft variant. |  |  |  |  |  |  |
|  |  | (h) In the case of CAT operations in helicopters, the 350 hours flying experience in multi-pilot operations defined in (c) may be reduced on an individual basis, as part of the approval of the training and checking programmes. The operator may apply for such a reduced flying experience based on the unavailability of experienced pilots in both multi-pilot operations and in their types of operations. An FI/TRI/SFI rating and MCC training in helicopters should be a prerequisite for any reduced flying experience in multi-pilot operations. In addition, the operator should define mitigation measures after having performed a risk assessment. The following should be taken into account: (1) flying experience criteria in single-pilot operations in the types of operations; (2) any other training, checking, recency and experience criteria; and (3) robustness and maturity of multi-pilot SOPs. |  |  |  |  |  |  |
|  |  | (i) ORO.FC.220 (f) allows the operator to develop a specific conversion course to address an operational circumstance, when the operator intends to have pilots temporally joining the operator to conduct line checks. The content of the specific operator’s conversion course is included in AMC1 ORO.FC.220(f). |  |  |  |  |  |  |
|  |  | SPO — suitably qualified pic or instructor nominated by the operator (j) For SPO, the person conducting the aircraft/FSTD training and the operator proficiency check should meet the following criteria: (1) Training and checking covering normal, abnormal and emergency procedures relevant to the type or variant should be conducted in accordance with AMC1 ORO.FC.146(b). (2) Training and checking covering the relevant aspects associated with HEC and HESLO should be conducted by a HEC or HESLO instructor as defined in AMC1 SPO.SPEC.HEC.100 and AMC1 SPO.SPEC.HESLO.100. (3) Training and checking covering the relevant aspects associated with a specialised operation other than HEC and HESLO should be conducted by a nominated PIC with the following flight experience: (i) at least 750 hours total flight time with at least 50 hours on the type, class or aircraft variant; (ii) for specialised operations other than HEC and HESLO, either: (A) at least 350 hours in the applicable specialised operation; or (B) 800 hours in specialised operations and the number of hours in the applicable specialised operation as defined by the operator, based on a risk assessment, taking into account the complexity of the relevant aspects associated with the applicable specialised operation. Flight experience in HHO, firefighting flight experience and flight experience in the search component of search and rescue flights may be credited towards the 800 hours in specialised operations. In addition, up to 200 hours of experience in CAT operations (other than HHO) may be credited towards the 800 hours in specialised operations. (4) In addition to (2) and (3) above, flight training and checking of sensitive type-related manoeuvres in combination with the training and checking of the relevant aspects associated with a specialised task, should be conducted by a qualified instructor. |  |  |  |  |  |  |
|  |  | (k) In addition to (j) above, if the SPO operator combines the operator proficiency check with a licence proficiency check, the person conducting the check should meet the requirements for licence proficiency checks. |  |  |  |  |  |  |
|  |  | - Check the described qualification/training requirements for personnel providing training and/or conducting the checks: - CRM training - UPRT training - Line check (list of nominated commanders to be maintained (if applicable)) - Aircraft/FSTD training - OPC - Ground training - Emergency and safety equipment training - LIFUS |  |  |  |  |  |  |
|  |  | - Check that the prerequisites include at least the following: - adequate knowledge of human performance and limitations (HPL), whilst: - having obtained a CPL; or - having followed a theoretical HPL course covering the whole syllabus of the HPL examination; - completion of FC initial operator’s CRM training; - training in group facilitation skills;, except for instructors holding a certificate in accordance with Commission Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
|  |  | - Check that the following additional criteria are included: - adequate knowledge of the relevant flight operations at one operator; - Knowledge to be assessed by the operator in accordance with a method described in the OM; - Training to be provided by the operator if the trainer has not the relevant knowledge. - Initial CRM training of the operator; - Assessed by that operator (see criteria later) and - If CRM trainer at an operator, adequate knowledge of the relevant flight operations at that operator (refer to evaluation criteria above); or - Part of a team of trainers (i.e. supported by a training assistant, under the following conditions: - Training assistant received initial/refresher trainings - Training assistant is an instructor or has experience in ground training; - Training sessions to be prepared/provided jointly, but the responsibility remains with the trainer. |  |  |  |  |  |  |
|  |  | - Check that the FC CRM trainer training programme is described in the OM and includes theoretical and practical parts: - Initial training: - introduction to CRM training and competencies for CRM trainers - hazards and risks identified by the operator’s management system - characteristics of the FC CRM training and its integration into line operations - Refresher training: - Group facilitation skills including team dynamics, moderation skills and use of questions - Course preparation, defining objectives and selecting methods to best convey knowledge (e.g. lecture, group work, case analysis, gamification, scenario-based training, individual research) - (Safety culture and management systems - An example of an analysis of CRM factors in an accident or serious incident. - New developments or research in human factors and CRM - TEM principles and their practical implementation in normal operations. |  |  |  |  |  |  |
|  |  | - Check that the methodology to conduct the assessment of FC CRM trainers is described in the OM and adequate. |  |  |  |  |  |  |
|  |  | - Check that assessments of FC CRM trainers are conducted by FC CRM trainers with a minimum of 3Y experience. |  |  |  |  |  |  |
|  |  | - Check that the qualification has a validity of 3Y |  |  |  |  |  |  |
|  |  | - Check that the renewal of the qualification is based on the following criteria: - 2 CRM training events in any 12-month period - Assessed within the last 12 M of the 3-Y validity period by the operator - CRM refresher training received within the 3 Y validity period. |  |  |  |  |  |  |
| 66. | ORO.FC.200 | Regulation (EU) No 965/2012 Composition of flight crew |  |  |  |  |  |  |
|  |  | (a) There shall not be more than one inexperienced flight crew member in any flight crew. |  |  |  |  |  |  |
|  |  | (b) The commander may delegate the conduct of the flight to another pilot suitably qualified in accordance with Annex I (Part-FCL) to Regulation (EU) No 1178/2011 provided that the requirements of ORO.FC.105(b)(1), (b)(2) and (c) are complied with. |  |  |  |  |  |  |
|  |  | (c) Specific requirements for aeroplane operations under instrument flight rules (IFR) or at night. |  |  |  |  |  |  |
|  |  | (1) The minimum flight crew shall be two pilots for all turbo-propeller aeroplanes with a maximum operational passenger seating configuration (MOPSC) of more than nine and all turbojet aeroplanes. (2) Aeroplanes other than those covered by (c)(1) shall be operated with a minimum crew of two pilots, unless the requirements of ORO.FC.202 are complied with, in which case they may be operated by a single pilot. |  |  |  |  |  |  |
|  |  | (d) Specific requirements for helicopter operations. |  |  |  |  |  |  |
|  |  | For all operations of helicopters with an MOPSC of more than 19 and for operations under IFR of helicopters with an MOPSC of more than 9, the minimum flight crew shall be two pilots. |  |  |  |  |  |  |
| 67. | AMC1 ORO.FC.200(a) | ED Decision 2014/017/R Crewing of inexperienced flight crew members The operator should establish procedures in the operations manual taking into account the following elements: |  |  |  |  |  |  |
|  |  | Aeroplanes |  |  |  |  |  |  |
|  |  | (a) The operator should consider that a flight crew member is inexperienced, following completion of a type rating or command course, and the associated line flying under supervision, until he/she has achieved on the type either: |  |  |  |  |  |  |
|  |  | (1) 100 flight hours and flown 10 sectors within a consolidation period of 120 consecutive days; or |  |  |  |  |  |  |
|  |  | (2) 150 flight hours and flown 20 sectors (no time limit). |  |  |  |  |  |  |
|  |  | (b) A lesser number of flight hours or sectors, subject to any other conditions that the competent authority may impose, may be acceptable to the competent authority when one of the following applies: |  |  |  |  |  |  |
|  |  | (1) a new operator is commencing operations; |  |  |  |  |  |  |
|  |  | (2) an operator introduces a new aeroplane type; |  |  |  |  |  |  |
|  |  | (3) flight crew members have previously completed a type conversion course with the same operator; |  |  |  |  |  |  |
|  |  | (4) credits are defined in the data established in accordance with Regulation (EC) No 748/2012; or |  |  |  |  |  |  |
|  |  | (5) the aeroplane has a maximum take-off mass of less than 10 tonnes or a maximum operational passenger seating configuration (MOPSC) of less than 20. |  |  |  |  |  |  |
|  |  | Helicopters |  |  |  |  |  |  |
|  |  | (c) The operator should consider that, when two flight crew members are required, a flight crew member, following completion of a type rating or command course, and the associated line flying under supervision, is inexperienced until either: |  |  |  |  |  |  |
|  |  | (1) he/she has achieved 50 flight hours on the type and/or in the role within a period of 60 days; or |  |  |  |  |  |  |
|  |  | (2) he/she has achieved 100 flight hours on the type and/or in the role (no time limit). |  |  |  |  |  |  |
|  |  | (d) A lesser number of flight hours, on the type and/or in the role, and subject to any other conditions which the competent authority may impose, may be acceptable to the competent authority when one of the following applies: |  |  |  |  |  |  |
|  |  | (1) a new operator is commencing operations; |  |  |  |  |  |  |
|  |  | (2) an operator introduces a new helicopter type; |  |  |  |  |  |  |
|  |  | (3) flight crew members have previously completed a type conversion course with the same operator (reconversion); or |  |  |  |  |  |  |
|  |  | (4) credits are defined in the data established in accordance with Regulation (EC) No 748/2012. |  |  |  |  |  |  |
| 68. | ORO.FC.A.201 | Regulation (EU) No 965/2012 In-flight relief of flight crew members |  |  |  |  |  |  |
|  |  | (a) The commander may delegate the conduct of the flight to: (1) another qualified commander; or (2) for operations only above flight level (FL) 200, a pilot who complies with the following minimum qualifications: (i) ATPL; (ii) conversion training and checking, including type rating training, in accordance with ORO.FC.220; (iii) all recurrent training and checking in accordance with ORO.FC.230 and ORO.FC.240; (iv) route/area and aerodrome competence in accordance with ORO.FC.105. |  |  |  |  |  |  |
|  |  | (b) The co-pilot may be relieved by: (1) another suitably qualified pilot; (2) for operations only above FL 200, a cruise relief co-pilot that complies with the following minimum qualifications: (i) valid commercial pilot licence (CPL) with an instrument rating; (ii) conversion training and checking, including type rating training, in accordance with ORO.FC.220 except the requirement for take-off and landing training; (iii) recurrent training and checking in accordance with ORO.FC.230 except the requirement for take-off and landing training. |  |  |  |  |  |  |
|  |  | (c) A flight engineer may be relieved in flight by a crew member suitably qualified in accordance with applicable national rules. |  |  |  |  |  |  |
| 69. | ORO.FC.202 | Regulation (EU) No 965/2012 Single-pilot operations under IFR or at night In order to be able to fly under IFR or at night with a minimum flight crew of one pilot, the following shall be complied with: |  |  |  |  |  |  |
|  |  | (a) The operator shall include in the operations manual a pilot’s conversion and recurrent training programme that includes the additional requirements for a single-pilot operation. The pilot shall have undertaken training on the operator’s procedures, in particular regarding: (1) engine management and emergency handling; (2) use of normal, abnormal and emergency checklist; (3) air traffic control (ATC) communication; (4) departure and approach procedures; (5) autopilot management, if applicable; (6) use of simplified in-flight documentation; (7) single-pilot crew resource management. |  |  |  |  |  |  |
|  |  | (b) intentionally left blank |  |  |  |  |  |  |
|  |  | (c) For aeroplane operations under IFR the pilot shall have: (1) a minimum of 50 hours flight time under IFR on the relevant type or class of aeroplane, of which 10 hours are as commander; and (2) completed during the preceding 90 days on the relevant type or class of aeroplane: (i) five IFR flights, including three instrument approaches, in a single-pilot role; or (ii) an IFR instrument approach check. |  |  |  |  |  |  |
|  |  | (d) For aeroplane operations at night the pilot shall have: (1) a minimum of 15 hours flight time at night which may be included in the 50 hours flight time under IFR in (c)(1); and (2) completed during the preceding 90 days on the relevant type or class of aeroplane: (i) three take-offs and landings at night in the single pilot role; or (ii) a night take-off and landing check. |  |  |  |  |  |  |
|  |  | (e) For helicopter operations under IFR the pilot shall have: (1) 25 hours total IFR flight experience in the relevant operating environment; and (2) 25 hours flight experience as a single pilot on the specific type of helicopter, approved for single-pilot IFR, of which 10 hours may be flown under supervision, including five sectors of IFR line flying under supervision using the single-pilot procedures; (3) completed during the preceding 90 days: (i) five IFR flights as a single pilot, including three instrument approaches, carried out on a helicopter approved for this purpose; or (ii) an IFR instrument approach check as a single pilot on the relevant type of helicopter, flight training device (FTD) or full flight simulator (FFS). |  |  |  |  |  |  |
| 70. | ORO.FC.205 | Regulation (EU) No 965/2012 Command course |  |  |  |  |  |  |
|  |  | (a) For aeroplane and helicopter operations, the command course shall include at least the following elements: (1) training in an FSTD, which includes line oriented flight training (LOFT) and/or flight training; (2) the operator proficiency check, operating as commander; (3) command responsibilities training; (4) line training as commander under supervision, for a minimum of: (i) 10 flight sectors, in the case of aeroplanes; and (ii) 10 hours, including at least 10 flight sectors, in the case of helicopters; (5) completion of a line check as commander and demonstration of adequate knowledge of the route or area to be flown and of the aerodromes, including alternate aerodromes, facilities and procedures to be used; and (6) crew resource management training. |  |  |  |  |  |  |
| 71. | AMC1 ORO.FC.205 | ED Decision 2014/017/R Combined upgrading and conversion course – HELICOPTER If a pilot is converting from one helicopter type to another when upgrading to commander: (a) the command course should also include a conversion course in accordance with ORO.FC.220; and (b) additional flight sectors should be required for a pilot transitioning onto a new type of helicopter. |  |  |  |  |  |  |
| 72. | ORO.FC.215 | Regulation (EU) No 965/2012 Initial operator's crew resource management (CRM) training |  |  |  |  |  |  |
|  |  | (a) The flight crew member shall have completed an initial CRM training course before commencing unsupervised line flying. |  |  |  |  |  |  |
|  |  | (b) Initial CRM training shall be conducted by at least one suitably qualified CRM trainer who may be assisted by experts in order to address specific areas. |  |  |  |  |  |  |
|  |  | (c) If the flight crew member has not previously received theoretical training in human factors to the ATPL level, he/she shall complete, before or combined with the initial CRM training, a theoretical course provided by the operator and based on the human performance and limitations syllabus for the ATPL as established in Annex I (Part-FCL) to Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
| 73. | AMC1 ORO.FC.215 | ED Decision 2015/022/R Training elements and trainer qualification Initial operator’s CRM training should: (a) cover the applicable provisions of AMC1 ORO.FC.115, including the training elements as specified in Table 1 thereof; and (b) be conducted by a flight crew CRM trainer who is qualified as specified in AMC2 ORO.FC.146. |  |  |  |  |  |  |
| 74. | ORO.FC.220 | Regulation (EU) No 71/2014 Operator conversion training and checking |  |  |  |  |  |  |
|  |  | (a) CRM training shall be integrated into the operator conversion training course. |  |  |  |  |  |  |
|  |  | (b) Once an operator conversion course has been commenced, the flight crew member shall not be assigned to flying duties on another type or class of aircraft until the course is completed or terminated. Crew members operating only performance class B aeroplanes may be assigned to flights on other types of performance class B aeroplanes during conversion courses to the extent necessary to maintain the operation. Crew members may be assigned to flights on single-engined helicopters during an operator conversion course on a single-engined helicopter, provided that the training is unaffected |  |  |  |  |  |  |
|  |  | (c) The amount of training required by the flight crew member for the operator’s conversion course shall be determined in accordance with the standards of qualification and experience specified in the operations manual, taking into account his/her previous training and experience. |  |  |  |  |  |  |
|  |  | (d) The flight crew member shall complete: (1) the operator proficiency check and the emergency and safety equipment training and checking before commencing line flying under supervision (LIFUS); and (2) the line check upon completion of line flying under supervision. For performance class B aeroplanes, LIFUS may be performed on any aeroplane within the applicable class. |  |  |  |  |  |  |
|  |  | (e) In the case of aeroplanes, pilots that have been issued a type rating based on a zero flight-time training (ZFTT) course shall: (1) commence line flying under supervision not later than 21 days after the completion of the skill test or after appropriate training provided by the operator. The content of that training shall be described in the operations manual; (2) complete six take-offs and landings in a FSTD not later than 21 days after the completion of the skill test under the supervision of a type rating instructor for aeroplanes (TRI(A)) occupying the other pilot seat. The number of take-offs and landings may be reduced when credits are defined in the data established in accordance with Regulation (EU) No 748/2012. If these take-offs and landings have not been performed within 21 days, the operator shall provide refresher training. The content of such training shall be described in the operations manual; (3) conduct the first four take-offs and landings of the LIFUS in the aeroplane under the supervision of a TRI(A) occupying the other pilot seat. The number of take-offs and landings may be reduced when credits are defined in the data established in accordance with Regulation (EU) No 748/2012. |  |  |  |  |  |  |
|  |  | (f) If operational circumstances, such as applying for a new AOC or adding a new aircraft type or class to the fleet, do not allow the operator to comply with the requirements in (d), the operator may develop a specific conversion course, to be used temporarily for a limited number of pilots. |  |  |  |  |  |  |
|  |  | - For (f) Check the criteria defined by the operator for the use of such procedure (e.g. start of operations). - Check that the conversion training includes training on: o normal procedures, which include flight planning and ground-handling and flight operations, including performance, mass and balance, fuel schemes, selection of alternates, and ground de-icing/anti-icing; o abnormal and emergency procedures, which include pilot incapacitation as applicable. - Check the limitations defined by the operator for these line-checkers: o 6 M validity o No duties at the controls of the aircraft o Line checks of pilots with non-expired LC. |  |  |  |  |  |  |
|  |  | - Check the content of the CC training programme, iaw AMC1: o Ground training o Emergency and safety equipment training o CRM training o Security training (see also AMC1 ORO.GEN.110(a) o Flight training and checking o LIFUS and Line check o First-aid training if the FC has not previously completed an operator’s conversion course, and if applicable ditching procedures training o Training for operations where no CC is required (if applicable) o UPRT training o Training on flight path management during unreliable airspeed indication and other failures at high altitude (A/C with a cruising altitude above FL 300). - Check that it foresees: o the conduct of the OPC and emergency and safety equipment training before commencing the LIFUS; o The completion of a line check at the end of the LIFUS. - Check, if applicable, the operator’s policy regarding the crediting of trainings received in another operator (For example, CRM training and procedures’ training may not be credited). - Check the operator’s policy regarding the assignment of flight duties on another type or class of aircraft before the completion of the conversion course. - Check the operator’s specific process for conversion course for a new AOC or when adding a new aircraft type. - Check that the duration of the training is defined and adequate - Check the adequacy of the facilities and audio, mechanical and visual aids used and the related supervision (if applicable). - Check the ground training syllabus: - Aircraft systems (incl. ACAS) - Normal, abnormal and emergency procedures (incl. flight without ADF, if applicable) - Relevant samples of accidents/incidents and occurrences. - Check that formal tests are foreseen - Check that it includes de-identified feedback from the management system, including occurrence reporting and flight data monitoring programmes. - Check that line check form includes the assessment of CRM skills |  |  |  |  |  |  |
| 75. | AMC1 ORO.FC.220 | ED Decision 2015/022/R Operator conversion training syllabus (a) General |  |  |  |  |  |  |
|  |  | (1) The operator conversion training should include, in the following order: (i) ground training and checking, including all of the following; (A) aircraft systems;, and (B) normal procedures, which include flight planning and ground-handling and flight operations, including performance, mass and balance, fuel schemes, selection of alternates, and ground de-icing/anti-icing;, (C) abnormal and emergency procedures, which include pilot incapacitation as applicable; (D) a review of relevant samples of accident/incident and occurrences to increase awareness of the occurrences that may be relevant for the intended operation; (ii) emergency and safety equipment training and checking, (completed before any flight training in an aircraft commences); (iii) flight training and checking (aircraft and/or FSTD); and (iv) line flying under supervision and line check. |  |  |  |  |  |  |
|  |  | (2) When the flight crew member has not previously completed an operator’s conversion course, he/she should undergo general first-aid training and, if applicable, ditching procedures training using the equipment in water. |  |  |  |  |  |  |
|  |  | (3) Where the emergency drills require action by the non-handling pilot, the check should additionally cover knowledge of these drills. |  |  |  |  |  |  |
|  |  | (4) The operator’s conversion may be combined with a new type/class rating training, as required by Commission Regulation (EU) No 1178/2011. |  |  |  |  |  |  |
|  |  | (5) The operator should ensure that: (i) applicable elements of CRM training, as specified in Table 1 of AMC1 ORO.FC.115, are integrated into all appropriate phases of the conversion training; and (ii) the personnel integrating elements of CRM into conversion training are suitably qualified, as specified in AMC2 ORO.FC.146. |  |  |  |  |  |  |
|  |  | (b) Ground training |  |  |  |  |  |  |
|  |  | (1) Ground training should comprise a properly organised programme of ground instruction supervised by training staff with adequate facilities, including any necessary audio, mechanical and visual aids. Self-study using appropriate electronic learning aids, computer-based training (CBT), etc., may be used with adequate supervision of the standards achieved. However, if the aircraft concerned is relatively simple, unsupervised private study may be adequate if the operator provides suitable manuals and/or study notes. |  |  |  |  |  |  |
|  |  | (2) The course of ground instruction should incorporate formal tests. |  |  |  |  |  |  |
|  |  | (c) Emergency and safety equipment training and checking |  |  |  |  |  |  |
|  |  | (1) Emergency and safety equipment training should take place in conjunction with cabin/technical crew undergoing similar training with emphasis on coordinated procedures and two-way communication between the flight crew compartment and the cabin. |  |  |  |  |  |  |
|  |  | (2) On the initial conversion course and on subsequent conversion courses as applicable, the following should be addressed: (i) Instruction on first-aid in general (initial conversion course only); instruction on first-aid as relevant to the aircraft type of operation and crew complement, including those situations where no cabin crew is required to be carried (initial and subsequent). (ii) Aero-medical topics, including: (A) hypoxia; (B) hyperventilation; (C) contamination of the skin/eyes by aviation fuel or hydraulic or other fluids; (D) hygiene and food poisoning; and (E) malaria. (iii) The effect of smoke in an enclosed area and actual use of all relevant equipment in a simulated smoke-filled environment. (iv) Actual fire fighting, using equipment representative of that carried in the aircraft on an actual or simulated fire except that, with Halon extinguishers, an alternative extinguisher may be used. (v) The operational procedures of security, rescue and emergency services. (vi) Survival information appropriate to their areas of operation (e.g. polar, desert, jungle or sea) and training in the use of any survival equipment required to be carried. (vii) A comprehensive drill to cover all ditching procedures where flotation equipment is carried. This should include practice of the actual donning and inflation of a life-jacket, together with a demonstration or audio-visual presentation of the inflation of life-rafts and/or slide-rafts and associated equipment. This practice should, on an initial conversion course, be conducted using the equipment in water, although previous certified training with another operator or the use of similar equipment will be accepted in lieu of further wet-drill training. (viii) Instruction on the location of emergency and safety equipment, correct use of all appropriate drills, and procedures that could be required of flight crew in different emergency situations. Evacuation of the aircraft (or a representative training device) by use of a slide where fitted should be included when the operations manual procedure requires the early evacuation of flight crew to assist on the ground. |  |  |  |  |  |  |
|  |  | (3) Operations where no cabin crew is required (i) Passenger handling Other than general training on dealing with people, emphasis should be placed on the following: (A) advice on the recognition and management of passengers who appear or are intoxicated with alcohol, under the influence of drugs or aggressive; (B) methods used to motivate passengers and the crowd control necessary to expedite an aircraft evacuation; and (C) the importance of correct seat allocation with reference to aircraft mass and balance. Particular emphasis should also be given on the seating of special categories of passengers. (ii) Discipline and responsibilities Emphasis should be placed on discipline and an individual’s responsibilities in relation to: (A) his or her ongoing competence and fitness to operate as a crew member with special regard to flight and duty time limitation (FTL) requirements; and (B) security procedures. (iii) Passenger briefing/safety demonstrations Training should be given in the preparation of passengers for normal and emergency situations. |  |  |  |  |  |  |
|  |  | (d) Flight training |  |  |  |  |  |  |
|  |  | (1) Flight training should be conducted to familiarise the flight crew member thoroughly with all aspects of limitations and normal, abnormal and emergency procedures associated with the aircraft and should be carried out by suitably qualified class and type rating instructors and/or examiners. For specific operations, such as steep approaches, ETOPS, or operations based on QFE, additional training should be carried out, based on any additional elements of training defined for the aircraft type in the operational suitability data in accordance with Commission Regulation (EU) No 748/2012, where they exist. |  |  |  |  |  |  |
|  |  | (2) In planning flight training on aircraft with a flight crew of two or more, particular emphasis should be placed on the practice of LOFT with emphasis on CRM, and the use of crew coordination procedures, including coping with incapacitation. |  |  |  |  |  |  |
|  |  | (3) Normally, the same training and practice in the flying of the aircraft should be given to co-pilots as well as commanders. The ‘flight handling’ sections of the syllabus for commanders and co-pilots alike should include all the requirements of the operator proficiency check required by ORO.FC.230. |  |  |  |  |  |  |
|  |  | (4) Unless the type rating training programme has been carried out in an FSTD usable for ZFTT, the training should include at least three take-offs and landings in the aircraft. |  |  |  |  |  |  |
|  |  | (e) Operator proficiency check (1) For aeroplanes, the operator proficiency check that is part of the operator’s conversion checking should follow the provisions in AMC1 ORO.FC.230. For EBT, the operator should include either an EBT module in accordance with ORO.FC.231 or an OPC in accordance with AMC1 ORO.FC.230. (2) For helicopters, the operator proficiency check that is part of the operator’s conversion checking should include at least the following emergency/abnormal procedures as relevant to the helicopter and operations: (i) engine fire; (ii) interior helicopter fire or smoke; (iii) emergency operation of undercarriage; (iv) hydraulic failure; (v) electrical failure; (vi) flight and engine control system malfunctions; (vii) recovery from unusual attitudes; (viii) landing with one or more engine(s) inoperative; (ix) instrument meteorological conditions (IMC) autorotation techniques; (x) autorotation to a designated area; (xi) pilot incapacitation; (xii) directional control failures and malfunctions; and (xiii) engine failure and if relevant, relight; and for multi-engined helicopters: (xiv) engine failure during take-off before decision point; (xv) engine failure during take-off after decision point; (xvi) engine failure during landing before decision point; and (xvii) engine failure during landing after decision point. (3) For helicopter pilots required to engage in IFR operations, the proficiency check should include the following additional normal/abnormal/emergency procedures: (i) 3D approach operation to minima; (ii) go-around on instruments; (iii) 2D approach operation to minima; (iv) if relevant, at least one of the 3D or 2D approach operations should be an RNP APCH or RNP AR APCH operation; (v) in the case of multi-engined helicopters, a simulated failure of one engine to be included in either the 3D or 2D approach operation to minima; and (vi) where appropriate to the helicopter type, approach with flight control system/flight director system malfunctions, flight instrument and navigation equipment failures. (4) For helicopters, the flight crew should be assessed on their CRM skills in accordance with the methodology described in AMC1 ORO.FC.115 and as specified in the operations manual. (5) The use of FSTDs, composition of the flight crew, and the possible combinations with training or with the licence proficiency check should be defined as per AMC1 ORO.FC.230. |  |  |  |  |  |  |
|  |  | (f) Line flying under supervision (LIFUS) |  |  |  |  |  |  |
|  |  | (1) Following completion of flight training and checking as part of the operator’s conversion course, each flight crew member should operate a minimum number of sectors and/or flight hours under the supervision of a flight crew member nominated by the operator. |  |  |  |  |  |  |
|  |  | (2) The minimum flight sectors/hours should be specified in the operations manual and should be determined by the following: (i) previous experience of the flight crew member; (ii) complexity of the aircraft; and (iii) the type and area of operation. |  |  |  |  |  |  |
|  |  | (3) For performance class B aeroplanes, the amount of LIFUS required is dependent on the complexity of the operations to be performed. |  |  |  |  |  |  |
| 76. | AMC2 ORO.FC.220 | ED Decision 2014/017/R Operator conversion training syllabus — flight engineers |  |  |  |  |  |  |
|  |  | (a) Operator conversion training for flight engineers should approximate to that of pilots. |  |  |  |  |  |  |
|  |  | (b) If the flight crew includes a pilot with the duties of a flight engineer, he/she should, after training and the initial check in these duties, operate a minimum number of flight sectors under the supervision of a nominated additional flight crew member. The minimum figures should be specified in the operations manual and should be selected after due note has been taken of the complexity of the aircraft and the experience of the flight crew member. |  |  |  |  |  |  |
| 77. | AMC3 ORO.FC.220 | Training programmes The operator should ensure that training programmes include the relevant de-identified feedback from the management system, including occurrence reporting and flight data monitoring programmes. |  |  |  |  |  |  |
| 78. | AMC1 ORO.FC.220(b) | Assignment to flights during an operator conversion course — helicopters |  |  |  |  |  |  |
|  |  | (a) A group of helicopter types should include either only single-engined turbine helicopters operated only under VFR or only single-engined piston helicopters operated only under VFR. |  |  |  |  |  |  |
|  |  | (b) The flight crew member should only be assigned to flights on a helicopter within the same group of helicopter types as the type used for the operator conversion training and checking. |  |  |  |  |  |  |
|  |  | (c) Once an operator conversion course has been commenced, the flight crew member should not start another operator conversion course on another helicopter type until that course is completed or terminated. |  |  |  |  |  |  |
| 79. | GM1 ORO.FC.220(b) | ED Decision 2014/017/R Completion of an operator’s conversion course |  |  |  |  |  |  |
|  |  | (a) The operator conversion course is deemed to have started when the flight training has begun. The theoretical element of the course may be undertaken ahead of the practical element. |  |  |  |  |  |  |
|  |  | (b) Under certain circumstances the course may have started and reached a stage where, for unforeseen reasons, it is not possible to complete it without a delay. In these circumstances, the operator may allow the pilot to revert to the original type. |  |  |  |  |  |  |
|  |  | (c) Before the resumption of the operator conversion course, the operator should evaluate how much of the course needs to be repeated before continuing with the remainder of the course. |  |  |  |  |  |  |
| 80. | GM1 ORO.FC.220(c) | ED Decision 2016/008/R Operator conversion course (OCC) for multi-crew pilot licence (MPL) holders |  |  |  |  |  |  |
|  |  | When defining the amount of training for MPL holders, who undertake their first conversion course on a new type or at an operator other than the one that was involved in their training for the MPL, the operator should put a process in place to ensure that corrective action can be taken if post-MPL licence training evaluation indicates the need to do so. |  |  |  |  |  |  |
| 81. | GM1 ORO.FC.220(d) | ED Decision 2014/017/R Line flying under supervision |  |  |  |  |  |  |
|  |  | (a) Line flying under supervision provides the opportunity for a flight crew member to carry into practice the procedures and techniques he/she has been made familiar with during the ground and flight training of an operator conversion course. This is accomplished under the supervision of a flight crew member specifically nominated and trained for the task. At the end of line flying under supervision the respective crew member should be able to perform a safe and efficient flight conducted within the tasks of his/her crew member station. |  |  |  |  |  |  |
|  |  | (b) A variety of reasonable combinations may exist with respect to: (1) a flight crew member's previous experience; (2) the complexity of the aircraft concerned; and (3) the type of route/role/area operations. |  |  |  |  |  |  |
|  |  | (c) Aeroplanes The following minimum figures for details to be flown under supervision are guidelines for operators to use when establishing their individual requirements: (1) turbo-jet aircraft |  |  |  |  |  |  |
|  |  | (i) co-pilot undertaking first operator conversion course: (A) total accumulated 100 hours or minimum 40 flight sectors; |  |  |  |  |  |  |
|  |  | (ii) co-pilot upgrading to commander: (A) minimum 20 flight sectors when converting to a new type; (B) minimum 10 flight sectors when already qualified on the aeroplane type. |  |  |  |  |  |  |
| 82. | AMC1 ORO.FC.220(f) | Specific conversion course — suitably qualified commander nominated by the operator — pilots who temporarily join the operator and will be nominated to conduct line checks |  |  |  |  |  |  |
|  |  | (a) In some cases, operational circumstances may require the operator to develop a specific conversion course to nominate pilots as suitably qualified commanders to conduct line checks in accordance with the requirements of ORO.FC.146. In this case, the operator conversion training should include training as follows: (1) normal procedures, which include flight planning and ground-handling and flight operations, including performance, mass and balance, fuel schemes, selection of alternates, and ground de-icing/anti-icing; (2) abnormal and emergency procedures, which include pilot incapacitation as applicable. |  |  |  |  |  |  |
|  |  | (b) The operator should ensure that the line checker is familiar with: (1) the operating procedures and the use of checklists used by the operator; (2) the emergency and safety equipment installed or carried on the operated aircraft. |  |  |  |  |  |  |
|  |  | (c) After the completion of the specific conversion course, the following apply: (1) The line checker should not exercise duties at the controls of the aircraft. (2) The line checker should only conduct recurrent line checks of pilots whose previous line check has not expired, in accordance with ORO.FC.230. |  |  |  |  |  |  |
|  |  | (d) The validity of the specific conversion course should be limited to 6 months. |  |  |  |  |  |  |
| 83. | GM1 ORO.FC.220(f) | Specif conversion course to be use temporarily for a limited number of pilots — new AOC or addition of a new aircraft type or class to the fleet |  |  |  |  |  |  |
|  |  | For a new AOC or for the addition of a new aircraft type or class to the fleet, the operator may contact the competent authority to agree on a specific conversion course to be included in the operations manual (CAT requires approval in accordance with ORO.FC.145 point (c)) to be used temporarily for a limited number of pilots. The specific course may include an agreement on the minimum experience of the pilots, the required experience of the line supervisor and line checkers amongst others. |  |  |  |  |  |  |
| 84. | AMC1 ORO.FC.220&230 | ED Decision 2019/005/R Upset prevention and recovery training (UPRT) for complex motor-powered aeroplanes with a maximum operational passenger seating configuration (MOPSC) of more than 19 |  |  |  |  |  |  |
|  |  | (a) Upset prevention training should: |  |  |  |  |  |  |
|  |  | (1) consist of ground training and flight training in an FSTD or an aeroplane; |  |  |  |  |  |  |
|  |  | (2) include upset prevention elements from Table 1 for the conversion training course; and |  |  |  |  |  |  |
|  |  | (3) include upset prevention elements in Table 1 for the recurrent training programme at least every 12 calendar months, such that all the elements are covered over a period not exceeding 3 years. |  |  |  |  |  |  |
|  |  | Table 1: Elements and respective components of upset prevention training |  |  |  |  |  |  |
|  |  | Elements and components; Ground training; FSTD/ Aeroplane training. A. Aerodynamics. 1.; General aerodynamic characteristics; •. 2.; Aeroplane certification and limitations; •. 3.; Aerodynamics (high and low altitudes); •; •. 4.; Aeroplane performance (high and low altitudes); •; •. 5.; Angle of attack (AOA) and stall awareness; •; •. 6.; Stick shaker or other stall-warning device activation (as applicable); •; •. 7.; Stick pusher (as applicable); •; •. 8.; Mach effects (if applicable to the aeroplane type); •; •. 9.; Aeroplane stability; •; •. 10.; Control surface fundamentals; •; •. 11.; Use of trims; •; •. 12.; Icing and contamination effects; •; •. 13.; Propeller slipstream (as applicable); •; •. B. Causes of and contributing factors to upsets. 1.; Environmental; •. 2.; Pilot-induced; •. 3.; Mechanical (aeroplane systems); •. C. Safety review of accidents and incidents relating to aeroplane upsets. 1.; Safety review of accidents and incidents relating to aeroplane upsets; •. D. g-load awareness and management. 1.; Positive/negative/increasing/decreasing g-loads; •; •. 2.; Lateral g awareness (sideslip); •; •. 3.; g-load management; •; •. E. Energy management. 1.; Kinetic energy vs potential energy vs chemical energy (power); •; •. F. Flight path management. 1.; Relationship between pitch, power and performance; •; •. 2.; Performance and effects of differing power plants (if applicable); •; •. 3.; Manual and automation inputs for guidance and control; •; •. 4.; Type-specific characteristics; •; •. 5.; Management of go-arounds from various stages during the approach; •; •. 6.; Automation management; •; •. 7.; Proper use of rudder; •; •. G. Recognition. 1.; Type-specific examples of physiological, visual and instrument clues during developing and developed upsets; •; •. 2.; Pitch/power/roll/yaw; •; •. 3.; Effective scanning (effective monitoring); •; •. 4.; Type-specific stall protection systems and cues; •; •. 5.; Criteria for identifying stalls and upsets; •; •. |
|  |  | Elements and components; Ground training; FSTD/ Aeroplane training. H. System malfunction (including immediate handling and subsequent operational considerations, as applicable). 1.; Flight control defects; •; •. 2.; Engine failure (partial or full); •; •. 3.; Instrument failures; •; •. 4.; Loss of reliable airspeed; •; •. 5.; Automation failures; •; •. 6.; Fly-by-wire protection degradations; •; •. 7.; Stall protection system failures including icing alerting systems; •; •. I. Manual handling skills (no autopilot, no autothrust/autothrottle and, where possible, without flight directors). 1.; Flight at different speeds, including slow flight, and altitudes within the full normal flight envelope; •. 2.; Procedural instrument flying and manoeuvring including instrument departure and arrival; •. 3.; Visual approach; •. 4.; Go-arounds from various stages during the approach (refer to point (d) of GM1 to Appendix 9 to Part-FCL1 for further guidance on go-around training); •; •. 5.; Steep turns; •. |
|  |  | (b) Upset recovery training should: |  |  |  |  |  |  |
|  |  | (1) consist of ground training and flight training in an FFS qualified for the training task; |  |  |  |  |  |  |
|  |  | (2) be completed from each seat in which a pilot’s duties require him/her to operate; and |  |  |  |  |  |  |
|  |  | (3) include the recovery exercises in Table 2 for the recurrent training programme, such that all the exercises are covered over a period not exceeding 3 years. |  |  |  |  |  |  |
|  |  | Table 2: Exercises for upset recovery training |  |  |  |  |  |  |
|  |  | Exercises; Ground training; FFS training. A. Recovery from developed upsets. 1.; Timely and appropriate intervention; •; •. 2.; Recovery from stall events, in the following configurations; — take-off configuration, — clean configuration low altitude, — clean configuration near maximum operating altitude, and — landing configuration during the approach phase.; •; •. 3.; Recovery from nose high at various bank angles; •; •. 4.; Recovery from nose low at various bank angles; •; •. 5.; Consolidated summary of aeroplane recovery techniques; •; •. |
|  |  | (c) The operator should ensure that personnel providing FSTD UPRT are competent and current to deliver the training, and understand the capabilities and limitations of the device used. |  |  |  |  |  |  |
|  |  | (d) An FFS that is used for the training referred to in point (b)(1) should be qualified in accordance with the special evaluation requirements set out in CS-FSTD(A) (Issue 2 or later). |  |  |  |  |  |  |
| 85. | AMC2 ORO.FC.220&230 | ED Decision 2015/012/R Upset prevention and recovery training (UPRT) for complex motor-powered aeroplanes with a maximum operational passenger seating configuration (MOPSC) of 19 or less |  |  |  |  |  |  |
|  |  | (a) Upset prevention training should: (1) consist of ground training and flight training in an FSTD or an aeroplane; (2) include upset prevention elements in Table 1 of AMC1 ORO.FC.220&230 for the conversion training course; and (3) include upset prevention elements in Table 1 of AMC1 ORO.FC.220&230 for the recurrent training programme at least every 12 calendar months, such that all the elements are covered over a period not exceeding 3 years. |  |  |  |  |  |  |
|  |  | (b) Upset recovery training should: (1) consist of ground training and flight training in an FFS qualified for the training task, if available; (2) be completed from each seat in which a pilot’s duties require him/her to operate; and (3) include the recovery exercises in Table 2 of AMC1 ORO.FC.220&230 for the recurrent training programme, such that all the exercises are covered over a period not exceeding 3 years. |  |  |  |  |  |  |
|  |  | (c) The operator should ensure that personnel providing FSTD UPRT are competent and current to deliver the training, and understand the capabilities and limitations of the device used. |  |  |  |  |  |  |
|  |  | (d) An FFS that is used for the training referred to in point (b)(1) should be qualified in accordance with the special evaluation requirements set out in CS-FSTD(A) (Issue 2 or later). |  |  |  |  |  |  |
| 86. | GM1 ORO.FC.220&230 | ED Decision 2019/005/R Upset prevention and recovery training (UPRT) for complex motor-powered aeroplanes |  |  |  |  |  |  |
|  |  | The objective of the UPRT is to help flight crew acquire the required competencies in order to prevent or recover from a developing or developed aeroplane upset. Prevention training prepares flight crew to avoid incidents whereas recovery training prepares flight crew to prevent an accident once an upset condition has developed. |  |  |  |  |  |  |
|  |  | Human factors |  |  |  |  |  |  |
|  |  | Threat and Error Management (TEM) and Crew Resource Management (CRM) principles should be integrated into the UPRT. In particular, the surprise and startle effect, and the importance of resilience development should be emphasised Training should also emphasise that an actual upset condition may expose flight crew to significant physiological and psychological challenges, such as visual illusions, spatial disorientation and unusual g-forces, with the objective to develop strategies to deal with such challenges. |  |  |  |  |  |  |
|  |  | Use of fstd for UPRT |  |  |  |  |  |  |
|  |  | The use of an FSTD provides valuable training without the risks associated with aeroplane training. The training envelope (envelope within which all training exercises will be carried out) should be specified by the operator in terms of the range of attitudes, speed and g-loads that can be used for training, taking into account: |  |  |  |  |  |  |
|  |  | (1) the training environment; |  |  |  |  |  |  |
|  |  | (2) the capabilities of the instructors; and |  |  |  |  |  |  |
|  |  | (3) in the case of training in FSTDs, the limitations of the FSTD (as per GM15 to Annex I (Definitions) to Commission Regulation (EU) No 965/2012 for the FSTD training envelope); and |  |  |  |  |  |  |
|  |  | (4) in the case of training in aeroplanes, the capabilities and certification of the aeroplane, while considering a margin of safety in order to ensure that unintentional deviations from the training envelope will not exceed aeroplane limitations. Different training envelopes may be specified for different aeroplane types even within a single training course. |  |  |  |  |  |  |
|  |  | Additional guidance Specific guidance to the UPRT elements and exercises contained in the AMC is available from the latest revision of the ICAO Document 10011 (‘Manual on UPRT’). Further guidance is available in: — Revision 2 (as regards training scenarios for UPRT) and Revision 3 of the Aeroplane Upset Recovery Training Aid (AURTA (Revision 2) / AUPRTA (Revision 3)); and — the Flight Safety Foundation Publication (‘A Practical Guide for Improving Flight Path Monitoring’), November 2014. |  |  |  |  |  |  |
| 87. | GM2 ORO.FC.220&230 | ED Decision 2019/005/R Upset prevention training for complex motor-powered aeroplanes |  |  |  |  |  |  |
|  |  | The recurrent training should prioritise the upset prevention elements and respective components according to the operator’s safety risk assessment. Upset prevention training should use a combination of manoeuvre-based and scenario-based training. Scenario-based training may be used to introduce flight crew to situations which, if not correctly managed, could lead to an upset condition. Relevant TEM and CRM aspects should be included in scenario-based training and the flight crew should understand the limitations of the FSTD in replicating the physiological and psychological aspects of exposure to upset prevention scenarios. In order to avoid negative training and negative transfer of training, operators should ensure that the selected upset prevention scenarios and exercises take into consideration the limitations of the FSTD and the extent to which it represents the handling characteristics of the actual aeroplane. If it is determined that the FSTD is not suitable, the operator should ensure that the required training outcome can be achieved by other means. Go-arounds from various stages during the approach Guidance on go-around training is provided in point (d) of GM1 to Appendix 9 to Part-FCL. |  |  |  |  |  |  |
| 88. | GM3 ORO.FC.220&230 | ED Decision 2019/005/R Upset recovery training for complex motor-powered aeroplanes |  |  |  |  |  |  |
|  |  | The upset recovery training exercises should be manoeuvre-based, which enables flight crew to apply their handling skills and recovery strategy whilst leveraging CRM principles to return the aeroplane from an upset condition to a stabilised flight path. The flight crew should understand the limitations of the FFS in replicating the physiological and psychological aspects of upset recovery exercises. In order to avoid negative training and negative transfer of training, operators should ensure that the selected upset recovery exercises take into consideration the limitations of the FFS. |  |  |  |  |  |  |
|  |  | Stall event recovery training |  |  |  |  |  |  |
|  |  | It is of utmost importance that stall event recovery training takes into account the capabilities of the FFS used. To deliver stall event recovery training, the FFS should be qualified against the relevant UPRT elements of CS-FSTD Issue 2. Stall event recovery training should include training up to the stall (approach-to-stall). Post-stall training may be delivered provided the device has been qualified against the relevant optional elements of CS-FSTD Issue 2 and the operator demonstrates that negative training or negative transfer of training is avoided. A ‘stall event’ is defined as an occurrence whereby the aeroplane experiences one or more conditions associated with an approach-to-stall or a stall. |  |  |  |  |  |  |
|  |  | Stall event recovery training should emphasise the requirement to reduce the angle of attack (AOA) whilst accepting the resulting altitude loss. High-altitude stall event training should be included so that flight crew appreciate the aeroplane control response, the significant altitude loss during the recovery, and the increased time required. The training should also emphasise the risk of triggering a secondary stall event during the recovery. |  |  |  |  |  |  |
|  |  | Recovery from a stall event should always be in accordance with the stall event recovery procedures of the OEMs. If an OEM-approved recovery procedure does not exist, operators should develop and train the aeroplane-specific stall recovery procedure based on the template in Table 1 below. Refer to Revision 3 of the Airplane Upset Prevention and Recovery Training Aid (AUPRTA) for a detailed explanation and rationale on the stall event recovery template as recommended by the OEMs. |  |  |  |  |  |  |
|  |  | Table 1: Recommended Stall Event Recovery Template |  |  |  |  |  |  |
|  |  | Stall Event Recovery Template. Pilot Flying - Immediately do the following at first indication of a stall (aerodynamic buffeting, reduced roll stability and aileron effectiveness, visual or aural cues and warnings, reduced elevator (pitch) authority, inability to maintain altitude or arrest rate of descent, stick shaker activation (if installed).) – during any flight phases except at lift-off.. Pilot Flying (PF); Pilot Monitoring (PM). 1.; AUTOPILOT – DISCONNECT (A large out-of-trim condition could be encountered when the autopilot is disconnected.); MONITOR airspeed and attitude throughout the recovery and ANNOUNCE any continued divergence. 2.; AUTOTHRUST/AUTOTHROTTLE – OFF. 3.; a) NOSE DOWN PITCH CONTROL apply until stall warning is eliminated b) NOSE DOWN PITCH TRIM (as needed) (Reduce the angle of attack (AOA) whilst accepting the resulting altitude loss.). 4.; BANK – WINGS LEVEL. 5.; THRUST – ADJUST (as needed) (Thrust reduction for aeroplanes with underwing mounted engines may be needed). 6.; SPEEDBRAKES/SPOILERS - RETRACT. 7.; When airspeed is sufficiently increasing - RECOVER to level flight (Avoid the secondary stall due premature recovery or excessive g-loading.). |
|  |  | Nose high and nose low recovery training |  |  |  |  |  |  |
|  |  | Nose-high and nose-low recovery training should be in accordance with the strategies recommended by the OEMs contained in the Tables 2 and 3 below. As the OEM procedures always take precedence over the recommendations, operators should consult their OEM on whether any approved type-specific recovery procedures are available prior to using the templates. Refer to Revision 3 of the Airplane Upset Prevention and Recovery Training Aid (AUPRTA) for a detailed explanation and rationale on the nose high and nose low recovery strategies as recommended by the OEMs. |  |  |  |  |  |  |
|  |  | Table 2: Recommended Nose High Recovery Strategy Template |  |  |  |  |  |  |
|  |  | Nose HIGH Recovery Strategy. Either pilot - Recognise and confirm the developing situation by announcing: ‘Nose High’. PF; PM. 1.; AUTOPILOT – DISCONNECT (A large out of trim condition could be encountered when the AP is disconnected.); MONITOR airspeed and attitude throughout the recovery and ANNOUNCE any continued divergence. 2.; AUTOTHRUST/AUTOTHROTTLE – OFF. 3.; APPLY as much nose-down control input as required to obtain a nose-down pitch rate. 4.; THRUST – ADJUST (if required) (Thrust reduction for aeroplanes with underwing mounted engines may be needed.). 5.; ROLL – ADJUST (if required) (Avoid exceeding 60 degrees bank.). 6.; When airspeed is sufficiently increasing - RECOVER to level flight (Avoid the secondary stall due premature recovery or excessive g-loading.). NOTE: 1) Recovery to level flight may require use of pitch trim. 2) If necessary, consider reducing thrust in aeroplanes with underwing-mounted engines to aid in achieving nose-down pitch rate. 3) WARNING: Excessive use of pitch trim or rudder may aggravate the upset situation or may result in high structural loads.. |
|  |  | Table 3: Recommended Nose Low Recovery Strategy Template |  |  |  |  |  |  |
|  |  | Nose LOW Recovery Strategy Template. Either pilot - Recognise and confirm the developing situation by announcing: ‘Nose Low’ (If the autopilot or autothrust/autothrottle is responding correctly, it may not be appropriate to decrease the level of automation while assessing if the divergence is being stopped.). PF; PM. 1.; AUTOPILOT – DISCONNECT (A large out of trim condition could be encountered when the AP is disconnected.); MONITOR airspeed and attitude throughout the recovery and ANNOUNCE any continued divergence. 2.; AUTOTHRUST/AUTOTHROTTLE – OFF. 3.; RECOVERY from stall if required. 4.; ROLL in the shortest direction to wings level. (It may be necessary to reduce the g-loading by applying forward control pressure to improve roll effectiveness). 5.; THRUST and DRAG – ADJUST (if required). 6.; RECOVER to level flight. (Avoid the secondary stall due premature recovery or excessive g-loading.). NOTE: 1) Recovery to level flight may require use of pitch trim. 2) WARNING: Excessive use of pitch trim or rudder may aggravate the upset situation or may result in high structural loads.. |
| 89. | GM5 ORO.FC.220&230 | ED Decision 2019/005/R Personnel providing fstd upset prevention and recovery training (UPRT) |  |  |  |  |  |  |
|  |  | It is of paramount importance that personnel providing UPRT in FSTDs have the specific competence to deliver such training, which may not have been demonstrated during previous instructor qualification training. Operators should, therefore, have a comprehensive training and standardisation programme in place, and may need to provide FSTD instructors with additional training to ensure such instructors have and maintain complete knowledge and understanding of the UPRT operating environment, and skill sets. Standardisation and training should ensure that personnel providing FSTD UPRT: |  |  |  |  |  |  |
|  |  | (1) are able to demonstrate the correct upset recovery techniques for the specific aeroplane type; |  |  |  |  |  |  |
|  |  | (2) understand the importance of applying type-specific Original Equipment Manufacturers (OEMs) procedures for recovery manoeuvres; |  |  |  |  |  |  |
|  |  | (3) are able to distinguish between the applicable SOPs and the OEMs recommendations (if available); |  |  |  |  |  |  |
|  |  | (4) understand the capabilities and limitations of the FSTD used for UPRT, based on the applicable FSTD training envelope; |  |  |  |  |  |  |
|  |  | (5) are aware of the potential of negative transfer of training that may exist when training outside the capabilities of the FSTD; |  |  |  |  |  |  |
|  |  | (6) understand and are able to use the IOS of the FSTD in the context of effective UPRT delivery; |  |  |  |  |  |  |
|  |  | (7) understand and are able to use the FSTD instructor tools available for providing accurate feedback on flight crew performance; |  |  |  |  |  |  |
|  |  | (8) understand the importance of adhering to the FSTD UPRT scenarios that have been validated by the training programme developer; and |  |  |  |  |  |  |
|  |  | (9) understand the missing critical human factor aspects due to the limitations of the FSTD and convey this to the flight crew receiving the training. |  |  |  |  |  |  |
| 90. | ORO.FC.230 | Regulation(EU) 2021/2237] Recurrent training and checking |  |  |  |  |  |  |
|  |  | (a) Each flight crew member shall complete recurrent training and checking relevant to the type or variant, and associated equipment of aircraft on which they operate. |  |  |  |  |  |  |
|  |  | (b) Operator proficiency check |  |  |  |  |  |  |
|  |  | (1) Each flight crew member shall complete operator proficiency checks as part of the normal crew complement. (2) When the flight crew member will be required to operate under IFR, the operator proficiency check shall be conducted without external visual reference, as appropriate. (3) The validity period of the operator proficiency check shall be 6 calendar months. For operations under VFR by day of performance class B aeroplanes that are conducted during seasons not longer than 8 consecutive months, one operator proficiency check shall be sufficient. The proficiency check shall be undertaken before commencing CAT operations. |  |  |  |  |  |  |
|  |  | (c) Line check |  |  |  |  |  |  |
|  |  | Each flight crew member shall complete a line check on the aircraft. The validity period of the line check shall be 12 calendar months.. |  |  |  |  |  |  |
|  |  | - Check the defined criteria for the selection of the route - Check that pilots are checked PF and PM, when such duties are assigned (one sector PF and one sector PM) - Check that it is foreseen that the line checker occupies an observer seat (or a forward-facing passenger seat allowing a good view). If not possible, the line checked might occupy a pilot seat. |  |  |  |  |  |  |
|  |  | (d) Each flight crew member shall complete recurrent training and checking on the location and use of all emergency and safety equipment carried on board the aircraft. The validity period of an emergency and safety equipment training and checking shall be 12 calendar months. |  |  |  |  |  |  |
|  |  | (e) CRM training |  |  |  |  |  |  |
|  |  | (1) Elements of CRM shall be integrated into all appropriate phases of the recurrent training. (2) Each flight crew member shall undergo specific modular CRM training. All major topics of CRM training shall be covered by distributing modular training sessions as evenly as possible over each 3-year period. |  |  |  |  |  |  |
|  |  | (f) Each flight crew member shall undergo ground training and flight training in an FSTD or an aircraft, or a combination of FSTD and aircraft training, at least every 12 calendar months. |  |  |  |  |  |  |
|  |  | - Check that programme of the aircraft/FSTD training allows to ensure that all major failures of aircraft systems and associated procedures to be covered within 3Y. - Check that it includes training on the use of the CDFA technique. - Check that the recurrent aircraft/FSTD training of a single task or manoeuvre is separated from, and does not take place at the same time as, an OPC of the item. |  |  |  |  |  |  |
| 91. | AMC1 ORO.FC.230 | ED Decision 2016/019/R Recurrent training and checking syllabus (a) Recurrent training Recurrent training should comprise the following: |  |  |  |  |  |  |
|  |  | (1) Ground training (i) The ground training programme should include: (A) aircraft systems; (B) normal procedures, which include flight planning and ground-handling and flight operations, including performance, mass and balance, fuel schemes, selection of alternates, and ground de-icing/anti-icing; (C) abnormal and emergency procedures, which include pilot incapacitation as applicable; (D) a review of relevant samples of accident/incident and occurrences to increase awareness of the occurrences that may be relevant for the intended operation. (ii) Knowledge of the ground training should be verified by a questionnaire or other suitable methods. |  |  |  |  |  |  |
|  |  | (2) Emergency and safety equipment training (i) Emergency and safety equipment training may be combined with emergency and safety equipment checking and should be conducted in an aircraft or a suitable alternative training device. |  |  |  |  |  |  |
|  |  | (ii) Every year the emergency and safety equipment training programme should include the following: (A) actual donning of a life-jacket, where fitted; (B) actual donning of protective breathing equipment, where fitted; (C) actual handling of fire extinguishers of the type used; (D) instruction on the location and use of all emergency and safety equipment carried on the aircraft; (E) instruction on the location and use of all types of exits; (F) security procedures. |  |  |  |  |  |  |
|  |  | (iii) Every 3 years the programme of training should include the following: (A) actual operation of all types of exits; (B) demonstration of the method used to operate a slide where fitted; (C) actual fire-fighting using equipment representative of that carried in the aircraft on an actual or simulated fire except that, with Halon extinguishers, an alternative extinguisher may be used; (D) the effects of smoke in an enclosed area and actual use of all relevant equipment in a simulated smoke-filled environment; (E) actual handling of pyrotechnics, real or simulated, where applicable; (F) demonstration in the use of the life-rafts where fitted. In the case of helicopters involved in extended over water operations, demonstration and use of the life-rafts. Helicopter water survival training Where life-rafts are fitted for helicopter extended overwater operations (such as sea pilot transfer, offshore operations, regular, or scheduled, coast-to-coast overwater operations), a comprehensive wet drill to cover all ditching procedures should be practised by aircraft crew. This wet drill should include, as appropriate, practice of the actual donning and inflation of a life-jacket, together with a demonstration or audio-visual presentation of the inflation of life-rafts. Crews should board the same (or similar) life-rafts from the water whilst wearing a life-jacket. Training should include the use of all survival equipment carried on board life-rafts and any additional survival equipment carried separately on board the aircraft; — consideration should be given to the provision of further specialist training such as underwater escape training. Where operations are predominately conducted offshore, operators should conduct 3-yearly helicopter underwater escape training at an appropriate facility; — wet practice drill should always be given in initial training unless the crew member concerned has received similar training provided by another operator; (G) particularly in the case where no cabin crew is required, first-aid, appropriate to the aircraft type, the kind of operation and crew complement. |  |  |  |  |  |  |
|  |  | (iv) The successful resolution of aircraft emergencies requires interaction between flight crew and cabin/technical crew and emphasis should be placed on the importance of effective coordination and two-way communication between all crew members in various emergency situations. |  |  |  |  |  |  |
|  |  | (v) Emergency and safety equipment training should include joint practice in aircraft evacuations so that all who are involved are aware of the duties other crew members should perform. When such practice is not possible, combined flight crew and cabin/technical crew training should include joint discussion of emergency scenarios. |  |  |  |  |  |  |
|  |  | (vi) Emergency and safety equipment training should, as far as practicable, take place in conjunction with cabin/technical crew undergoing similar training with emphasis on coordinated procedures and two-way communication between the flight crew compartment and the cabin. |  |  |  |  |  |  |
|  |  | (3) CRM Elements of CRM training, as specified in Table 1 of AMC1 ORO.FC.115, should be integrated into all appropriate phases of recurrent training. |  |  |  |  |  |  |
|  |  | (4) Aircraft/FSTD training (i) General (A) The aircraft/FSTD training programme should be established in a way that all major failures of aircraft systems and associated procedures will have been trained in the preceding 3 year period. (B) When engine-out manoeuvres are carried out in an aircraft, the engine failure should be simulated. (C) The recurrent aircraft/FSTD training of a single task or manoeuvre should be separate from, and should not take place at the same time as, an operator proficiency check of the item.. |  |  |  |  |  |  |
|  |  | (ii) Helicopters (A) If the operator is able to demonstrate, on the basis of a compliance and risk assessment, that alternating the use of an FSTD with the use of an aircraft for this training provides equivalent standards of training with safety levels similar to those achieved using an FSTD, the aircraft may be used (alternating with the use of an FSTD) for this training to the extent necessary. (B) Where a suitable FSTD is available and accessible, it should be used to complete the following additional items: — settling with power and vortex ring; — loss of tail rotor effectiveness. |  |  |  |  |  |  |
|  |  | (5) For operations with other-than-complex motor-powered aeroplanes, all training and checking should be relevant to the type of operation and class of aeroplane on which the flight crew member operates with due account taken of any specialised equipment used. |  |  |  |  |  |  |
|  |  | (b) Recurrent checking Recurrent checking should comprise the following: |  |  |  |  |  |  |
|  |  | (1) Operator proficiency checks (i) Aeroplanes Operator proficiency checks should take place as part of the normal crew complement and should include, where applicable, the following manoeuvres as pilot flying: (A) rejected take-off when an FSTD is available to represent that specific aeroplane, otherwise touch drills only; (B) take-off with engine failure between V1 and V2 (take-off safety speed) or, if carried out in an aeroplane, at a safe speed above V2; (C) 3D approach operation to minima with, in the case of multi-engine aeroplanes, one-engine-inoperative; (D) 2D approach operation to minima; (E) at least one of the 3D or 2D approach operations should be an RNP APCH or RNP AR APCH operation; (F) missed approach on instruments from minima with, in the case of multi-engined aeroplanes, one-engine-inoperative; (G) landing with one-engine-inoperative. For single-engined aeroplanes a practice forced landing is required. |  |  |  |  |  |  |
|  |  | (2) Emergency and safety equipment checks The items to be checked should be those for which training has been carried out in accordance with (a)(2). |  |  |  |  |  |  |
|  |  | (3) Line checks (i) A line check should establish the ability to perform satisfactorily a complete line operation, including pre-flight and post-flight procedures and use of the equipment provided, as specified in the operations manual. The route chosen should be such as to give adequate representation of the scope of a pilot’s normal operations. When weather conditions preclude a manual landing, an automatic landing is acceptable. The commander, or any pilot who may be required to relieve the commander, should also demonstrate his/her ability to ‘manage’ the operation and take appropriate command decisions. |  |  |  |  |  |  |
|  |  | (ii) The flight crew should be assessed on their CRM skills in accordance with the methodology described in AMC1 ORO.FC.115 and as specified in the operations manual. |  |  |  |  |  |  |
|  |  | (iii) CRM assessment should not be used as a reason for a failure of the line check, unless the observed behaviour could lead to an unacceptable reduction in safety margin. |  |  |  |  |  |  |
|  |  | (iv) When pilots are assigned duties as pilot flying and pilot monitoring, they should be checked in both functions. |  |  |  |  |  |  |
|  |  | (v) A line check should be conducted by a commander nominated by the operator. The operator should maintain a list of nominated commanders and inform the competent authority about the persons nominated. The person conducting the line check should occupy an observer’s seat where installed. (A) For aeroplanes, in the case of long haul operations where additional operating flight crew are carried, the person conducting the line check may fulfil the function of a cruise relief pilot and should not occupy either pilot’s seat during take-off, departure, initial cruise, descent, approach and landing. (B) If an observer’s seat is not installed but a forward-facing passenger seat allows a good view and sound of the cockpit and the crew, this seat should be used as an observer’s seat. (C) If an observer’s seat is not available and cannot be installed, the commander nominated by the operator should occupy a pilot seat to conduct the line check. |  |  |  |  |  |  |
|  |  | (vi) CRM assessment during the line check (A) The CRM assessment taking place during the line check should be solely based on observations made during the initial briefing, cabin briefing, flight crew compartment briefing and those phases where the line checker occupies the observer’s seat. (B) If an observer’s seat is not available and cannot be installed, then the operator should define the best way to assess CRM taking into account the CRM principles above. |  |  |  |  |  |  |
|  |  | (vii) Complementary CRM assessment If a suitable FSTD is available and accessible for operator proficiency checks or FSTD training, then a CRM assessment should take place in a line-oriented flight scenario (LOFT or line-oriented section of the OPC) of an FSTD session. This assessment complements the CRM assessment taking place during the line check, but is not part of the line check. |  |  |  |  |  |  |
|  |  | (viii) Where a pilot is required to operate as pilot flying and pilot monitoring, they should be checked on one flight sector as pilot flying and on another flight sector as pilot monitoring. |  |  |  |  |  |  |
|  |  | (4) In the case of single-pilot operations, the recurrent checks referred to in (b)(1) and (3) should be performed in the single-pilot role in an environment representative of the operation |  |  |  |  |  |  |
|  |  | (5) In the case of single-pilot operations with helicopters, the recurrent checks referred to in (b)(1), (2) and (3) should be performed in the single-pilot role on a particular helicopter type in an environment representative of the operation. |  |  |  |  |  |  |
|  |  | (c) Flight crew incapacitation training, except single-pilot operations (1) Procedures should be established to train flight crew to recognise and handle flight crew incapacitation. This training should be conducted every year and can form part of other recurrent training. It should take the form of classroom instruction, discussion, audio-visual presentation or other similar means. (2) If an FSTD is available for the type of aircraft operated, practical training on flight crew incapacitation should be carried out at intervals not exceeding 3 years. |  |  |  |  |  |  |
|  |  | (d) Use of FSTD (1) Training and checking provide an opportunity to practice abnormal/emergency procedures that rarely arise in normal operations and should be part of a structured programme of recurrent training. This should be carried out in an FSTD when available and accessible. |  |  |  |  |  |  |
|  |  | (2) The line check should be performed in the aircraft. All other training and checking should be performed in an FSTD, or, if it is not reasonably practicable to gain access to such devices, in an aircraft of the same type or in the case of emergency and safety equipment training, in a representative training device. The type of equipment used for training and checking should be representative of the instrumentation, equipment and layout of the aircraft type operated by the flight crew member. |  |  |  |  |  |  |
|  |  | (3) Because of the unacceptable risk when simulating emergencies such as engine failure, icing problems, certain types of engine(s) (e.g. during continued take-off or go-around, total hydraulic failure), or because of environmental considerations associated with some emergencies (e.g. fuel dumping) these emergencies should preferably be covered in an FSTD. If no FSTD is available, these emergencies may be covered in the aircraft using a safe airborne simulation, bearing in mind the effect of any subsequent failure, and the exercise must be preceded by a comprehensive briefing. |  |  |  |  |  |  |
| 92. | AMC2 ORO.FC.230 | ED Decision 2014/017/R Flight engineers (a) The recurrent training and checking for flight engineers should meet the requirements for pilots and any additional specific duties, omitting those items that do not apply to flight engineers. (b) Recurrent training and checking for flight engineers should, whenever possible, take place concurrently with a pilot undergoing recurrent training and checking. © The line check should be conducted by a commander or by a flight engineer nominated by the operator, in accordance with national rules, if applicable. |  |  |  |  |  |  |
| 93. | AMC3 ORO.FC.230 | Training programmes The operator should ensure that training programmes include the relevant de-identified feedback from the management system, including occurrence reporting and flight data monitoring programmes. |  |  |  |  |  |  |
| 94. | GM1 ORO.FC.230 | ED Decision 2014/017/R Line check and proficiency training and checking |  |  |  |  |  |  |
|  |  | (a) Line checks, route and aerodrome knowledge and recent experience requirements are intended to ensure the crew member’s ability to operate efficiently under normal conditions, whereas other checks and emergency and safety equipment training are primarily intended to prepare the crew member for abnormal/emergency procedures. |  |  |  |  |  |  |
|  |  | (b) The line check is considered a particularly important factor in the development, maintenance and refinement of high operating standards, and can provide the operator with a valuable indication of the usefulness of its training policy and methods. Line checks are a test of a flight crew member’s ability to perform a complete line operation, including pre-flight and post-flight procedures and use of the equipment provided, and an opportunity for an overall assessment of their ability to perform the duties required as specified in the operations manual. The line check is not intended to determine knowledge on any particular route. |  |  |  |  |  |  |
|  |  | (c) Proficiency training and checking When an FSTD is used, the opportunity should be taken, where possible, to use LOFT. |  |  |  |  |  |  |
|  |  | Major failures — helicopters (d) The list of major failures as defined by the operator in AMC1 ORO.FC.230 for the purpose of training may be more extensive than the list covered in the 3-yearly operator proficiency checking programme for the following reasons: (1) It may happen that several training elements are covered by a single check; and (2) Certain complex system malfunctions are best explored under recurrent training, where the trainee will derive more benefit and training to proficiency is also employed. |  |  |  |  |  |  |
| 95. | ORO.FC.235 | Regulation (EU) 2021/2237 Pilot qualification to operate in either pilot's seat |  |  |  |  |  |  |
|  |  | (a) Commanders of aeroplanes whose duties require them to operate in either pilot’s seat and carry out the duties of a co-pilot, or commanders required to conduct training or checking duties shall complete additional training and checking to ensure that they are proficient in conducting the relevant normal, abnormal and emergency procedures from either seat. Such training and checking shall be specified in the operations manual. The checking may be conducted together with the operator proficiency check prescribed in ORO. FC.230(b) or in the EBT programme prescribed in ORO.FC.231. |  |  |  |  |  |  |
|  |  | (b) The additional training and checking shall include at least the following: (1) an engine failure during take-off; (2) a one-engine-inoperative approach and go-around; and (3) a one-engine-inoperative landing. |  |  |  |  |  |  |
|  |  | (c) The validity period shall be 12 calendar months. For operators with an approved EBT programme, the validity is determined by the assessment and training topics in accordance with ORO.FC.232. |  |  |  |  |  |  |
|  |  | (d) When operating in the co-pilot’s seat, the checks required by ORO.FC.230 or the assessment and training required by ORO.FC.231 for operating in the commander’s seat shall, in addition, be valid and current. |  |  |  |  |  |  |
|  |  | (e) The pilot relieving the commander shall have demonstrated, concurrent with the operator proficiency checks prescribed in ORO.FC.230(b) or the assessment and training required by ORO.FC.231, practice of drills and procedures that would not normally be his or her responsibility. Where the differences between left- and right-hand seats are not significant, practice may be conducted in either seat. |  |  |  |  |  |  |
|  |  | (f) The pilot, other than the commander, occupying the commander’s seat shall demonstrate practice of drills and procedures, concurrent with the operator proficiency checks prescribed in ORO.FC.230(b) or the assessment and training required by ORO.FC.231, which are the commander’s responsibility acting as pilot monitoring. Where the differences between left- and right-hand seats are not significant, practice may be conducted in either seat. |  |  |  |  |  |  |
| 96. | GM1 ORO.FC.235€,(f); | ED Decision 2014/017/R Differences between left and right-hand seats The differences between left- and right-hand seats may not be significant in cases where, for example, the autopilot is used. |  |  |  |  |  |  |
| 97. | ORO.FC.236 | Regulation (EU) 2021/2237 Pilot qualification to operate in either pilot’s seat — helicopters |  |  |  |  |  |  |
|  |  | (a) Helicopter pilots whose duties require them to operate in either pilot’s seat shall complete additional training and checking to ensure that they are proficient in conducting the relevant normal, abnormal and emergency procedures from either seat. The validity period of this qualification shall be 12 calendar months. |  |  |  |  |  |  |
|  |  | (b) Current FIs or TRIs on the relevant type are considered to fulfil the requirement of point (a) if they have had a FI or TRI activity in the last 6 months on that type and on the helicopter. |  |  |  |  |  |  |
| 98. | AMC1 ORO.FC.236 | General |  |  |  |  |  |  |
|  |  | (a) The operator should either conduct a check every year or alternate training and checking every year. The training and checking may take place during or together with an operator proficiency check or an aircraft/FSTD training session. |  |  |  |  |  |  |
|  |  | (b) When engine-out manoeuvres are carried out in an aircraft, the engine failure should be simulated. |  |  |  |  |  |  |
|  |  | (c) Helicopter pilots should meet one of the following criteria: (1) complete their operator proficiency checks from left- and right-hand seats, on alternate proficiency checks; or (2) for multi-engined helicopters, if two consecutive operator proficiency checks are conducted from the same seat, the pilot should complete at least the following from the other pilot’s seat: (i) an engine failure during take-off; (ii) a one-engine-inoperative approach and go-around; and (iii) a one-engine-inoperative landing; (3) for single-engined helicopters, if two consecutive operator proficiency checks are conducted from the same seat, the pilot should complete at least one autorotation training or checking from the other pilot’s seat. |  |  |  |  |  |  |
| 99. | GM1 ORO.FC.236 | Qualification to fly in either pilot’s seat — nominated commander conducting line checks In the case of a line check revalidation of a fully qualified commander in single-pilot operations, the line checker does not require a qualification to operate in either pilot’s seat, regardless of the seat he or she occupies, provided that the line checker has no pilot duties other than checking. |  |  |  |  |  |  |
| 100. | ORO.FC.240 | Regulation (EU) No 965/2012 Operation on more than one type or variant |  |  |  |  |  |  |
|  |  | (a) The procedures or operational restrictions for operation on more than one type or variant established in the operations manual and approved by the competent authority shall cover: (1) the flight crew members’ minimum experience level; (2) the minimum experience level on one type or variant before beginning training for and operation of another type or variant; (3) the process whereby flight crew qualified on one type or variant will be trained and qualified on another type or variant; and (4) all applicable recent experience requirements for each type or variant. |  |  |  |  |  |  |
|  |  | (b) INTENTIONALLY LEFT BLANK |  |  |  |  |  |  |
|  |  | (c) Point (a) shall not apply to operations of performance class B aeroplanes if they are limited to single-pilot classes of reciprocating engine aeroplanes under VFR by day. |  |  |  |  |  |  |
| 101. | AMC1 ORO.FC.240 | Operation on more than one type or variant General |  |  |  |  |  |  |
|  |  | (a) Aeroplanes (1) When a flight crew member operates more than one aeroplane class, type or variant, as determined by the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for class-single pilot or type-single pilot, but not within a single licence endorsement, the operator should ensure that the flight crew member does not operate more than: (i) three reciprocating engine aeroplane types or variants; (ii) three turbo-propeller aeroplane types or variants; (iii) one turbo-propeller aeroplane type or variant and one reciprocating engine aeroplane type or variant; or (iv) one turbo-propeller aeroplane type or variant and any aeroplane within a particular class. |  |  |  |  |  |  |
|  |  | (2) When a flight crew member operates more than one aeroplane type or variant within one or more licence endorsement, as determined by the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012, the operator should ensure that: (i) the minimum flight crew complement specified in the operations manual is the same for each type or variant to be operated; (ii) the flight crew member does not operate more than two aeroplane types or variants for which a separate licence endorsement is required, unless credits related to the training, checking, and recent experience requirements are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for the relevant types or variants; and (iii) only aeroplanes within one licence endorsement are flown in any one flight duty period, unless the operator has established procedures to ensure adequate time for preparation. |  |  |  |  |  |  |
|  |  | (3) When a flight crew member operates more than one aeroplane type or variant as determined by the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for type-single pilot and type-multi pilot, but not within a single licence endorsement, the operator should comply with points (a)(2) and (4). |  |  |  |  |  |  |
|  |  | (4) When a flight crew member operates more than one aeroplane type or variant as determined by the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for type multi-pilot, but not within a single licence endorsement, or combinations of aeroplane types or variants as determined by the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for class single-pilot and type multi-pilot, the operator should comply with the following: (i) point (a)(2); (ii) before exercising the privileges of more than one licence endorsement: |  |  |  |  |  |  |
|  |  | (A) flight crew members should have completed two consecutive OPCs and should have: — 500 hours in the relevant crew position in CAT operations with the same operator; or — for IFR and VFR night operations with performance class B aeroplanes, 100 hours or flight sectors in the relevant crew position in CAT operations with the same operator, if at least one licence endorsement is related to a class. A check flight should be completed before the pilot is released for duties as commander; |  |  |  |  |  |  |
|  |  | (B) in the case of a pilot having experience with an operator and exercising the privileges of more than one licence endorsement, and then being promoted to command with the same operator on one of those types, the required minimum experience as commander is 6 months and 300 hours, and the pilot should have completed two consecutive OPCs before again being eligible to exercise more than one licence endorsement; |  |  |  |  |  |  |
|  |  | (iii) before commencing training for and operation of another type or variant, flight crew members should have completed 3 months and 150 hours flying on the base aeroplane, which should include at least one proficiency check, unless credits related to the training, checking and recent experience requirements are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for the relevant types or variants; (iv) after completion of the initial line check on the new type, 50 hours flying or 20 sectors should be achieved solely on aeroplanes of the new type rating, unless credits related to the training, checking and recent experience requirements are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for the relevant types or variants; (v) recent experience requirements established in Commission Regulation (EU) No 1178/2011 for each type operated; (vi) the period within which line flying experience is required on each type should be specified in the operations manual; (vii) when credits are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for the relevant type or variant, this should be reflected in the training required in ORO.FC.230 and: |  |  |  |  |  |  |
|  |  | (A) ORO.FC.230 (b) requires two OPCs every year. When credits are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for OPCs to alternate between the types, each OPC should revalidate the OPC for the other type(s). The OPC may be combined with the proficiency checks for revalidation or renewal of the aeroplane type rating or the instrument rating in accordance with Commission Regulation (EU) No 1178/2011. For EBT programmes, ORO.FC.231(a)(3) requires the pilot to complete a minimum of two modules of the EBT programme, separated by a period of more than 3 months, within a 12-month period. In addition, the pilot is required to be trained according to assessment and training topics distributed across a 3-year period at the defined frequency relevant to the type or variant of aircraft. When credits are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012, EBT modules should alternate between types. The EBT modules may be combined for revalidation or renewal of the aeroplane type rating or the instrument rating in accordance with Commission Regulation (EU) No 1178/2011. When operating more than one type of different generations, the operator has to fulfil both generation table of assessment and training topics as per ORO.FC.232. |  |  |  |  |  |  |
|  |  | (B) ORO.FC.230 (c) requires one line check every year. When credits are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for line checks to alternate between types or variants, each line check should revalidate the line check for the other type or variant. For EBT programmes, ORO.FC.231(h) requires one line evaluation of competence every year. When credits are defined in the operational suitability data established in accordance with Commission Regulation (EU) No 748/2012 for line evaluation of competence to alternate between types or variants, each line evaluation of competence should revalidate the line evaluation of competence for the other type or variant. In such case, the operator should meet the requirements to extend the validity of the line evaluation of competence to 2 years. Extension to 3 years should not be allowed. |  |  |  |  |  |  |
|  |  | (C) Annual emergency and safety equipment training and checking should cover all requirements for each type. |  |  |  |  |  |  |
| 102. | ORO.FC.A.250 | Regulation (EU) 2017/363 Commanders holding a CPL(A) |  |  |  |  |  |  |
|  |  | (a) The holder of a CPL(A) (aeroplane) shall only act as commander in commercial air transport on a single-pilot aeroplane if: (1) when carrying passengers under VFR outside a radius of 50 NM (90 km) from an aerodrome of departure, he/she has a minimum of 500 hours of flight time on aeroplanes or holds a valid instrument rating; or (2) when operating on a multi-engine type under IFR, he/she has a minimum of 700 hours of flight time on aeroplanes, including 400 hours as pilot-in-command. These hours shall include 100 hours under IFR and 40 hours in multi-engine operations. The 400 hours as pilot-in-command may be substituted by hours operating as co-pilot within an established multi-pilot crew system prescribed in the operations manual, on the basis of two hours of flight time as co-pilot for one hour of flight time as pilot-in command. (3) when operating on a single-engined aeroplane under IFR, he/she has a minimum of 700 hours of flight time on aeroplanes, including 400 hours as pilot-in-command. Those hours shall include 100 hours under IFR. The 400 hours as pilot-in-command may be substituted by hours operating as co-pilot within an established multi-pilot crew system prescribed in the operations manual, on the basis of two hours of flight time as co-pilot for one hour of flight time as pilot-in command. |  |  |  |  |  |  |
|  |  | (b) For operations under VFR by day of performance class B aeroplanes (a)(1) shall not apply. |  |  |  |  |  |  |
| 103. | ORO.FC.H.250 | Regulation (EU) No 965/2012 Commanders holding a CPL(H) |  |  |  |  |  |  |
|  |  | (a) Holders of a CPL(H) (helicopter) shall only act as commanders in CAT operations on a single-pilot helicopter if: (1) when operating under IFR, they have a minimum of 700 hours total flight time on helicopters, including 300 hours as pilot-in-command. The total flight time on helicopters shall include 100 hours under IFR. Up to 50 hours instrument time performed on an FFS(H) level B or FTD level 3 qualification or higher qualified for instrument training, may be credited towards the 100 hours. The 300 hours as pilot-in-command may be substituted by hours operating as co-pilot within an established multi-pilot crew system prescribed in the operations manual on the basis of 2 hours of flight time as co-pilot for 1 hour flight time as pilot-in command; |  |  |  |  |  |  |
|  |  | (2) when operating under visual meteorological conditions (VMC) at night, he/she has: (i) a valid instrument rating; or (ii) 300 hours of flight time on helicopters, including 100 hours as pilot-in-command and 10 hours as pilot flying at night. |  |  |  |  |  |  |
| 109. |  | Guidelines for inspectors: |  |  |  |  |  |  |
|  |  | Оператора има ли описание на съдържанието и структурата на: -делата на персонала, включително на записите от всички проверки, обучения, квалификации и други |  |  |  |  |  |  |
|  |  | -проверете дела на пилоти за наличие на всички приложими документи описани в процедурата на оператора |  |  |  |  |  | (Моля, опишете кои дела са прегледани) |
|  |  | -при прегледа на делата сравнете използваните бланки с актуалното одобрено издание/ревизия на ОМD |  |  |  |  |  | (Моля, опишете кои дела са прегледани) |
|  |  | -при прегледа на делата сравнете опита на пилота и преминатото обучение и/или LPC/OPC с изискванията одобрени в ОМD и описани в Регламент 965/2012 и/или Регламент 1178/2011 и последното издание на OSD за съответния тип ВС |  |  |  |  |  | (Моля, опишете кои дела са прегледани) |
|  |  | -при прегледа на делата, проверете периодите на преминатите обучения |  |  |  |  |  | (Моля, опишете кои дела са прегледани) |
|  |  | -при прегледа на делата, проверете дали се съхраняват записите, съгласно описания период за съхранение |  |  |  |  |  | (Моля, опишете кои дела са прегледани) |

Table 1: Flight crew CRM training

| CRM training elements | Initial operator’s CRM training | Operator conversion course when changing aircraft type | Operator conversion course when joining an operator | Annual recurrent training | Command course |
| --- | --- | --- | --- | --- | --- |
| General principles |  |  |  |  |  |
| Human factors in aviation; General instructions on CRM principles and objectives; Human performance and limitations; Threat and error management. | In-depth | Not required | Required | Required | Required |
| Relevant to the individual flight crew member |  |  |  |  |  |
| Personality awareness, human error and reliability, attitudes and behaviours, self-assessment and self-critique; Stress and stress management; Fatigue and vigilance; Assertiveness, situation awareness, information acquisition and processing. | In-depth | Not required | Required | Required | In-depth |
| Relevant to the flight crew |  |  |  |  |  |
| Automation and philosophy on the use of automation | Required | In-depth | In-depth | In-depth | In-depth |
| Specific type-related differences | Required | In-depth | Not required | Required | Required |
| Monitoring and intervention | Required | In-depth | In-depth | Required | Required |
| Relevant to the entire aircraft crew |  |  |  |  |  |
| Shared situation awareness, shared information acquisition and processing; Workload management; Effective communication and coordination inside and outside the flight crew compartment; Leadership, cooperation, synergy, delegation, decision-making, actions; Resilience development; Surprise and startle effect; Cultural differences. | In-depth | Required | Required | Required | In-depth |
| Relevant to the operator and the organisation |  |  |  |  |  |
| Operator’s safety culture and company culture, standard operating procedures (SOPs), organisational factors, factors linked to the type of operations; Effective communication and coordination with other operational personnel and ground services. | In-depth | Required | In-depth | Required | In-depth |
| Case studies | In-depth | In-depth | In-depth | In-depth | In-depth |

Забележка: След класифициране на несъответствията информацията да се изпрати на оператора за съставяне на План за коригиращи действия.

Забележка: В колоните под заглавие „СЪОТВЕТСВИЕ/Compliance“ се отбелязва с  знак Х.

Забележка: При извършване на одит/инспекция на място в офиса на оператора, моля взимайте копия от прегледани документи за доказателства.

Забележка: Елементите от чек листа, които няма да бъдат прегледжани и оценявани в конкретен одит, следва да се отбележат в колонота „NC“, за да може при последващи инспекции да бъдат прегледани и оценени.

Легенда:

C - Checked

NC – Not checked

S - Satisfactory

NS - Not satisfactory

NA - Not applicable