|  | ЧЕК-ЛИСТ / checklist (Търговски въздушен транспорт) (Commercial Air Transport) МОДУЛ / Module SPA / DG |  | № SPA_DG- |  |
| --- | --- | --- | --- | --- |
| Оператор/Организация: Operator /Organisation: | Инспектор / Inspector (име, фамилия / name, surname) | Отдел: Department: | САО №: AOC No: |  |
|  |  |  | Дата: Date: |  |
| Обхват на инспекцията/одита: Scope of Audit: | Annex V, Part SPA - SPECIFIC APPROVALS Subpart G - TRANSPORT OF DANGEROUS GOODS(DG) |  |  |  |

Забележка: В полето „№“ след абревиатурата се поставя номера на заповедта за комисия.(Пример - № ORO_PR-45-01-00/01.01.2016)

| № | ОСНОВАНИЕ References | ВЪПРОСИ, НАСОКИ ЗА ИНСПЕКТОРА Questions, guidelines to inspectors | СЪОТВЕТСВИЕ Compliance |  |  |  |  | ДОКАЗАТЕЛСТВА/ ЗАБЕЛЕЖКА Evidence/Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | NC | S | NS | NA |  |
| 1. | SPA.DG.100 | Transport of dangerous goods |  |  |  |  |  |  |
|  |  | Except as provided for in Annex IV (Part-CAT), Annex VI (Part-NCC), Annex VII (Part-NCO) and Annex VIII (Part-SPO), the operator shall only transport dangerous goods by air if the operator has been approved by the competent authority. |  |  |  |  |  |  |
| 2. | SPA.DG.105 | Approval to transport dangerous goods |  |  |  |  |  |  |
|  |  | To obtain the approval to transport dangerous goods, the operator shall in accordance with the technical instructions: |  |  |  |  |  |  |
|  |  | (a) establish and maintain a training programme for all personnel involved and demonstrate to the competent authority that adequate training has been given to all personnel; |  |  |  |  |  |  |
|  |  | (b) establish operating procedures to ensure the safe handling of dangerous goods at all stages of air transport, containing information and instructions on: |  |  |  |  |  |  |
|  |  | (1) the operator’s policy to transport dangerous goods; |  |  |  |  |  |  |
|  |  | (2) the requirements for acceptance, handling, loading, stowage and segregation of dangerous goods; |  |  |  |  |  |  |
|  |  | (3) actions to take in the event of an aircraft accident or incident when dangerous goods are being carried; |  |  |  |  |  |  |
|  |  | (4) the response to emergency situations involving dangerous goods; |  |  |  |  |  |  |
|  |  | (5) the removal of any possible contamination; |  |  |  |  |  |  |
|  |  | (6) the duties of all personnel involved, especially with relevance to ground handling and aircraft handling; |  |  |  |  |  |  |
|  |  | (7) inspection for damage, leakage or contamination; |  |  |  |  |  |  |
|  |  | (8) dangerous goods accident and incident reporting |  |  |  |  |  |  |
| 3. | AMC1 SPA.DG.105(a) | TRAINING PROGRAMME |  |  |  |  |  |  |
|  |  | The operator should indicate for the approval of the training programme how the training will be carried out. For formal training courses, the course objectives, the training programme syllabus/curricula and examples of the written examination to be undertaken should be included. |  |  |  |  |  |  |
|  |  | (b) Instructors should have knowledge of training techniques as well as in the field of transport of dangerous goods by air so that the subject is covered fully and questions can be adequately answered. |  |  |  |  |  |  |
|  |  | (c) Training intended to give general information and guidance may be by any means including handouts, leaflets, circulars, slide presentations, videos, computer-based training, etc., and may take place on-the-job or off-the-job. The person being trained should receive an overall awareness of the subject. This training should include a written, oral or computer-based examination covering all areas of the training programme, showing that a required minimum level of knowledge has been acquired. |  |  |  |  |  |  |
|  |  | (d) Training intended to give an in-depth and detailed appreciation of the whole subject or particular aspects of it should be by formal training courses, which should include a written examination, the successful passing of which will result in the issue of the proof of qualification. The course may be by means of tuition, as a self-study programme, or a mixture of both. The person being trained should gain sufficient knowledge so as to be able to apply the detailed rules of the Technical Instructions. |  |  |  |  |  |  |
|  |  | (e) Training in emergency procedures should include as a minimum: |  |  |  |  |  |  |
|  |  | (1) for personnel other than crew members: |  |  |  |  |  |  |
|  |  | (i) dealing with damaged or leaking packages; and |  |  |  |  |  |  |
|  |  | (ii) other actions in the event of ground emergencies arising from dangerous goods; |  |  |  |  |  |  |
|  |  | (2) for flight crew members: |  |  |  |  |  |  |
|  |  | (i) actions in the event of emergencies in flight occurring in the passenger compartment or in the cargo compartments; and |  |  |  |  |  |  |
|  |  | (ii) the notification to ATS should an in-flight emergency occur; |  |  |  |  |  |  |
|  |  | (3) for crew members other than flight crew members: |  |  |  |  |  |  |
|  |  | (i) dealing with incidents arising from dangerous goods carried by passengers; or |  |  |  |  |  |  |
|  |  | (ii) dealing with damaged or leaking packages in flight. |  |  |  |  |  |  |
|  |  | (f) Training should be conducted at intervals of no longer than 2 years. If the recurrent training is undertaken within the last 3 calendar months of the validity period, the new validity period should be counted from the original expiry date. |  |  |  |  |  |  |
| 4. | AMC1 SPA.DG.105(b) | Provision of information in the event of an in-flight emergency |  |  |  |  |  |  |
|  |  | If an in-flight emergency occurs the pilot-in-command/commander should, as soon as the situation permits, inform the appropriate ATS unit of any dangerous goods carried as cargo on board the aircraft, as specified in the Technical Instructions. |  |  |  |  |  |  |
| 5. | GM1 SPA.DG.105(b)(6) | Personnel Personnel include all persons involved in the transport of dangerous goods, whether they are employees of the operator or not. |  |  |  |  |  |  |
| 6. | SPA.DG.110 | Dangerous goods information and documentation |  |  |  |  |  |  |
|  |  | The operator shall, in accordance with the technical instructions: |  |  |  |  |  |  |
|  |  | (a) provide written information to the pilot-in-command/commander: |  |  |  |  |  |  |
|  |  | (1) about dangerous goods to be carried on the aircraft; |  |  |  |  |  |  |
|  |  | (2) for use in responding to in-flight emergencies; |  |  |  |  |  |  |
|  |  | (b) use an acceptance checklist; |  |  |  |  |  |  |
|  |  | (c) ensure that dangerous goods are accompanied by the required dangerous goods transport document(s), as completed by the person offering dangerous goods for air transport, except when the information applicable to the dangerous goods is provided in electronic form; |  |  |  |  |  |  |
|  |  | (d) ensure that where a dangerous goods transport document is provided in written form, a copy of the document is retained on the ground where it will be possible to obtain access to it within a reasonable period until the goods have reached their final destination; |  |  |  |  |  |  |
|  |  | (e) ensure that a copy of the information to the pilot-in-command/commander is retained on the ground and that this copy, or the information contained in it, is readily accessible to the aerodromes of last departure and next scheduled arrival, until after the flight to which the information refers; |  |  |  |  |  |  |
|  |  | (f) retain the acceptance checklist, transport document and information to the pilot-in-command/commander for at least three months after completion of the flight; |  |  |  |  |  |  |
|  |  | (g) retain the training records of all personnel for at least three years. |  |  |  |  |  |  |
| 7. | AMC1 SPA.DG.110(a) | Information to the pilot-in-command /commander |  |  |  |  |  |  |
|  |  | If the volume of information provided to the pilot-in-command/commander by the operator is such that it would be impracticable to transmit it in the event of an in-flight emergency, an additional summary of the information should also be provided, containing at least the quantities and class or division of the dangerous goods in each cargo compartment. |  |  |  |  |  |  |
| 8. | AMC1 SPA.DG.110(b) | Acceptance of dangerous goods |  |  |  |  |  |  |
|  |  | (a) The operator should not accept dangerous goods unless: |  |  |  |  |  |  |
|  |  | (1) the package, overpack or freight container has been inspected in accordance with the acceptance procedures in the Technical Instructions; |  |  |  |  |  |  |
|  |  | (2) they are accompanied by two copies of a dangerous goods transport document or the information applicable to the consignment is provided in electronic form, except when otherwise specified in the Technical Instructions; and |  |  |  |  |  |  |
|  |  | (3) the English language is used for: |  |  |  |  |  |  |
|  |  | (i) package marking and labelling; and |  |  |  |  |  |  |
|  |  | (ii) the dangerous goods transport document, |  |  |  |  |  |  |
|  |  | in addition to any other language provision. |  |  |  |  |  |  |
|  |  | (b) The operator or his/her handling agent should use an acceptance checklist which allows for: |  |  |  |  |  |  |
|  |  | (1) all relevant details to be checked; and |  |  |  |  |  |  |
|  |  | (2) the recording of the results of the acceptance check by manual, mechanical or computerised means. |  |  |  |  |  |  |
| 9. | ICAO T1 4.1.1 | Dangerous Goods Training Program |  |  |  |  |  |  |
|  |  | Initial and recurrent dangerous goods training programs must be established and maintained by or on behalf of: |  |  |  |  |  |  |
|  |  | Shippers of dangerous goods, including packers and persons or organizations undertaking the responsibilities of the shipper |  |  |  |  |  |  |
|  |  | Operators |  |  |  |  |  |  |
|  |  | Ground handling agencies which perform, on behalf of the operator, the act of accepting, handling, loading, unloading, transferring or other processing of cargo, mail or stores |  |  |  |  |  |  |
|  |  | Ground handling agencies located at an airport which perform, on behalf of the operator, the act of processing passengers |  |  |  |  |  |  |
|  |  | Agencies, not located at an airport, which perform, on behalf of the operator, the act of checking in passengers |  |  |  |  |  |  |
|  |  | Freight forwarders; and |  |  |  |  |  |  |
|  |  | Agencies engaged in the security screening of passengers and their baggage and/or cargo, mail or stores |  |  |  |  |  |  |
|  |  | Designated postal operators |  |  |  |  |  |  |
| 10. | ICAO TI 4.2.1 | Training Curricula |  |  |  |  |  |  |
|  |  | Personnel must receive training in the requirements commensurate with their responsibilities |  |  |  |  |  |  |
|  |  | Familiarization training - which must be aimed at providing familiarity with the general provisions |  |  |  |  |  |  |
|  |  | Function-specific training - which must provide detailed training in the requirements applicable to the function for which that person is responsible |  |  |  |  |  |  |
|  |  | Safety training - which must cover the hazards, presented by dangerous goods, safe handling and emergency response procedures |  |  |  |  |  |  |
| 11. | ICAO TI 4.1 | Initial Dangerous Good Training Course |  |  |  |  |  |  |
|  |  | Category 12. Security Staff involved with screening of passengers, their baggage, cargo or mail |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Classification |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 11. Crew members other than flight crew member |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 10. Flight crew members, load masters & load planners |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | List of dangerous goods |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Storage and loading procedures |  |  |  |  |  |  |
|  |  | Pilots’ notification |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 9. Passenger handling staff |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category- 8. Operator and ground handling agents staff involved with storage, loading of cargo, mail and baggage |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Storage and loading procedures |  |  |  |  |  |  |
|  |  | Pilots’ notification |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category- 7. Operator and ground handling agents staff accepting cargo or mail (other than dangerous goods) |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Dangerous goods transport documents and other relevant documentation |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 6. Operator and Ground handling agents staff accepting dangerous goods |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | General requirements for shippers |  |  |  |  |  |  |
|  |  | Classification |  |  |  |  |  |  |
|  |  | List of dangerous goods |  |  |  |  |  |  |
|  |  | packing requirements |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Dangerous goods transport documents and other relevant documentation |  |  |  |  |  |  |
|  |  | Acceptance procedures |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Storage and loading procedures |  |  |  |  |  |  |
|  |  | Pilots’ notification |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 5. Staff freight forwarders involved in the handling, storage, loading of cargo or mail |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Storage and loading procedures |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 4. Staff freight forwarders involved in processing cargo or mail (other than dangerous goods) |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Dangerous goods transport documents and other relevant documentation |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 3. Staff freight forwarders involved in processing dangerous goods |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | General requirements for shippers |  |  |  |  |  |  |
|  |  | Classification |  |  |  |  |  |  |
|  |  | List of dangerous goods |  |  |  |  |  |  |
|  |  | packing requirements |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Dangerous goods transport documents and other relevant documentation |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 2. Packers |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Classification |  |  |  |  |  |  |
|  |  | List of dangerous goods |  |  |  |  |  |  |
|  |  | packing requirements |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category 1. Shippers and persons under taken the responsibilities of shippers |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | General requirements for shippers |  |  |  |  |  |  |
|  |  | Classification |  |  |  |  |  |  |
|  |  | List of dangerous goods |  |  |  |  |  |  |
|  |  | packing requirements |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Dangeroui goods transport documents and other relevant documentation |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | Provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Designated Postal Operators |  |  |  |  |  |  |
|  |  | Category A. Staff of designated postal operators involved in accepting mail containing dangerous goods |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | General requirements for shippers |  |  |  |  |  |  |
|  |  | Classification |  |  |  |  |  |  |
|  |  | List of dangerous goods |  |  |  |  |  |  |
|  |  | packing requirements |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Dangerous goods transport documents and other relevant documentation |  |  |  |  |  |  |
|  |  | Acceptance of dangerous goods listed in TI 1;2.3.2 1 |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
|  |  | Category B. Staff of designated postal operators involved in processing mail (other than dangerous goods) |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Dangerous goods transport documents and other relevant documentation |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency' procedures |  |  |  |  |  |  |
|  |  | Category C. Staff of designated postal operators involved in the handling, storage and loading of mail |  |  |  |  |  |  |
|  |  | General philosophy |  |  |  |  |  |  |
|  |  | Limitations |  |  |  |  |  |  |
|  |  | Labeling and marking |  |  |  |  |  |  |
|  |  | Recognition of undeclared dangerous goods |  |  |  |  |  |  |
|  |  | storage and loading procedures |  |  |  |  |  |  |
|  |  | provisions for passengers and crew |  |  |  |  |  |  |
|  |  | Emergency procedures |  |  |  |  |  |  |
| 12. | ICAO TI 4.2.3 | Dangerous Goods Recurrent Training |  |  |  |  |  |  |
|  |  | Recurrent training must take place within 24 months of previous training to ensure knowledge is current. |  |  |  |  |  |  |
| 13. | ICAO TI 4.2.4 | Dangerous Goods Checking |  |  |  |  |  |  |
|  |  | A test to verify understanding must be provided following training, and conformation that the test has been completed satisfactory is required |  |  |  |  |  |  |
| 14. | ICAO TI 4.2.5 | Dangerous Goods Training Records |  |  |  |  |  |  |
|  |  | A record of training must be maintained which must include: |  |  |  |  |  |  |
|  |  | The individual's name; |  |  |  |  |  |  |
|  |  | The most recent training completion date; |  |  |  |  |  |  |
|  |  | A description, copy or reference to training materials used to meet the training requirements: |  |  |  |  |  |  |
|  |  | The name and address of the organization providing the training; and |  |  |  |  |  |  |
|  |  | Evidence which shows that a test has been completed satisfactorily. |  |  |  |  |  |  |
|  |  | Training records must be retained by the employer for a minimum period of 36 months from the most recent training completion months and must be made available upon request |  |  |  |  |  |  |

Забележка: След класифициране на несъответствията информацията да се изпрати на оператора за съставяне на План за коригиращи действия.

Забележка: В колоните под заглавие „СЪОТВЕТСВИЕ/Compliance“ се отбелязва с  знак Х.

Забележка: При извършване на одит/инспекция на място в офиса на оператора, моля взимайте копия от прегледани документи за доказателства.

Забележка: Елементите от чек листа, които няма да бъдат прегледжани и оценявани в конкретен одит, следва да се отбележат в колонота „NC“, за да може при последващи инспекции да бъдат прегледани и оценени.

Легенда:

C - Checked

NC – Not checked

S - Satisfactory

NS - Not satisfactory

NA - Not applicable