|  | ЧЕК-ЛИСТ / checklist (Търговски въздушен транспорт) (Commercial Air Transport) МОДУЛ / Module саT / POL (AEROPLANES) |  | № CAT_POLA- |  |
| --- | --- | --- | --- | --- |
| Оператор/Организация: Operator /Organisation: | Инспектор / Inspector (име, фамилия / name, surname) | Отдел: Department: | САО №: AOC No: |  |
|  |  |  | Дата: Date: |  |
| Обхват на инспекцията/одита: Scope of Audit: | Annex IV Part CAT - COMMERCIAL AIR TRANSPORT OPERATIONS Subpart C - AIRCRAFT PERFORMANCE AND OPERATING LIMITATIONS |  |  |  |

Забележка: В полето „№“ след абревиатурата се поставя номера на заповедта за комисия.(Пример - № ORO_PR-45-01-00/01.01.2016)

| Забележка Note | ОСНОВАНИЕ References | ВЪПРОСИ, НАСОКИ ЗА ИНСПЕКТОРА Questions, guidelines to inspectors | СЪОТВЕТСТВИЕ Compliance |  |  |  |  | ДОКАЗАТЕЛСТВА/ ЗАБЕЛЕЖКА Evidence/Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | NC | S | NS | NA |  |
| 1. | Make sure you have the latest revision or/and easy access rules of the Regulation (EU) No 965/2012, either in electronic or hard copy, readily available. |  |  |  |  |  |  |  |
| 2. | Коя ревизия на Easy Access Rules for Air Operations е използвал инспектора при извършване на одита/инспекцията? |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | (Please insert reference issue/revision and date) |
| 3. | Кое Издание/Ревизия на кое/кои ОМ преглежда инспектора при извършване на одита/инспекцията? |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | (Please insert reference issue/revision and date) |
| 4. | Моля, при прегледа на настоящият чек лист и след установяване на пълно съответствие на процедурите, тези които подлежат на одобрение, да бъдат изрично упоменати на този ред (виж примера*), в обобщения доклад и вписано в списъка на одобренията на оператора. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | (Моля отбележете процедурите, които подлежат на одобрение) *Пример: подлежи на одобрение - CAT.POL.A.245 Approval of steep approach operations |

| № | ОСНОВАНИЕ References | ВЪПРОСИ, НАСОКИ ЗА ИНСПЕКТОРА Questions, guidelines to inspectors | СЪОТВЕТСТВИЕ Compliance |  |  |  |  |  |  |  |  | ДОКАЗАТЕЛСТВА/ ЗАБЕЛЕЖКА Evidence/Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C |  | NC | S | NS |  | NA | I | D |  |
| 1. | CAT.POL.A.100 | Performance classes |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The aeroplane shall be operated in accordance with the applicable performance class requirements. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Where full compliance with the applicable requirements of this Section cannot be shown due to specific design characteristics, the operator shall apply approved performance standards that ensure a level of safety equivalent to that of the appropriate chapter. |  |  |  |  |  |  |  |  |  |  |
| 2. | CAT.POL.A.105 | General |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The mass of the aeroplane: (1) at the start of the take-off; or (2) in the event of in-flight replanning, at the point from which the revised operational flight plan applies,shall not be greater than the mass at which the requirements of the appropriate chapter can be complied with for the flight to be undertaken. Allowance may be made for expected reductions in mass as the flight proceeds and for fuel jettisoning. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) The approved performance data contained in the AFM shall be used to determine compliance with the requirements of the appropriate chapter, supplemented as necessary with other data as prescribed in the relevant chapter. The operator shall specify other data in the operations manual. When applying the factors prescribed in the appropriate chapter, account may be taken of any operational factors already incorporated in the AFM performance data to avoid double application of factors. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) Due account shall be taken of aeroplane configuration, environmental conditions and the operation of systems that have an adverse effect on performance. |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) The operator shall take account of charting accuracy when assessing the take-off requirements of the applicable chapters. |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCE CLASS A |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. | CAT.POL.A.200 | General |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The approved performance data in the AFM shall be supplemented as necessary with other data if the approved performance data in the AFM is insufficient in respect of items such as: (1) accounting for reasonably expected adverse operating conditions such as take-off and landing on contaminated runways; and (2) consideration of engine failure in all flight phases. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) For wet and contaminated runways, performance data determined in accordance with applicable standards on certification of large aeroplanes or equivalent shall be used. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) The use of other data referred to in (a) and equivalent requirements referred to in (b) shall be specified in the operations manual. |  |  |  |  |  |  |  |  |  |  |
| 4. | AMC1 CAT.POL.A.200 | Wet and contaminated runway data The determination of take-off performance data for wet and contaminated runways should be based on the reported runway surface condition in terms of contaminant and depth. The determination of landing performance data should be based on information provided in the OM on the reported RWYCC. The RWYCC is determined by the aerodrome operator using the RCAM and associated procedures defined in Annex V (Part-ADR.OPS) to Regulation (EU) No 139/2014. The RWYCC is reported through an RCR in the SNOWTAM format in accordance with ICAO Annex 15. |  |  |  |  |  |  |  |  |  |  |
| 5. | CAT.POL.A.205 | Take-off |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The take-off mass shall not exceed the maximum take-off mass specified in the AFM for the pressure altitude and the ambient temperature at the aerodrome of departure. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) The following requirements shall be met when determining the maximum permitted take-off mass: (1) the accelerate-stop distance shall not exceed the accelerate-stop distance available (ASDA); (2) the take-off distance shall not exceed the take-off distance available, with a clearway distance not exceeding half of the take-off run available (TORA); (3) the take-off run shall not exceed the TORA; (4) a single value of V1 shall be used for the rejected and continued take-off; and (5) on a wet or contaminated runway, the take-off mass shall not exceed that permitted for a take-off on a dry runway under the same conditions. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) When showing compliance with (b), the following shall be taken into account: (1) the pressure altitude at the aerodrome; (2) the ambient temperature at the aerodrome; (3) the runway surface condition and the type of runway surface; (4) the runway slope in the direction of take-off; (5) not more than 50 % of the reported headwind component or not less than 150 % of the reported tailwind component; and (6) the loss, if any, of runway length due to alignment of the aeroplane prior to take-off. |  |  |  |  |  |  |  |  |  |  |
| 6. | AMC1 CAT.POL.A.205 | Loss of runway length due to alignment |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The length of the runway that is declared for the calculation of take-off distance available (TODA), accelerate-stop distance available (ASDA) and take-off run available (TORA) does not account for line-up of the aeroplane in the direction of take-off on the runway in use. This alignment distance depends on the aeroplane geometry and access possibility to the runway in use. Accountability is usually required for a 90°-taxiway entry to the runway and 180°-turnaround on the runway. There are two distances to be considered: |  |  |  |  |  |  |  |  |  |  |
|  |  | (1) the minimum distance of the main wheels from the start of the runway for determining TODA and TORA,’L’; and |  |  |  |  |  |  |  |  |  |  |
|  |  | (2) the minimum distance of the most forward wheel(s) from the start of the runway for determining ASDA,’N’. |  |  |  |  |  |  |  |  |  |  |
|  |  | Figure 1 Line-up of the aeroplane in the direction of take-off — L and N |  |  |  |  |  |  |  |  |  |  |
|  |  | Where the aeroplane manufacturer does not provide the appropriate data, the calculation method given in (b) should be used to determine the alignment distance. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Alignment distance calculation |  |  |  |  |  |  |  |  |  |  |
|  |  | The distances mentioned in (a)(1) and (a)(2) are: |  |  |  |  |  |  |  |  |  |  |
|  |  | 90° entry; 180° turnaround. L=; RM + X; RN + Y. N=; RM + X + WB; RN + Y + WB. |
|  |  | where: RN = A + WN = WB/cos(90°-α) + WN RM = B + WM = WB tan(90°-α) + WM X = safety distance of outer main wheel during turn to the edge of the runway Y = safety distance of outer nose wheel during turn to the edge of the runway Note: Minimum edge safety distances for X and Y are specified in FAA AC 150/5300-13 and ICAO Annex 14, 3.8.3 RN = radius of turn of outer nose wheel RM = radius of turn of outer main wheel WN = distance from aeroplane centre-line to outer nose wheel WM = distance from aeroplane centre-line to outer main wheel WB = wheel base α = steering angle. |  |  |  |  |  |  |  |  |  |  |
| 7. | GM1 CAT.POL.A.205 | Runway surface condition (a) Operation on runways contaminated with water, slush, snow or ice implies uncertainties with regard to runway friction and contaminant drag and, therefore, to the achievable performance and control of the aeroplane during take-off, since the actual conditions may not completely match the assumptions on which the performance information is based. In the case of a contaminated runway, the first option for the commander is to wait until the runway is cleared. If this is impracticable, he/she may consider a take-off, provided that he/she has applied the applicable performance adjustments, and any further safety measures he/she considers justified under the prevailing conditions. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) An adequate overall level of safety will only be maintained if operations in accordance with AMC 25.1591 or equivalent are limited to rare occasions. Where the frequency of such operations on contaminated runways is not limited to rare occasions, the operator should provide additional measures ensuring an equivalent level of safety. Such measures could include special crew training, additional distance factoring and more restrictive wind limitations. |  |  |  |  |  |  |  |  |  |  |
| 8. | CAT.POL.A.210 | Take-off obstacle clearance |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The net take-off flight path shall be determined in such a way that the aeroplane clears all obstacles by a vertical distance of at least 35 ft or by a horizontal distance of at least 90 m plus 0,125 × D, where D is the horizontal distance the aeroplane has travelled from the end of the take-off distance available (TODA) or the end of the take-off distance if a turn is scheduled before the end of the TODA. For aeroplanes with a wingspan of less than 60 m, a horizontal obstacle clearance of half the aeroplane wingspan plus 60 m, plus 0,125 × D may be used. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) When showing compliance with (a): (1) The following items shall be taken into account: (i) the mass of the aeroplane at the commencement of the take-off run; (ii) the pressure altitude at the aerodrome; (iii) the ambient temperature at the aerodrome; and (iv) not more than 50 % of the reported headwind component or not less than 150 % of the reported tailwind component. (2) Track changes shall not be allowed up to the point at which the net take-off flight path has achieved a height equal to one half the wingspan but not less than 50 ft above the elevation of the end of the TORA. Thereafter, up to a height of 400 ft it is assumed that the aeroplane is banked by no more than 15°. Above 400 ft height bank angles greater than 15°, but not more than 25° may be scheduled. (3) Any part of the net take-off flight path in which the aeroplane is banked by more than 15° shall clear all obstacles within the horizontal distances specified in (a), (b)(6) and (b)(7) by a vertical distance of at least 50 ft. (4) Operations that apply increased bank angles of not more than 20° between 200 ft and 400 ft, or not more than 30° above 400 ft, shall be carried out in accordance with CAT.POL.A.240. |  |  |  |  |  |  |  |  |  |  |
|  |  | (5) Adequate allowance shall be made for the effect of bank angle on operating speeds and flight path including the distance increments resulting from increased operating speeds. (6) For cases where the intended flight path does not require track changes of more than 15°, the operator does not need to consider those obstacles that have a lateral distance greater than: (i) 300 m, if the pilot is able to maintain the required navigational accuracy through the obstacle accountability area; or (ii) 600 m, for flights under all other conditions. (7) For cases where the intended flight path requires track changes of more than 15°, the operator does not need to consider those obstacles that have a lateral distance greater than: (i) 600 m, if the pilot is able to maintain the required navigational accuracy through the obstacle accountability area; or (ii) 900 m, for flights under all other conditions. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) The operator shall establish contingency procedures to satisfy the requirements in (a) and (b) and to provide a safe route, avoiding obstacles, to enable the aeroplane to either comply with the en-route requirements of CAT.POL.A.215, or land at either the aerodrome of departure or at a take-off alternate aerodrome. |  |  |  |  |  |  |  |  |  |  |
| 9. | AMC1 CAT.POL.A.210 | Take-off obstacle clearance (a) In accordance with the definitions used in preparing the take-off distance and take-off flight path data provided in the AFM: (1) The net take-off flight path is considered to begin at a height of 35 ft above the runway or clearway at the end of the take-off distance determined for the aeroplane in accordance with (b) below. (2) The take-off distance is the longest of the following distances: (i) 115 % of the distance with all engines operating from the start of the take-off to the point at which the aeroplane is 35 ft above the runway or clearway; (ii) the distance from the start of the take-off to the point at which the aeroplane is 35 ft above the runway or clearway assuming failure of the critical engine occurs at the point corresponding to the decision speed (V1) for a dry runway; or (iii) if the runway is wet or contaminated, the distance from the start of the take-off to the point at which the aeroplane is 15 ft above the runway or clearway assuming failure of the critical engine occurs at the point corresponding to the decision speed (V1) for a wet or contaminated runway. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) The net take-off flight path, determined from the data provided in the AFM in accordance with (a)(1) and (a)(2), should clear all relevant obstacles by a vertical distance of 35 ft. When taking off on a wet or contaminated runway and an engine failure occurs at the point corresponding to the decision speed (V1) for a wet or contaminated runway, this implies that the aeroplane can initially be as much as 20 ft below the net take-off flight path in accordance with (a) and, therefore, may clear close-in obstacles by only 15 ft. When taking off on wet or contaminated runways, the operator should exercise special care with respect to obstacle assessment, especially if a take-off is obstacle-limited and the obstacle density is high. |  |  |  |  |  |  |  |  |  |  |
| 10. | AMC2 CAT.POL.A.210 | Effect of bank angles (a) The AFM generally provides a climb gradient decrement for a 15° bank turn. For bank angles of less than 15°, a proportionate amount should be applied, unless the manufacturer or AFM has provided other data. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Unless otherwise specified in the AFM or other performance or operating manuals from the manufacturer, acceptable adjustments to assure adequate stall margins and gradient corrections are provided by the following table: (See relevant Table 1 in the Regulation (EU) No 965/2012) |  |  |  |  |  |  |  |  |  |  |
|  |  | Table 1 Effect of bank angles Bank; Speed; Gradient correction. 15°; V2; 1 x AFM 15° gradient loss. 20°; V2 + 5 kt; 2 x AFM 15° gradient loss. 25°; V2 + 10 kt; 3 x AFM 15° gradient loss. |
| 11. | AMC3 CAT.POL.A.210 | Required navigational accuracy |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Navigation systems The obstacle accountability semi-widths of 300 m and 600 m may be used if the navigation system under OEI conditions provides a two standard deviation accuracy of 150 m and 300 m respectively. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Visual course guidance (1) The obstacle accountability semi-widths of 300 m and 600 m may be used where navigational accuracy is ensured at all relevant points on the flight path by use of external references. These references may be considered visible from the flight crew compartment if they are situated more than 45° either side of the intended track and with a depression of not greater than 20° from the horizontal. (2) For visual course guidance navigation, the operator should ensure that the weather conditions prevailing at the time of operation, including ceiling and visibility, are such that the obstacle and/or ground reference points can be seen and identified. The operations manual should specify, for the aerodrome(s) concerned, the minimum weather conditions which enable the flight crew to continuously determine and maintain the correct flight path with respect to ground reference points, so as to provide a safe clearance with respect to obstructions and terrain as follows: (i) the procedure should be well-defined with respect to ground reference points so that the track to be flown can be analysed for obstacle clearance requirements; (ii) the procedure should be within the capabilities of the aeroplane with respect to forward speed, bank angle and wind effects; (iii) a written and/or pictorial description of the procedure should be provided for crew use; and (iv) the limiting environmental conditions (such as wind, the lowest cloud base, ceiling, visibility, day/night, ambient lighting, obstruction lighting) should be specified. |  |  |  |  |  |  |  |  |  |  |
| 12. | GM1 CAT.POL.A.210 | Contingency procedures for obstacles clearances If compliance with CAT.POL.A.210 is based on an engine failure route that differs from the all engine departure route or SID normal departure, a ‘deviation point’ can be identified where the engine failure route deviates from the normal departure route. Adequate obstacle clearance along the normal departure route with failure of the critical engine at the deviation point will normally be available. However, in certain situations the obstacle clearance along the normal departure route may be marginal and should be checked to ensure that, in case of an engine failure after the deviation point, a flight can safely proceed along the normal departure route. |  |  |  |  |  |  |  |  |  |  |
| 13. | CAT.POL.A.215 | En-route – one-engine-inoperative (OEI) |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The OEI en-route net flight path data shown in the AFM, appropriate to the meteorological conditions expected for the flight, shall allow demonstration of compliance with (b) or (c) at all points along the route. The net flight path shall have a positive gradient at 1 500 ft above the aerodrome where the landing is assumed to be made after engine failure. In meteorological conditions requiring the operation of ice protection systems, the effect of their use on the net flight path shall be taken into account. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) The gradient of the net flight path shall be positive at least 1 000 ft above all terrain and obstructions along the route within 9,3 km (5 NM) on either side of the intended track. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) The net flight path shall permit the aeroplane to continue flight from the cruising altitude to an aerodrome where a landing can be made in accordance with CAT.POL.A.225 or CAT.POL.A.230, as appropriate. The net flight path shall clear vertically, by at least 2 000 ft, all terrain and obstructions along the route within 9,3 km (5 NM) on either side of the intended track in accordance with the following: (1) the engine is assumed to fail at the most critical point along the route; (2) account is taken of the effects of winds on the flight path; ‘(3) fuel jettisoning is permitted to an extent consistent with reaching the aerodrome where the aeroplane is assumed to land after engine failure with the required fuel reserves in accordance with point CAT.OP. MPA.181, appropriate for an alternate aerodrome, if a safe procedure is used;’ (4) the aerodrome where the aeroplane is assumed to land after engine failure shall meet the following criteria: (i) the performance requirements at the expected landing mass are met; and (ii) weather reports and/or forecasts and field condition reports indicate that a safe landing can be accomplished at the estimated time of landing. (5) if the AFM does not contain en-route net flight path data, the gross OEI en-route flight path shall be reduced by a climb gradient of 1,1 % for two-engined aeroplanes, 1,4 % for three-engined aeroplanes, and 1,6 % for four-engined aeroplanes. |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) The operator shall increase the width margins provided for in points (b) and (c) to 18,5 km (10 NM) if the navigational accuracy does not meet at least navigation specification RNAV 5. |  |  |  |  |  |  |  |  |  |  |
| 14. | AMC1 CAT.POL.A.215 | Route analysis |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The high terrain or obstacle analysis required should be carried out by a detailed analysis of the route. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) A detailed analysis of the route should be made using contour maps of the high terrain and plotting the highest points within the prescribed corridor’s width along the route. The next step is to determine whether it is possible to maintain level flight with OEI 1 000 ft above the highest point of the crossing. If this is not possible, or if the associated weight penalties are unacceptable, a drift down procedure should be worked out, based on engine failure at the most critical point and clearing critical obstacles during the drift down by at least 2 000 ft. The minimum cruise altitude is determined by the intersection of the two drift down paths, taking into account allowances for decision making (see Figure 1). This method is time-consuming and requires the availability of detailed terrain maps. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) Alternatively, the published minimum flight altitudes (MEA or minimum off-route altitude (MORA)) should be used for determining whether OEI level flight is feasible at the minimum flight altitude, or if it is necessary to use the published minimum flight altitudes as the basis for the drift down construction (see Figure 1). This procedure avoids a detailed high terrain contour analysis, but could be more penalising than taking the actual terrain profile into account as in (b). |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) In order to comply with CAT.POL.A.215 (c), one means of compliance is the use of MORA and, with CAT.POL.A.215 (d), MEA provided that the aeroplane meets the navigational equipment standard assumed in the definition of MEA. |  |  |  |  |  |  |  |  |  |  |
|  |  | Figure 1 Intersection of the two drift down paths Note: MEA or MORA normally provide the required 2 000 ft obstacle clearance for drift down. However, at and below 6 000 ft altitude, MEA and MORA cannot be used directly as only 1 000 ft clearance is ensured. |  |  |  |  |  |  |  |  |  |  |
| 15. | CAT.POL.A.220 | En-route – aeroplanes with three or more engines, two engines inoperative |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) An aeroplane that has three or more engines shall not be away from an aerodrome at which the requirements of points CAT.POL.A.230 or CAT.POL.A.235(a) for the expected landing mass are met accordingly, at any point along the intended track for more than 90 minutes, with all engines operating at cruising power or thrust, as appropriate, at standard temperature in still air, unless points (b) to (f) of this point are complied with. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) The two-engines-inoperative en-route net flight path data shall allow the aeroplane to continue the flight, in the expected meteorological conditions, from the point where two engines are assumed to fail simultaneously to an aerodrome at which it is possible to land and come to a complete stop when using the prescribed procedure for a landing with two engines inoperative. The en-route net flight path shall clear vertically, by at least 2 000 ft, all terrain and obstructions along the route within 9,3 km (5 NM) on either side of the intended track. At altitudes and in meteorological conditions that require ice protection systems to be operable, the effect of their use on the en-route net flight path data shall be taken into account. If the navigational accuracy does not meet at least navigation specification RNAV 5, the operator shall increase the prescribed width margin provided for in the second sentence to 18,5 km (10 NM). |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) The two engines shall be assumed to fail at the most critical point of that portion of the route where the aeroplane is operated for more than 90 minutes, with all engines operating at cruising power or thrust, as appropriate, at standard temperature in still air, away from the aerodrome referred to in point (a). |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) The net flight path shall have a positive gradient at 1 500 ft above the aerodrome where the landing is assumed to be made after the failure of two engines. |  |  |  |  |  |  |  |  |  |  |
|  |  | (e) Fuel jettisoning shall be permitted to an extent consistent with reaching the aerodrome with the required fuel reserves referred to in point (f), if a safe procedure is used. |  |  |  |  |  |  |  |  |  |  |
|  |  | (f) The expected mass of the aeroplane at the point where the two engines are assumed to fail shall not be less than that which would include sufficient fuel/energy to proceed to an aerodrome where the landing is assumed to be made, and to arrive there at an altitude of at least 1 500 ft (450 m) directly over the landing area, and thereafter, to fly for 15 minutes at cruising power or thrust, as appropriate. |  |  |  |  |  |  |  |  |  |  |
| 16. | CAT.POL.A.225 | Landing – destination and alternate aerodromes (a) The landing mass of the aeroplane determined in accordance with CAT.POL.A.105(a) shall not exceed the maximum landing mass specified for the altitude and the ambient temperature expected for the estimated time of landing at the destination aerodrome and alternate aerodrome. |  |  |  |  |  |  |  |  |  |  |
| 17. | AMC1 CAT.POL.A.225 | Altitude measuring The operator should use either pressure altitude or geometric altitude for its operation and this should be reflected in the operations manual. |  |  |  |  |  |  |  |  |  |  |
| 18. | AMC2 CAT.POL.A.225 | MISSED APPROACH (a) For instrument approaches with a missed approach climb gradient greater than 2.5 %, the operator should verify that the expected landing mass of the aeroplane allows for a missed approach with a climb gradient equal to or greater than the applicable missed approach gradient in the OEI missed approach configuration and at the associated speed. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) For instrument approaches with DH below 200 ft, the operator should verify that the expected landing mass of the aeroplane allows a missed approach gradient of climb, with the critical engine failed and with the speed and configuration used for a missed approach of at least 2.5 %, or the published gradient, whichever is greater. |  |  |  |  |  |  |  |  |  |  |
| 19. | GM1 CAT.POL.A.225 | Missed approach gradient |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Where an aeroplane cannot achieve the missed approach gradient specified in AMC2 CAT.POL.A.225, when operating at or near maximum certificated landing mass and in engine-out conditions, the operator has the opportunity to propose an alternative means of compliance to the competent authority demonstrating that a missed approach can be executed safely taking into account appropriate mitigating measures. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) The proposal for an alternative means of compliance may involve the following: (1) considerations to mass, altitude and temperature limitations and wind for the missed approach; (2) a proposal to increase the DA/H or MDA/H; and (3) a contingency procedure ensuring a safe route and avoiding obstacles. |  |  |  |  |  |  |  |  |  |  |
| 20. | CAT.POL.A.230 | Landing – dry runways |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) The landing mass of the aeroplane determined in accordance with point CAT.POL.A.105(a) for the estimated time of landing at the destination aerodrome and at any alternate aerodrome shall allow a full-stop landing from 50 ft above the threshold: (1) for turbojet-powered aeroplanes, within 60 % of the landing distance available (LDA); (2) for turbopropeller-powered aeroplanes, within 70 % of the LDA; (3) by way of derogation from points (a)(1) and (a)(2), for aeroplanes that are approved for reduced landing distance operations under point CAT.POL.A.255, within 80 % of the LDA. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) For steep approach operations, the operator shall use the landing distance data factored in accordance with point (a)(1) or (a)(2), as applicable, based on a screen height of less than 60 ft, but not less than 35 ft, and shall comply with point CAT.POL.A.245. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) For short landing operations, the operator shall use the landing distance data factored in accordance with point (a)(1) or (a)(2), as applicable, and shall comply with point CAT.POL.A.250. |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) When determining the landing mass, the operator shall take into account the following: (1) not more than 50 % of the headwind component or not less than 150 % of the tailwind component; (2) corrections as provided in the AFM. |  |  |  |  |  |  |  |  |  |  |
|  |  | ‘(e) For dispatching the aeroplane, the aeroplane shall: (1) land on the most favourable runway, in still air; and (2) land on the runway most likely to be assigned, considering the probable wind speed and direction, the ground-handling characteristics of the aeroplane and other conditions such as landing aids and terrain.’; |  |  |  |  |  |  |  |  |  |  |
|  |  | (f) If the operator is unable to comply with point (e)(2) for the destination aerodrome, the aeroplane shall only be dispatched if an alternate aerodrome is designated that allows full compliance with one of the following: (1) points (a) to (d), if the runway at the estimated time of arrival is dry; (2) points CAT.POL.A.235(a) to (d), if the runway at the estimated time of arrival is wet or contaminated. |  |  |  |  |  |  |  |  |  |  |
| 21. | AMC1 CAT.POL.A.230 | Factoring of automatic landing distance performance data In those cases where the landing requires the use of an automatic landing system, and the distance published in the AFM includes safety margins equivalent to those contained in CAT.POL.A.230(a)(1) and CAT.POL.A.230(a)(2), the landing mass of the aeroplane should be the lesser of: (a) the landing mass determined in accordance with CAT.POL.A.230(a)(1) and CAT.POL.A.230(a)(2); or (b) the landing mass determined for the automatic landing distance for the appropriate surface condition, as given in the AFM or equivalent document. Increments due to system features such as beam location or elevations, or procedures such as use of overspeed, should also be included. |  |  |  |  |  |  |  |  |  |  |
| 22. | AMC2 CAT.POL.A.230 | Factoring of landing distance performance data when using a head-up display (hud) or an equivalent display with flare cue In those cases where the landing requires the use of a HUD or an equivalent display with flare cue, and the landing distance published in the AFM includes safety factors, the landing mass of the aeroplane should be the lesser of: (a) the landing mass determined in accordance with CAT.POL.A.230(a)(1); or (b) the landing mass determined, when using a HUD or an equivalent display with flare cue for the appropriate surface condition, as given in the AFM or equivalent document. |  |  |  |  |  |  |  |  |  |  |
| 23. | GM1 CAT.POL.A.230 | Landing mass CAT.POL.A.230 establishes two considerations in determining the maximum permissible landing mass at the destination and alternate aerodromes: |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Firstly, the aeroplane mass will be such that on arrival the aeroplane can be landed within 60 %, 70 %, or 80 % (as applicable) of the landing distance available (LDA) on the most favourable (normally the longest) runway in still air. Regardless of the wind conditions, the maximum landing mass for an aerodrome/aeroplane configuration at a particular aerodrome cannot be exceeded |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Secondly, consideration should be given to anticipated conditions and circumstances. The expected wind, or ATC and noise abatement procedures, may indicate the use of a different runway. These factors may result in a lower landing mass than that permitted under (a), in which case dispatch should be based on this lesser mass. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) The expected wind referred to in (b) is the wind expected to exist at the time of arrival. |  |  |  |  |  |  |  |  |  |  |
| 24. | GM1 CAT.POL.A.230 & CAT.POL.A.235 | Landing — dry runways & Landing — wet and contaminated runways – see tables in Regulation 965. |  |  |  |  |  |  |  |  |  |  |
|  |  | Workflow of the landing distance assessment at the time of dispatch — general |  |  |  |  |  |  |  |  |  |  |
|  |  | Workflow of the landing distance assessment at the time of dispatch — runway suitability check |  |  |  |  |  |  |  |  |  |  |
|  |  | Workflow of the landing distance assessment at the time of dispatch — dry runways |  |  |  |  |  |  |  |  |  |  |
|  |  | Workflow of the landing distance assessment at the time of dispatch — wet runways |  |  |  |  |  |  |  |  |  |  |
|  |  | Workflow of the landing distance assessment at the time of dispatch — contaminated runways |  |  |  |  |  |  |  |  |  |  |
| 25. | GM2 CAT.POL.A.230 & CAT.POL.A.235 | Landing distances and corrective factors |  |  |  |  |  |  |  |  |  |  |
|  |  | The AFM provides performance data for landing distance under conditions defined in the applicable certification standards. This distance, commonly referred to as the actual landing distance (ALD), is the distance from the position on the runway of the screen height to the point where the aeroplane comes to a full stop on a dry runway. The determination of the ALD is based on the assumption that the landing is performed in accordance with the conditions and the procedures set out in the AFM on the basis of the applicable certification standards. As a matter of fact, any particular landing may be different from the landing technique that is assumed in the AFM for certification purposes. The aircraft may approach the runway faster and/or higher than assumed; the aircraft may touch down further along the runway than the optimum point; the actual winds and other weather factors may be different from those assumed in the calculation of the ALD; and maximum braking may not be always achievable. For this reason, the LDA is required by CAT.POL.A.230 and CAT.POL.A.235 to be longer than the ALD. The margins by which the LDA shall exceed the ALD on dry runways, in accordance with CAT.POL.A.230, are shown in the following Table 1. |  |  |  |  |  |  |  |  |  |  |
|  |  | Table 1: Corrective factors for dry runways Aeroplane category; Required margin (dry runway); Resulting factor (dry runway). Turbojet-powered aeroplanes; ALD < 60 % of the LDA; LDA = at least 1.67 x ALD. Turbopropeller-powered aeroplanes; ALD < 70 % of the LDA; LDA = at least 1.43 x ALD. Aeroplanes approved under CAT.POL.A.255; ALD < 80 % of the LDA; LDA = at least 1.25 x ALD. |
|  |  | If the runway is wet and the AFM does not provide specific performance data for dispatch on wet runways, a further increase of 15 % of the landing distance on dry runways has to be applied, in accordance with CAT.POL.A.235, as shown in the following Table 2. |  |  |  |  |  |  |  |  |  |  |
|  |  | Table 2: Corrective factors for wet runways Aeroplane category; Resulting factor (dry runway). Turbojet-powered aeroplanes; LDA = at least 1.15 x 1.67 x ALD = 1.92 x ALD. Turbopropeller-powered aeroplanes; LDA = at least 1.15 x 1.43 x ALD = 1.64 x ALD. Aeroplanes approved under CAT.POL.A.255; LDA = at least 1.15 x 1.25 X ALD = 1.44 x ALD. |
|  |  | However, for aeroplanes that are approved under CAT.POL.A.255, when landing on wet runways, CAT.POL.A.255 further requires the flight crew to apply the longer of the landing distance resulting from the above table and the landing distance resulting from the application of CAT.OP.MPA.303(a) or (b) as applicable. If performance information for the assessment of LDTA is not available as per CAT.OP.MPA.303(b)(2), the required landing distance on wet runways should be at least: 1.15 x 1.67 x ALD for turbojet-powered aircraft and 1.15 x 1.43 x ALD for turbopropeller-powered aircraft. |  |  |  |  |  |  |  |  |  |  |
| 26. | GM1 CAT.POL.A.230(a) | Alternate aerodromes The alternate aerodromes for which the landing mass is required to be determined in accordance with CAT.POL.A.230 are: (a) destination alternate aerodromes; (b) fuel ERA aerodromes; and (c) re-dispatch or re-clearance aerodromes. |  |  |  |  |  |  |  |  |  |  |
| 27. | GM1 CAT.POL.A.230(d)(2) | AFM landing performance corrections Landing performance data is provided in the AFM at least for the certified range of pressure altitudes. AFM data may include other influence parameters such as, but not limited to, runway slope and temperature. The effect of speed increments over threshold should also be accounted for when these increments are required by the applicable AFM procedures, such as autoland or steep approach. |  |  |  |  |  |  |  |  |  |  |
| 28. | CAT.POL.A.235 | Landing – wet and contaminated runways |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) When the appropriate weather reports or forecasts, or both, indicate that the runway at the estimated time of arrival may be wet, the LDA shall be one of the following distances: (1) a landing distance provided in the AFM for use on wet runways at time of dispatch, but not less than that required by point CAT.POL.A.230(a)(1) or (a)(2), as applicable; (2) if a landing distance is not provided in the AFM for use on wet runways at time of dispatch, at least 115 % of the required landing distance, determined in accordance with point CAT.POL.A.230(a)(1) or (a)(2), as applicable; (3) a landing distance shorter than that required by point (a)(2), but not less than that required by point CAT.POL.A.230(a)(1) or (a)(2), as applicable, if the runway has specific friction-improving characteristics and the AFM includes specific additional information for landing distance on that runway type; (4) by way of derogation from points (a)(1), (a)(2) and (a)(3), for aeroplanes that are approved for reduced landing distance operations under point CAT.POL.A.255, the landing distance determined in accordance with point CAT.POL.A.255(b)(2)(v)(B). |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) When the appropriate weather reports or forecasts indicate that the runway at the estimated time of arrival may be contaminated, the LDA shall be one of the following distances: (1) at least the landing distance determined in accordance with point (a), or at least 115 % of the landing distance determined in accordance with approved contaminated landing distance data or equivalent, whichever is greater; (2) on specially prepared winter runways, a landing distance shorter than that required by point (b)(1), but not less than that required by point (a), may be used if the AFM includes specific additional information about landing distances on contaminated runways. Such landing distance shall be at least 115 % of the landing distance contained in the AFM. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) By way of derogation from point (b), the increment of 15 % needs not to be applied if it is already included in the approved landing distance data or equivalent. |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) For points (a) and (b), the criteria of points CAT.POL.A.230(b), (c) and (d) shall apply accordingly. |  |  |  |  |  |  |  |  |  |  |
|  |  | ‘(e) For dispatching the aeroplane, the aeroplane shall: (1) land on the most favourable runway, in still air; and (2) land on the runway most likely to be assigned, considering the probable wind speed and direction, the ground-handling characteristics of the aeroplane and other conditions such as landing aids and terrain.’; |  |  |  |  |  |  |  |  |  |  |
|  |  | (f) If the operator is unable to comply with point (e)(1) for a destination aerodrome where the appropriate weather reports or forecasts indicate that the runway at the estimated time of arrival may be contaminated and where a landing depends upon a specific wind component, the aeroplane shall only be dispatched if two alternate aerodromes are designated. |  |  |  |  |  |  |  |  |  |  |
|  |  | (g) If the operator is unable to comply with point (e)(2) for the destination aerodrome where the appropriate weather reports or forecasts indicate that the runway at the estimated time of arrival may be wet or contaminated, the aeroplane shall only be dispatched if an alternate aerodrome is designated. |  |  |  |  |  |  |  |  |  |  |
|  |  | (h) For points (f) and (g), the designated alternate aerodrome or aerodromes shall allow compliance with one of the following: (1) points CAT.POL.A.230(a) to (d), if the runway at the estimated time of arrival is dry; (2) points CAT.POL.A.235(a) to (d), if the runway at the estimated time of arrival is wet or contaminated. |  |  |  |  |  |  |  |  |  |  |
| 29. | AMC1 CAT.POL.A.235 | Factoring of automatic landing distance performance data |  |  |  |  |  |  |  |  |  |  |
|  |  | In those cases where the landing requires the use of an automatic landing system, and the distance published in the AFM includes safety margins equivalent to thse contained in CAT.POL.A.235, the landing mass of the aeroplane should be the lesser of: (a) the landing mass determined in accordance with CAT.POL.A.235; or |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) the landing mass determined for the automatic landing distance for the appropriate surface condition, as given in the AFM or equivalent document. Increments due to system features such as beam location or elevations, or procedures such as use of overspeed, should also be included. |  |  |  |  |  |  |  |  |  |  |
| 30. | AMC2 CAT.POL.A.235 | Factoring of landing distance performance data when a using head-up display (hud) or an equivalent display with flare cue |  |  |  |  |  |  |  |  |  |  |
|  |  | In those cases where the landing requires the use of a HUD or an equivalent display with flare cue, and the landing distance published in the AFM includes safety factors, the landing mass of the aeroplane should be the lesser of: (a) the landing mass determined in accordance with CAT.POL.A.235; or |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) the landing mass determined, when using a HUD or an equivalent display with flare cue for the appropriate surface condition, as given in the AFM or equivalent document. |  |  |  |  |  |  |  |  |  |  |
| 31. | GM1 CAT.POL.A.235(a) and (b) | Dispatch considerations for marginal cases The LDTA required by CAT.OP.MPA.303 may, in some cases, and in particular on wet or contaminated runways, exceed the landing distance considered at the time of dispatch. The requirements for dispatch remain unchanged, however, when the conditions at the time of arrival are expected to be marginal, it is a good practice to carry out at the time of dispatch a preliminary calculation of the LDTA. |  |  |  |  |  |  |  |  |  |  |
| 32. | GM1 CAT.POL.A.235(a)(1) | AFM landing distances for wet runways Specific landing distances provided in the AFM for dispatch on wet runways, unless otherwise indicated, include a safety factor, which renders not necessary the application of the 15 % safety factor used in CAT.POL.A.235(a)(2). This implies that the AFM distance may be presented as factored distance. When the AFM distance is not factored, a safety factor of 15 % should be applied. These distances may be longer or shorter than those resulting from CAT.POL.A.235(a)(2), but when provided, they are intended as a replacement of CAT.POL.A.235(a)(2) and mandatory for use at the time of dispatch. |  |  |  |  |  |  |  |  |  |  |
| 33. | AMC1 CAT.POL.A.235(a)(3) | Runways with friction improving characteristics Materials or construction techniques meant to improve the friction characteristics of a runway may be grooved runways, runways treated with porous friction course (PFC) or other materials or techniques for which the AFM provides specific performance data. Before taking the AFM performance credit for such runways, the operator should verify that the runways intended to be operated on are maintained to the extent necessary to ensure the expected improved friction characteristics. |  |  |  |  |  |  |  |  |  |  |
| 34. | CAT.POL.A.240 | Approval of operations with increased bank angles |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Operations with increased bank angles require prior approval by the competent authority. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) To obtain the approval, the operator shall provide evidence that the following conditions are met: (1) the AFM contains approved data for the required increase of operating speed and data to allow the construction of the flight path considering the increased bank angles and speeds; (2) visual guidance is available for navigation accuracy; (3) weather minima and wind limitations are specified for each runway; and (4) the flight crew has obtained adequate knowledge of the route to be flown and of the procedures to be used in accordance with Subpart FC of Part-ORO. |  |  |  |  |  |  |  |  |  |  |
| 35. | CAT.POL.A.245 | Approval of steep approach operations |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Steep approach operations using glideslope angles of 4,5° or more and with screen heights of less than 60 ft, but not less than 35 ft, require prior approval by the competent authority. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) To obtain the approval, the operator shall provide evidence that the following conditions are met: (1) the AFM states the maximum approved glideslope angle, any other limitations, normal, abnormal or emergency procedures for the steep approach as well as amendments to the field length data when using steep approach criteria; (2) for each aerodrome at which steep approach operations are to be conducted: (i) a suitable glide path reference system comprising at least a visual glide path indicating system shall be available; (ii) weather minima shall be specified; and (iii) the following items shall be taken into consideration: (A) the obstacle situation; (B) the type of glide path reference and runway guidance; (C) the minimum visual reference to be required at decision height (DH) and MDA; (D) available airborne equipment; (E) pilot qualification and special aerodrome familiarisation; (F) AFM limitations and procedures; and (G) missed approach criteria. |  |  |  |  |  |  |  |  |  |  |
| 36. | GM1 CAT.POL.A.245(a) | Screen height For the purpose of steep approach operations, the screen height is the reference height above the runway surface, typically above the runway threshold, from which the landing distance is measured. The screen height is set at 50 ft for normal operations and at another value between 60 ft and 35 ft for steep approach operations. |  |  |  |  |  |  |  |  |  |  |
| 37. | CAT.POL.A.250 | Approval of short landing operations |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Short landing operations require prior approval by the competent authority. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) To obtain the approval, the operator shall provide evidence that the following conditions are met: (1) the distance used for the calculation of the permitted landing mass may consist of the usable length of the declared safe area plus the declared LDA; (2) the State of the aerodrome has determined a public interest and operational necessity for the operation, either due to the remoteness of the aerodrome or to physical limitations relating to extending the runway; (3) the vertical distance between the path of the pilot’s eye and the path of the lowest part of the wheels, with the aeroplane established on the normal glide path, does not exceed 3 m; (4) RVR/VIS minimum shall not be less than 1 500 m and wind limitations are specified in the operations manual; (5) minimum pilot experience, training and special aerodrome familiarisation requirements are specified and met; (6) the crossing height over the beginning of the usable length of the declared safe area is 50 ft; (7) the use of the declared safe area is approved by the State of the aerodrome; (8) the usable length of the declared safe area does not exceed 90 m; |  |  |  |  |  |  |  |  |  |  |
|  |  | (9) the width of the declared safe area is not less than twice the runway width or twice the wing span, whichever is greater, centred on the extended runway centre line; (10) the declared safe area is clear of obstructions or depressions that would endanger an aeroplane undershooting the runway and no mobile object is permitted on the declared safe area while the runway is being used for short landing operations; (11) the slope of the declared safe area does not exceed 5 % upward nor 2 % downward in the direction of landing; (11a) reduced required landing distance operations in accordance with CAT.POL.A.255 are prohibited; and (12) additional conditions, if specified by the competent authority, taking into account aeroplane type characteristics, orographic characteristics in the approach area, available approach aids and missed approach/balked landing considerations. |  |  |  |  |  |  |  |  |  |  |
| 38. | CAT.POL.A.255 | Approval of reduced required landing distance operations |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) An aeroplane operator may conduct landing operations within 80 % of the landing distance available (LDA) if it complies with the following conditions: (1) the airplane has an MOPSC of 19 or less; (2) the airplane has an eligibility statement for reduced required landing distance in the AFM; (3) the airplane is used in non-scheduled on-demand commercial air transport (CAT) operations; (4) the landing mass of the aeroplane allows a full-stop landing within that reduced landing distance; (5) the operator has obtained a prior approval of the competent authority. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) To obtain the approval referred to in point (a)(5), the operator shall provide evidence of either of the following circumstances: (1) that a risk assessment has been conducted to demonstrate that a level of safety equivalent to that intended by point CAT.POL.A.230(a)(1) or (2), as applicable, is achieved; |  |  |  |  |  |  |  |  |  |  |
|  |  | (2) that the following conditions are met: (i) special-approach procedures, such as steep approaches, planned screen heights higher than 60 ft or lower than 35 ft, low-visibility operations, approaches outside stabilised approach criteria approved under point CAT.OP.MPA.115(a), are prohibited; (ii) short landing operations in accordance with point CAT.POL.A.250 are prohibited; (iii) landing on contaminated runways is prohibited; (iv) an adequate training, checking and monitoring process for the flight crew is established; (v) an aerodrome landing analysis programme (ALAP) is established by the operator to ensure that the following conditions are met: (A) no tailwind is forecast at the expected time of arrival; (B) if the runway is forecast to be wet at the expected time of arrival, the landing distance at dispatch shall either be determined in accordance with point CAT.OP.MPA.303(a) or (b) as applicable, or shall be 115 % of the landing distance determined for dry runways, whichever is longer; (C) no forecast contaminated runway conditions exist at the expected time of arrival; (D) no forecast adverse weather conditions exist at the expected time of arrival; (vi) all the equipment that affects landing performance is operative before commencing the flight; (vii) the flight crew is composed of at least two qualified and trained pilots that have recency in reduced required landing distance operations; (viii) based on the prevailing conditions for the intended flight, the commander shall make the final decision to conduct reduced required landing distance operations and may decide not to do so when he or she considers that to be in the interest of safety; (ix) additional aerodrome conditions, if specified by the competent authority that has certified the aerodrome, taking into account orographic characteristics of the approach area, available approach aids, missed-approach and balked-landing considerations. |  |  |  |  |  |  |  |  |  |  |
| 39. | GM1 CAT.POL.A.255(a)(2) | AEROPLANE ELIGIBILITY |  |  |  |  |  |  |  |  |  |  |
|  |  | The factors required by CAT.POL.A.230(a)(1) or (a)(2), as applicable, provide an operational safety margin to take into account landing distance operational variability in normal operations compared to the conditions and procedures set out to determine the actual landing distances during the certification of the aeroplane. The reduction of this margin, allowed when operating with reduced required landing distance, is based on a set of mitigating conditions required by CAT.POL.A.255. |  |  |  |  |  |  |  |  |  |  |
|  |  | However, if the factors required by CAT.POL.A.230(a)(1) or (a)(2), as applicable, have been used during the certification of the aeroplane to demonstrate compliance with certification standards such as, but not limited to, CS 25.1309 or equivalent, the aeroplane is not eligible for a reduction of the margin provided by those factors. |  |  |  |  |  |  |  |  |  |  |
|  |  | Furthermore, certification methods offer different options for the determination of the air distance portion of the landing distance in terms of assumption that can be made for parameters such as, but not limited to, glide path angle and sink rate at touchdown. The assumptions made during the certification of the aeroplane may increase the landing distance operational variability in normal operations. The effect of parameters such as temperature or runway slope, when these were not considered during certification, may as well increase the landing distances achievable in normal operations. Overall, the set of assumptions made during the certification of the aeroplane may not be always compatible with the operational safety margin reduction allowed in reduced required landing distance operations under CAT.POL.A.255. |  |  |  |  |  |  |  |  |  |  |
|  |  | Whether the factors required by CAT.POL.A.230(a)(1) or (a)(2), as applicable, have been used to demonstrate compliance with certification standards, or the set of assumptions made to determine actual landing distances during the certification of the aeroplane are compatible with reduced landing distance operations, may be only declared by the aeroplane manufacturer or by the TC/STC holder. [applicable from 12 August 2021] |  |  |  |  |  |  |  |  |  |  |
| 40. | GM1 CAT.POL.A.255(a)(3) | Non-scheduled on-demand commercial air transport (CAT) operations For the purpose of reduced required landing distance operations, non-scheduled on-demand CAT operations are those CAT operations conducted upon request of the customer. Non-scheduled on-demand CAT operations eligible for reduced required landing distance operations do not include holiday charters, i.e. charter flights that are part of a holiday travel package. [applicable from 12 August 2021] |  |  |  |  |  |  |  |  |  |  |
| 41. | AMC1 CAT.POL.A.255(b)(1) | Equivalent level of safety |  |  |  |  |  |  |  |  |  |  |
|  |  | A level of safety equivalent to that intended by CAT.POL.A.230(a)(1) or CAT.POL.A.230(a)(2), as applicable, may be achieved when conducting reduced required landing distance operations if mitigating measures are established and implemented. Such measures should address flight crew, aircraft characteristics and performance, aerodromes and operations. It is, however, essential that all conditions established are adhered to as it is the combination of said conditions that achieves the intended level of safety. The operator should in fact also consider the interrelation of the various mitigating measures. |  |  |  |  |  |  |  |  |  |  |
|  |  | The mitigating measures may be determined by the operator by using a risk assessment or by fulfilling all the conditions established under CAT.POL.A.255(b)(2). An operator willing to establish a set of conditions different from those under CAT.POL.A.255(b)(2) needs to demonstrate to the competent authority the equivalent level of safety through a risk assessment. |  |  |  |  |  |  |  |  |  |  |
|  |  | The risk assessment required by CAT.POL.A.255(b)(1) should include at least the following elements: (a) flight crew qualification in terms of training, checking and recency; (b) flight crew composition; (c) runway surface conditions; (d) dispatch criteria; (e) weather conditions and limitations, including crosswind; (f) aerodrome characteristics, including available approach guidance; (g) aeroplane characteristics and limitations; (h) aeroplane equipment and systems affecting landing performance; (i) aeroplane performance data; (j) operating procedures and operating minima; and (k) analysis of operators’s performance and occurrence reports related to unstable approaches and long landings. |  |  |  |  |  |  |  |  |  |  |
|  |  | The competent authority may require other mitigating measures in addition to those proposed by the operator. |  |  |  |  |  |  |  |  |  |  |
| 42. | AMC1 CAT.POL.A.255(b)(2)(iv) | Approval of reduced required landing distance operations |  |  |  |  |  |  |  |  |  |  |
|  |  | General (a) The operator should ensure that flight crew training programmes for reduced required landing distance operations include ground training, flight simulation training device (FSTD), and/or flight training. (b) Flight crew with no reduced required landing distance operations experience should have completed the full training programme of (a) above. (c) Flight crew with previous reduced required landing distance operations experience of a similar type of operation with another EU operator, may undertake the following: (1) an abbreviated ground training course if operating an aircraft of a type or class different from that of the aircraft on which the previous reduced required landing distance operations experience was gained; (2) an abbreviated ground, FSTD and/or flight training course if operating the same type or class and variant of the same aircraft type or class on which the previous reduced required landing distance operations experience was gained; this course should include at least the provisions of the conversion training contained in this AMC; the operator may reduce the number of approaches/landings required by the conversion training if the type/class or the variant of the aircraft type or class has the same or similar operating procedures, handling characteristics and performance characteristics as the previously operated aircraft type or class. (d) Flight crew with reduced required landing distance operations experience with the operator may undertake an abbreviated ground, FSTD and/or flight training course according to the following conditions: (1) when changing aircraft type or class, the abbreviated course should include at least the content of the conversion training; (2) when changing to a different variant of aircraft within the same type or class rating that has the same or similar operating procedures, handling characteristics and performance characteristics, as the previously operated aircraft type or class, a difference course or familiarisation appropriate to the change of variant should fulfil the abbreviated course’s purposes; and (3) when changing to a different variant of aircraft within the same type or class rating that has significantly different operating procedures, handling characteristics and performance characteristics, the abbreviated course should include the content of the conversion training. |  |  |  |  |  |  |  |  |  |  |
|  |  | Ground training (a) The initial ground training course for reduced required landing distance operations should include at least the following: (1) operational procedures and limitations, including flight preparation and planning; (2) characteristics of the runway visual aids and runway markings; (3) aircraft performance related to reduced required landing distance operations, including: (i) aircraft-specific decelerating devices and equipment; (ii) items that increase the aircraft landing distance, e.g. excess speed at touchdown, threshold crossing height, delayed brake application, delayed spoiler/speed brake or thrust reverser application; and (iii) runway surface conditions; (4) in-flight assessment of landing performance, including maximum landing masses and runway conditions; (5) stabilised approach criteria; (6) correct vertical flight path after the DA/MDA; (7) correct flare, touchdown and braking techniques; (8) touchdown within the appropriate touchdown zone; (9) recognition of failure of aircraft equipment affecting aircraft performance, and action to be taken in that event; (10) flight crew task allocation and pilot monitoring duties, including monitoring of the activation of deceleration devices; (11) go-around/balked-landing criteria and decision-making; (12) selection of precision approaches versus non-precision approaches if both are available; and (13) qualification requirements for pilots to obtain and retain reduced required landing distance operations, including aerodrome landing analysis programme (ALAP) procedures. |  |  |  |  |  |  |  |  |  |  |
|  |  | FSTD training and/or flight training (a) FSTD and/or flight training should be undertaken by all flight crew on flight duty at the controls during landing when performing reduced required landing distance operations. (b) FSTD and/or flight training for reduced required landing distance operations should include checks of equipment functionality, both on the ground and in flight. (c) Initial reduced required landing distance operations training should consist of a minimum of two approaches and landings to include at least the following exercises which may be combined: (1) an approach and landing at the maximum landing mass; (2) an approach and landing without the use of visual approach; (3) a landing on a wet runway; (4) a landing with crosswind; (5) a malfunction of a stopping device on landing; and (6) a go-around/balked landing. (d) Special emphasis should be given to the following items: (1) in-flight assessment of landing performance; (2) stabilised approach, recognition of an unstable approach and, consequentially, a go-around; (3) flight crew task allocation and pilot monitoring duties, including monitoring of the activation of deceleration devices; (4) timely and correct activation of deceleration devices; (5) correct flare technique; and (6) landing within the appropriate touchdown zone. |  |  |  |  |  |  |  |  |  |  |
|  |  | Conversion training Flight crew members should complete the following reduced required landing distance operations training if converting to a new type or class or variant of aircraft in which reduced required landing distance operations will be conducted. (a) Ground training, taking into account the flight crew member’s reduced required landing distance operations experience. (b) FSTD training and/or flight training. |  |  |  |  |  |  |  |  |  |  |
|  |  | Recurrent training and checking (a) The operator should ensure that in conjunction with the normal recurrent training and operator’s proficiency checks, the pilot’s knowledge and ability to perform the tasks associated with reduced required landing distance operations are adequate. (b) The items of the ground training should cover a 3-year period. (c) An annual reduced required landing distance operations training should consist of a minimum of two approaches and landings so that it includes at least the following exercises which may be combined: (1) an approach and landing at the maximum landing mass; (2) an approach and landing without the use of visual approach; (3) a landing on a wet runway; (4) a malfunction of a stopping device on landing; and (5) a go-around/balked landing. (6) Operations in crosswind conditions |  |  |  |  |  |  |  |  |  |  |
|  |  | Flight crew qualification and experience (a) Flight crew qualification and experience are specific to the operator and type of aircraft operated. (b) The operator should ensure that each flight crew member successfully completes the specified FSTD and/or flight training before conducting reduced required landing distance operations. (c) The operator should ensure that no inexperienced flight crew members, as defined in AMC1.ORO.FC.200(a), perform an approach and landing with reduced required landing distance operations. |  |  |  |  |  |  |  |  |  |  |
| 43. | AMC2 CAT.POL.A.255(b)(2)(iv) | Monitoring |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Reduced required landing distance operations should be continuously monitored by the operator to detect any undesirable trends before they become hazardous. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) A flight data monitoring (FDM) programme, as required by ORO.AOC.130, is an acceptable method to monitor operational risks related to reduced required landing distance operations. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) When an FDM programme is in use, it should include FDM events or FDM measurements relevant for monitoring the risk of runway excursions at landing. |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) When FDM is neither required by ORO.AOC.130, nor implemented on a voluntary basis, flight crew reports should be used. Specific guidance for reporting events and exceedances during reduced required landing distance operations should be provided to the flight crew. |  |  |  |  |  |  |  |  |  |  |
| 44. | GM1 CAT.POL.A.255(b)(2)(iv) | General Flight crew training should be conducted preferably at aerodromes representative of the intended operations. An FSTD generic aerodrome with the same characteristics of an aerodrome requiring the reduced required landing distance is also acceptable for the initial and recurrent training. |  |  |  |  |  |  |  |  |  |  |
| 45. | GM2 CAT.POL.A.255(b)(2)(iv) | Monitoring |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Although ORO.AOC.130 requires an FDM programme only for aeroplanes with a maximum certified take-off mass (MCTOM) of more than 27 000 kg, FDM may be used voluntarily on aeroplanes having a lower MCTOM. It is recommended for all operators conducting reduced required landing distance operations. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Guidance on the definition of FDM events and FDM measurements relevant for monitoring the risk of runway excursion at landing may be found in the publications of the European Operators Flight Data Monitoring (EOFDM) forum. |  |  |  |  |  |  |  |  |  |  |
| 46. | AMC1 CAT.POL.A.255(b)(2)(v) | Aerodrome landing analysis programme (ALAP) The intent of an ALAP is to ensure that the aerodrome critical data related to landing performance in reduced required landing distance operations is known and taken into account in order to avoid any further increase of the landing distance. Two important aerodrome-related variables largely contribute to increasing the landing distance: landing (ground) speed and deceleration capability. Related factors to consider should include at least the following elements: |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Topography Terrain around the aerodrome should be considered. High, fast-rising terrain may require special approach or decision points, missed approach or balked landing procedures and may affect landing performance. Aerodromes located on top of hilly terrain or downwind of mountainous terrain may occasionally experience conditions of wind shear and gusts. Such conditions are particularly relevant during the landing manoeuvre, particularly during the flare, and may increase landing distance. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Runway conditions Runway characteristics, such as unknown slope and surface composition, can cause the actual landing distance to be longer than the calculated landing distance. The braking action always impacts the landing distance required as it deteriorates. To this regard, consideration should be given to, and information obtained on, the maintenance status of the runway, as a wet runway surface may be significantly degraded due to poor aerodrome maintenance. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) Aerodrome or area weather Some aerodromes may not have current weather reports and forecast available for flight planning. Others may have automated observations for operational use. Others may depend on the weather forecast of a nearby aerodrome. Area forecasts are also valuable in evaluating weather conditions for a particular operation. Comparing forecasted conditions to current conditions provides insight on upcoming changes as weather systems move and forecasts are updated. Longer flight segments may lean more heavily on the forecast for the estimated time of arrival (ETA), as current conditions may change significantly as weather systems move. The most important factors that should be considered are contained in AMC1 CAT.OP.MPA.300(a), AMC1 CAT.OP.MPA.311, GM1 CAT.OP.MPA.311, GM1 CAT.OP.MPA.303 and GM2 CAT.OP.MPA.303. |  |  |  |  |  |  |  |  |  |  |
|  |  | (d) Adverse weather Adverse weather conditions include, but are not restricted to, thunderstorms, showers, downbursts, squall lines, tornadoes, moderate or severe turbulence on approach, heavy precipitation, wind shear and icing conditions. In general, all weather phenomena having the potential to increase the landing distance should be carefully assessed. Among these, tailwind is particularly relevant. Wind variations should be carefully monitored as they may lead to variations in the reported and/or actual wind at the touchdown zone. Due consideration should be given also to the crosswind perpendicular to the landing runway as a slight variation in the direction of the crosswind may result in a considerable tailwind component. |  |  |  |  |  |  |  |  |  |  |
|  |  | (e) Runway safety margins Displaced thresholds, aerodrome construction, and temporary obstacles (such as cranes and drawbridges) may impact the runway length available for landing. Notices to airmen (NOTAMs) must be consulted during the flight preparation. Another safety margin is the size and adequacy of the runway strip and the runway end safety area (RESA). A well-designed and well-maintained runway strip and RESA decrease the risk of damaging the aircraft in case of a runway excursion. ICAO Annex 14 provides the Standards and Recommended Practices (SARPs) to this regard. |  |  |  |  |  |  |  |  |  |  |
| 47. | GM1 CAT.POL.A.255(b)(2)(v) | Aerodrome landing analysis programme (ALAP) — aerodrome facilities The ALAP may also consider the services that are available at the aerodrome. Services such as communications, maintenance, and fuelling, availability of adequate rescue and firefighting services (RFFS) and medical services may have an impact on operations to and from that aerodrome, though not directly related to the landing distance. It is also worth considering whether the aerodrome is only meeting ICAO and national standards or also ICAO recommendations, as well as when the aerodrome bearing ratios are below the design and maintenance criteria indicated in ICAO Doc 9157 ‘Aerodrome Design Manual’. |  |  |  |  |  |  |  |  |  |  |
| 48. | AMC1 CAT.POL.A.255(b)(2)(vi) | Equipment affecting landing performance Equipment affecting landing performance typically includes flaps, slats, spoilers, brakes, anti-skid, autobrakes, reversers, etc. The operator should establish procedures to identify, based on the aircraft characteristics, those systems and the equipment that are performance relevant, and to ensure that they are verified to be operative before commencing the flight. Appropriate entries should be included in the minimum equipment list (MEL) to prohibit dispatch with such equipment inoperative when conducting reduced required landing distance operations. |  |  |  |  |  |  |  |  |  |  |
| 49. | GM1 CAT.POL.A.255(b)(2)(vi) | Equipment affecting landing performance Should any item of equipment affecting landing performance become inoperative during flight, the failure will be dealt with in accordance with the abnormal/emergency procedures established in the OM and, based on the prevailing conditions for the remainder of the flight, the commander will decide upon the discontinuation of the planned operation of reduced required landing distance. |  |  |  |  |  |  |  |  |  |  |
| 50. | AMC1 CAT.POL.A.255(b)(2)(vii) | RECENCY Flight crew conducting reduced landing distance operations should perform at least two landings with reduced landing distance, either in actual operations or in an FSTD, performed within the validity period of the operator proficiency check (OPC). |  |  |  |  |  |  |  |  |  |  |
| 51. | AMC1 CAT.POL.A.255(b)(2)(ix) | Additional aerodrome conditions |  |  |  |  |  |  |  |  |  |  |
|  |  | (a) Operators should establish procedures to ensure that: (1) the aerodrome information is obtained from an authoritative source, or when this is not available, from a source that has been verified by the operator to meet quality standards that are adequate for the intended use; (2) any change reducing landing distances that has been declared by the aerodrome operator has been taken into account; and (3) no steep approaches, screen heights lower than 35 ft or higher than 60 ft, operations outside the stabilised approach criteria, or low-visibility operations are required at the aerodrome when reduced required landing distance operations are conducted. |  |  |  |  |  |  |  |  |  |  |
|  |  | (b) Additional aerodrome conditions related to aeroplane type characteristics, orographic characteristics in the approach area, available approach aids and missed approach/balked landing considerations, as well as operating limitations, should also be taken into account. |  |  |  |  |  |  |  |  |  |  |
|  |  | (c) When assessing the aerodrome characteristics and the level of risk of the aeroplane undershooting or overrunning the runway, the operator should consider the nature and location of any hazard beyond the runway end, including the topography and obstruction environment beyond the runway strip, the length of the RESA and the effectiveness of any other mitigation measures that may be in place to reduce the likelihood and the consequences of a runway overrun. |  |  |  |  |  |  |  |  |  |  |

Забележка: След класифициране на несъответствията информацията да се изпрати на оператора за съставяне на План за коригиращи действия.

Забележка: В колоните под заглавие „СЪОТВЕТСВИЕ/Compliance“ се отбелязва с  знак Х.

Забележка: При извършване на одит/инспекция на място в офиса на оператора, моля взимайте копия от прегледани документи за доказателства.

Забележка: Елементите от чек листа, които няма да бъдат прегледжани и оценявани в конкретен одит, следва да се отбележат в колонота „NC“, за да може при последващи инспекции да бъдат прегледани и оценени.

Забележка: Колона “I” се отбелязва, когато се преглежда изпълнението от страна на оператора на съответното изискване. Възможна е и комбинация от колона “I” и  “D”.

Забележка: Колона “D” се отбелязва, когато се преглежда въвеждането в ръководствата на оператора на съответното изискване. Възможна е и комбинация от колона “I” и  “D”.

Легенда:

C - Checked

NC – Not checked

S - Satisfactory

NS - Not satisfactory

NA - Not applicable

I - Implemented

D – Documented